| Logo of OPC Foundation, only if this is a joint work. Both logos shall be side by side.. |  | Logo of other organization. Both logos shall be side by side.. | OPC UA Companion Specification |
| --- | --- | --- | --- |
| OPC nnnnn-m |  |  |  |
| OPC UA for Asset Administration Shell Part 1: Asset Administration Shell Draft 3.00-draft3 YYYY-MM-DD Edit the advanced properties to complete the title page. Boxes are used to provide context or guidelines on expected content. Any words or phrases written in red font colour shall be replaced by actual values as appropriate for the companion specification. |  |  |  |

| Specification Type: | Industry Standard Specification | Comments: |  |
| --- | --- | --- | --- |
| Doc-Number | OPC nnnnn-m |  |  |
| Title: | OPC UA for Asset Administration ShellPart 1 :Asset Administration Shell | Date: | YYYY-MM-DD |
| Version: | Draft 3.00-draft3 | Software: | MS-Word |
|  |  | Source: | OPC 20020 - UA Companion Specification Template v1.01.18.docx |
| Author: | <organization> | Status: | Draft |

Template Revisions

| Version | Date | Description |
| --- | --- | --- |
| 1.01.09 | Mar 09, 2020 | M-4665: Link to NodeSet indicates that also ERRATA, Amendments or Revisions are applied. Fixed various styles. |
| 1.01.10 | Apr 29, 2020 | Minor update for the Value attribute (M-5314). |
| 1.01.11 | July 09, 2020 | M-5595: Added better wording and recommendations for NamespaceMetadata as suggested. M-5520: Fixed quotation marks. M-5781: Title "Additional Variable Attributes" misleading. M-5673: Added HasInterface Reference to figure. M-5468: Use of Namespace Index also in Method signatures. M-5789: Clarify use of NamespaceIndex for Profiles or ConformanceUnits in other specifications. |
| 1.01.12 | Nov 16, 2020 | M-5904: BrowseName convention for placeholders. M-6184: The "additional attributes" table shall not be restricted to Value and Description. M-6183: Fix heading names for columns with a browse path. M-6112: Add an example state machine. M-6185: Add new table types to conventions. |
| 1.01.13 | Feb 17, 2021 | M-6302: Updated format for NamespaceMetadata table to make it validatable. M-6317: Added guideline for uncaptioned embedded figures. |
| 1.01.14 | Nov 15, 2021 | M-6264: GeneratesEvent example added. M-7147: Added abbreviated terms used in template. M-6265: No HasSubtype References. M-7265: Added max size for ConformanceUnit description. M-6612: Added definition of capability ID (Annex A.2). M-6150: Extended tables with Conformance Units row. |
| 1.01.15 | Oct 01, 2022 | M-7528: Namespace index also for the “Subtype of” rows. M-8025: Added missing BrowsePath tables for subcomponents. M-6776: Changed the examples for Structure and Union to start with an uppercase character. M-7341: Added extended definition for structures. |
| 1.01.16 | Mar 14, 2023 | M-8638: Updated URL format for schema files (Annex A). Updated URL format for normative references to documents (Clause 2). |
| 1.01.17 | Nov 17, 2023 | M-9196: Conformance Units and Profiles should be in profile database. M-8765: Added URL syntax for individual files. M-9170: Casing of UANodeSet wrong. M-9182: Added reference to IEC specifications. |
| 1.01.18 | Dec 03, 2024 | M-9275: Missing value rank 0. M-9270: Added guidelines to make figures smaller. M-8648: Well-Known BrowseNames. M-9723: Hyperlinks to online reference. |
| 1.01.19 | Jan 21, 2025 | M-9833: Added description on issue reporting to the “Agreement of Use”. |

CONTENTS

Right-click and choose Update Field.

Figures

Right-click and choose Update Field.

Tables

Right-click and choose Update Field.

OPC FOUNDATION, Industrial Digital Twin Association (IDTA)

____________

This work (OPC UA Companion Specification Template, by OPC Foundation), identified by OPC Foundation, is free of known copyright restrictions.

<Remove the above Public Domain notice and add a copyright date in the following Agreement of use for the completed specification. If possible (but not legally required), provide attribution to the original author in the following way:

“This specification is a derivative work of the public domain document OPC UA Companion Specification Template, by OPC Foundation”

>

AGREEMENT OF USE

<For joint work, this is the agreement accepted by OPC Foundation and used by most if not all joint companion specifications.

Changes to this agreement will require OPC Foundation review. For non-joint work replace this section with appropriate agreement of use and copyright verbiage.>

COPYRIGHT RESTRICTIONS

- This document is provided "as is" by the OPC Foundation and the Industrial Digital Twin Association (IDTA).

- Right of use for this specification is restricted to this specification and does not grant rights of use for referred documents.

- Right of use for this specification will be granted without cost.

- This document may be distributed through computer systems, printed or copied as long as the content remains unchanged and the document is not modified.

- OPC Foundation and Industrial Digital Twin Association (IDTA) do not guarantee usability for any purpose and shall not be made liable for any case using the content of this document.

- The user of the document agrees to indemnify OPC Foundation and Industrial Digital Twin Association (IDTA) and their officers, directors and agents harmless from all demands, claims, actions, losses, damages (including damages from personal injuries), costs and expenses (including attorneys' fees) which are in any way related to activities associated with its use of content from this specification.

- The document shall not be used in conjunction with company advertising, shall not be sold or licensed to any party.

- The intellectual property and copyright is solely owned by the OPC Foundation and the Industrial Digital Twin Association (IDTA).

PATENTS

The attention of adopters is directed to the possibility that compliance with or adoption of OPC or Industrial Digital Twin Association (IDTA) specifications may require use of an invention covered by patent rights. OPC Foundation or Industrial Digital Twin Association (IDTA) shall not be responsible for identifying patents for which a license may be required by any OPC or Industrial Digital Twin Association (IDTA) specification, or for conducting legal inquiries into the legal validity or scope of those patents that are brought to its attention. OPC or Industrial Digital Twin Association (IDTA) specifications are prospective and advisory only. Prospective users are responsible for protecting themselves against liability for infringement of patents.

WARRANTY AND LIABILITY DISCLAIMERS

WHILE THIS PUBLICATION IS BELIEVED TO BE ACCURATE, IT IS PROVIDED "AS IS" AND MAY CONTAIN ERRORS OR MISPRINTS. THE OPC FOUDATION NOR Industrial Digital Twin Association (IDTA) MAKES NO WARRANTY OF ANY KIND, EXPRESSED OR IMPLIED, WITH REGARD TO THIS PUBLICATION, INCLUDING BUT NOT LIMITED TO ANY WARRANTY OF TITLE OR OWNERSHIP, IMPLIED WARRANTY OF MERCHANTABILITY OR WARRANTY OF FITNESS FOR A PARTICULAR PURPOSE OR USE. IN NO EVENT SHALL THE OPC FOUNDATION NOR Industrial Digital Twin Association (IDTA) BE LIABLE FOR ERRORS CONTAINED HEREIN OR FOR DIRECT, INDIRECT, INCIDENTAL, SPECIAL, CONSEQUENTIAL, RELIANCE OR COVER DAMAGES, INCLUDING LOSS OF PROFITS, REVENUE, DATA OR USE, INCURRED BY ANY USER OR ANY THIRD PARTY IN CONNECTION WITH THE FURNISHING, PERFORMANCE, OR USE OF THIS MATERIAL, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

The entire risk as to the quality and performance of software developed using this specification is borne by you.

RESTRICTED RIGHTS LEGEND

This Specification is provided with Restricted Rights. Use, duplication, or disclosure by the U.S. government is subject to restrictions as set forth in (a) this Agreement pursuant to DFARs 227.7202-3(a); (b) subparagraph (c)(1)(i) of the Rights in Technical Data and Computer Software clause at DFARs 252.227-7013; or (c) the Commercial Computer Software Restricted Rights clause at FAR 52.227-19 subdivision (c)(1) and (2), as applicable. Contractor / manufacturer are the OPC Foundation, 16101 N. 82nd Street, Suite 3B, Scottsdale, AZ, 85260-1830

COMPLIANCE

The combination of Industrial Digital Twin Association (IDTA) and OPC Foundation shall at all times be the sole entities that may authorize developers, suppliers and sellers of hardware and software to use certification marks, trademarks, or other special designations to indicate compliance with these materials as specified within this document. Products developed using this specification may claim compliance or conformance with this specification if and only if the software satisfactorily meets the certification requirements set by Industrial Digital Twin Association (IDTA) or the OPC Foundation. Products that do not meet these requirements may claim only that the product was based on this specification and must not claim compliance or conformance with this specification.

Trademarks

Most computer and software brand names have trademarks or registered trademarks. The individual trademarks have not been listed here.

GENERAL PROVISIONS

Should any provision of this Agreement be held to be void, invalid, unenforceable, or illegal by a court, the validity and enforceability of the other provisions shall not be affected thereby.

This Agreement shall be governed by and construed under the laws of Germany.

This Agreement embodies the entire understanding between the parties with respect to, and supersedes any prior understanding or agreement (oral or written) relating to, this specification.

ISSUE REPORTING

If an error or problem is found in this specification, the UaNodeSet, or any associated supplementary files, it should be reported as an issue.

The reporting process can be found here: https://opcfoundation.org/resources/issue-tracking/

The Link to the issue tracking project for this document is here:

https://mantis.opcfoundation.org/set_project.php?project_id=<nnn>&make_default=no

<nnn> is the project_id in Mantis which is created for any document when requested by the working group. Example: https://mantis.opcfoundation.org/set_project.php?project_id=142&make_default=no is the Link for OPC 40001-* (Machinery).

If you have no Mantis Project or do not know the project_id, please send a request to TechnicalDirector@opcfoundation.org.

Release 3.00-draft3 highlights

<Revision Highlights specify the interesting changes to the previous revision. Such a "Revision Highlights" clause will not exist in the first revision.>

The table below follows the design and process used for the OPC UA specification. All "interesting" changes need to be reported in the problem tracking tool used for the specification. The OPC Foundation uses Mantis and provides it for companion specs as well. If a different problem tracking tool is used, "Mantis" needs to be replaced.

<The two rows in the following table are just examples. Note that the "Mantis IDs" are real hyperlinks – when clicked the reader opens the Mantis page.>

Scope shall specify the type of change. The three options are Errata (corrected something that was wrong), Clarification (make something clearer that was not wrong), and Feature (added new functionality).

The following table includes the Mantis issues resolved with this revision.

| Mantis ID | Scope | Summary | Resolution |
| --- | --- | --- | --- |
| 3165 | Errata | Wrong variable data types in Program example. | Fixed datatypes and replaced Table A.14 by description of the variables. |
| 3521 | Clarification | Modelling rules for states and transitions. | Removed the modelling rules from state and transition objects as they are never created (multiple tables). |

OPC UA FOR ASSET ADMINISTRATION SHELL

Part 1: Mapping the Asset Administration Shell V3 metamodel and registry to OPC UA

# Scope

This companion specification defines an OPC UA information model for the Asset Administration Shell

(AAS). It covers two things a Server does with shells, and a Server may implement either or both:

- The metamodel. An AAS, its submodels and its submodel elements are projected onto the AddressSpace as typed nodes, so a Client reads a value, writes one, or invokes an operation through ordinary OPC UA Services.

- The registry. A catalogue of shells is projected onto the AddressSpace as folders of files, so a Client discovers which shells exist, retrieves a submodel as a document, reads an earlier revision, or follows a shell served by another Server.

The metamodel mapping is lossless in both directions: an AAS materialized into the AddressSpace

and read back reproduces the original, and an AddressSpace subtree serialized and materialized again

reproduces itself. Clause 6.1 defines the rules this requires and clause 6.4 defines how it is tested.

Losslessness is not decoration — it is what allows an AAS to be compiled into a Server by a source

generator, because a mapping in which any choice is left to the implementer cannot be generated.

The same mapping is reached from a W3C Thing Description. A Thing Description carrying the AAS

vocabulary materializes the AddressSpace this document defines, and an AddressSpace subtree is

expressible as a Thing Description again, so an AAS authored as a WoT document and an AAS

materialized from a package are the same nodes. Annex F states the correspondence, the

fixture of the conformance corpus.

OPC 30270 v1.00 maps the AAS v1.x metamodel in a separate namespace. Clause 4.3 defines its

relationship to this specification.

NOTE   PROVISIONAL WORKING DRAFT — not normative, not endorsed by the OPC Foundation. The namespace URI and NodeIds in this document are provisional until assigned by the OPC Foundation.

OPC Foundation

OPC is the interoperability standard for the secure and reliable exchange of data and information in the industrial automation space and in other industries. It is platform independent and ensures the seamless flow of information among devices from multiple vendors. The OPC Foundation is responsible for the development and maintenance of this standard.

OPC UA is a platform independent service-oriented architecture that integrates all the functionality of the individual OPC Classic specifications into one extensible framework. This multi-layered approach accomplishes the original design specification goals of:

- Platform independence: from an embedded microcontroller to cloud-based infrastructure

- Secure: encryption, authentication, authorisation, and auditing

- Extensible: ability to add new features including transports without affecting existing applications

- Comprehensive information modelling capabilities: for defining any model from simple to complex

Industrial Digital Twin Association (IDTA)

# Normative references

The following referenced documents are indispensable for the application of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments and errata) applies.

NOTE   The OPC UA core specifications are regularly published as IEC 62541.

OPC 10000-1, OPC Unified Architecture - Part 1: Overview and Concepts

https://www.opcfoundation.org/documents/10000-1/

OPC 10000-2, OPC Unified Architecture - Part 2: Security Model

https://www.opcfoundation.org/documents/10000-2/

OPC 10000-3, OPC Unified Architecture - Part 3: Address Space Model

https://www.opcfoundation.org/documents/10000-3/

OPC 10000-4, OPC Unified Architecture - Part 4: Services

https://www.opcfoundation.org/documents/10000-4/

OPC 10000-5, OPC Unified Architecture - Part 5: Information Model

https://www.opcfoundation.org/documents/10000-5/

OPC 10000-6, OPC Unified Architecture - Part 6: Mappings

https://www.opcfoundation.org/documents/10000-6/

OPC 10000-7, OPC Unified Architecture - Part 7: Profiles

https://www.opcfoundation.org/documents/10000-7/

OPC 10000-10, OPC Unified Architecture - Part 10: Programs

https://www.opcfoundation.org/documents/10000-10/

OPC 10000-12, OPC Unified Architecture - Part 12: Discovery and Global Services

https://www.opcfoundation.org/documents/10000-12/

OPC 10000-20, OPC Unified Architecture - Part 20: File Transfer

https://www.opcfoundation.org/documents/10000-20/

OPC 20020, OPC UA Companion Specification Template and Rules

https://opcfoundation.org/Guidelines-And-Templates/

OPC 30270, Part 1, version 1.00, OPC UA for Asset Administration Shell - Part 1: Asset Administration Shell

OPC 30450-1, OPC UA for Digital Product Passport - Part 1: Information Model

OPC UA for xRegistry, Abstract registry information model defining RegistryType, GroupType and ResourceType

xRegistry specification, Cloud-native registry core, model and HTTP binding

https://github.com/xregistry/spec

IDTA-01001, Specification of the Asset Administration Shell - Part 1: Metamodel, version 3

https://industrialdigitaltwin.org/en/content-hub/aasspecifications/specification-of-the-asset-administration-shell-part-1-metamodel-idta-number-01001

IDTA-01002, Specification of the Asset Administration Shell - Part 2: Application Programming Interfaces, version 3

https://industrialdigitaltwin.org/en/content-hub/aasspecifications/specification-of-the-asset-administration-shell-part-2-application-programming-interfaces-idta-number-01002

IDTA-01005, Specification of the Asset Administration Shell - Part 5: Package File Format (AASX), version 3

https://industrialdigitaltwin.org/en/content-hub/aasspecifications/specification-of-the-asset-administration-shell-part-5-package-file-format-aasx-idta-number-01005

IEC 63278-1, Asset Administration Shell for industrial applications - Part 1: Asset Administration Shell structure

https://webstore.iec.ch/en/publication/79086

W3C XML Schema Part 2, XML Schema Part 2: Datatypes Second Edition

https://www.w3.org/TR/xmlschema-2/

W3C Web of Things Thing Description 1.1, Web of Things (WoT) Thing Description 1.1

https://www.w3.org/TR/wot-thing-description11/

OPC UA - Web of Things Binding, OPC UA binding for W3C Thing Descriptions and Thing Models

OPC UA - WoT Connectivity, Thing Description registry and derived AddressSpace projection

AAS JSON-LD mapping, JSON-LD mapping for the Asset Administration Shell

# Terms, abbreviated terms and conventions

## Overview

## Asset Administration Shell terms

It is assumed that basic concepts of OPC UA information modelling and of Asset Administration Shell are understood in this document. This document uses these concepts to describe the Asset Administration Shell Information Model. For the purposes of this document, the terms and definitions given in OPC 10000-1, OPC 10000-3, OPC 10000-4, OPC 10000-5 and OPC 10000-7, as well as the following, apply.

OPC UA terms and terms defined in this document are italicized in the document.

## Abbreviated terms

Shell

An Asset Administration Shell: the digital representation of one asset.

Submodel

One coherent aspect of an asset, identified in its own right and typed by its semantic identifier.

Element

A submodel element: one datum, collection or operation within a submodel.

Environment

The container of shells, submodels and concept descriptions that an AAS serialization carries.

Materialization

Producing an AddressSpace subtree from an AAS, per clause 6.1.6.

Serialization

Producing an AAS from an AddressSpace subtree — the reverse direction.

Registry

The catalogue half of this specification: shells and their documents as an xRegistry projection.

Source identity

The domain string that names what a registry entity is, from which its identifier is derived.

## Conventions used in this document

Following are basic conventions that shall be followed for all formal definitions used.

### Conventions for Node descriptions

#### Node definitions

Node definitions are specified using tables (see Table 2).

Attributes are defined by providing the Attribute name and a value, or a description of the value.

References are defined by providing the ReferenceType name, the BrowseName of the TargetNode and its NodeClass.

If the TargetNode is a component of the Node being defined in the table the Attributes of the composed Node are defined in the same row of the table.

The DataType is only specified for Variables; “[<number>]” indicates a single-dimensional array, for multi-dimensional arrays the expression is repeated for each dimension (e.g. [2][3] for a two-dimensional array). For all arrays the ArrayDimensions is set as identified by <number> values. If no <number> is set, the corresponding dimension is set to 0, indicating an unknown size. If no number is provided at all the ArrayDimensions can be omitted. If no brackets are provided, it identifies a scalar DataType and the ValueRank is set to the corresponding value (see OPC 10000-3). In addition, ArrayDimensions is set to null or is omitted. If it can be Any or ScalarOrOneDimension, the value is put into “{<value>}”, so either “{Any}” or “{ScalarOrOneDimension}” and the ValueRank is set to the corresponding value (see OPC 10000-3) and the ArrayDimensions is set to null or is omitted. Examples are given in Table 1.

Table 1 – Examples of DataTypes

| Notation | DataType | ValueRank | ArrayDimensions | Description |
| --- | --- | --- | --- | --- |
| 0:Int32 | 0:Int32 | -1 | omitted or null | A scalar Int32. |
| 0:Int32{OneOrMoreDimensions} | 0:Int32 | 0 | omitted or null | The Int32 value is an array with one or more dimensions. |
| 0:Int32[] | 0:Int32 | 1 | omitted or {0} | Single-dimensional array of Int32 with an unknown size. |
| 0:Int32[][] | 0:Int32 | 2 | omitted or {0,0} | Two-dimensional array of Int32 with unknown sizes for both dimensions. |
| 0:Int32[3][] | 0:Int32 | 2 | {3,0} | Two-dimensional array of Int32 with a size of 3 for the first dimension and an unknown size for the second dimension. |
| 0:Int32[5][3] | 0:Int32 | 2 | {5,3} | Two-dimensional array of Int32 with a size of 5 for the first dimension and a size of 3 for the second dimension. |
| 0:Int32{Any} | 0:Int32 | -2 | omitted or null | An Int32 where it is unknown if it is scalar or array with any number of dimensions. |
| 0:Int32{ScalarOrOneDimension} | 0:Int32 | -3 | omitted or null | An Int32 where it is either a single-dimensional array or a scalar. |

The TypeDefinition is specified for Objects and Variables.

The TypeDefinition column specifies a symbolic name for a NodeId, i.e., the specified Node points with a HasTypeDefinition Reference to the corresponding Node.

The ModellingRule of the referenced component is provided by specifying the symbolic name of the rule in the ModellingRule column. In the AddressSpace, the Node shall use a HasModellingRule Reference to point to the corresponding ModellingRule Object.

If the NodeId of a DataType is provided, the symbolic name of the Node representing the DataType shall be used.

Note that if a symbolic name of a different namespace is used, it is prefixed by the NamespaceIndex (see 3.4.2.2).

Nodes of all other NodeClasses cannot be defined in the same table; therefore, only the used ReferenceType, their NodeClass and their BrowseName are specified. A reference to another part of this document points to their definition. Table 2 illustrates the table. If no components are provided, the DataType, TypeDefinition and Other columns can be omitted and only a Comment column is introduced to point to the Node definition.

Each Type Node or well-known Instance Node defined shall have one or more ConformanceUnits defined in 0 that require the Node to be in the AddressSpace.

The relations between Nodes and ConformanceUnits are defined at the end of the tables defining Nodes, one row per ConformanceUnit. The ConformanceUnits are reflected in the Category element for the Node definition in the UANodeSet (see OPC 10000-6).

The list of ConformanceUnits in the UANodeSet allows Servers to optimize resource consumption by using a list of supported ConformanceUnits to select a subset of the Nodes in an Information Model.

When a Node is selected in this way, all dependencies implied by the References are also selected.

Dependencies exist if the Node is the source of HasTypeDefinition, HasInterface, HasAddIn or any HierarchicalReference. Dependencies also exist if the Node is the target of a HasSubtype Reference. For Variables and VariableTypes, the value of the DataType Attribute is a dependency. For DataType Nodes, any DataTypes referenced in the DataTypeDefinition Attribute are also dependencies.

For additional details see OPC 10000-5.

Table 2 – Type Definition Table

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Attribute name | Attribute value. If it is an optional Attribute that is not set “--” is used. |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| ReferenceType name | NodeClass of the target Node. | BrowseName of the target Node. | DataType of the referenced Node, only applicable for Variables. | TypeDefinition of the referenced Node, only applicable for Variables and Objects. | Additional characteristics of the TargetNode such as the ModellingRule or AccessLevel. |
| NOTE Notes referencing footnotes of the table content. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| Name of ConformanceUnit, one row per ConformanceUnit |  |  |  |  |  |

Components of Nodes can be complex that is containing components by themselves. The TypeDefinition, NodeClass and DataType can be derived from the type definitions, and the symbolic name can be created as defined in 3.4.3.1. Therefore, those containing components are not explicitly specified; they are implicitly specified by the type definitions.

The Other column defines additional characteristics of the Node. Examples of characteristics that can appear in this column are show in Table 3.

Table 3 – Examples of Other characteristics

| Name | Short Name | Description |
| --- | --- | --- |
| 0:Mandatory | M | The Node has the Mandatory ModellingRule. |
| 0:Optional | O | The Node has the Optional ModellingRule. |
| 0:MandatoryPlaceholder | MP | The Node has the MandatoryPlaceholder ModellingRule. |
| 0:OptionalPlaceholder | OP | The Node has the OptionalPlaceholder ModellingRule. |
| ReadOnly | RO | The Node AccessLevel has the CurrentRead bit set but not the CurrentWrite bit. |
| ReadWrite | RW | The Node AccessLevel has the CurrentRead and CurrentWrite bits set. |
| WriteOnly | WO | The Node AccessLevel has the CurrentWrite bit set but not the CurrentRead bit. |

If multiple characteristics are defined, they are separated by commas. The name or the short name can be used.

#### Additional References

To provide information about additional References, the format as shown in Table 4 is used.

Table 4 – <some>Type additional References

| SourceBrowsePath | Reference Type | Is Forward | TargetBrowsePath |
| --- | --- | --- | --- |
| SourceBrowsePath is always relative to the TypeDefinition. Multiple elements are defined as separate rows of a nested table. | ReferenceType name | True = forward Reference. | TargetBrowsePath points to another Node, which can be a well-known instance or a TypeDefinition. You can use BrowsePaths here as well, which is either relative to the TypeDefinition or absolute. If absolute, the first entry needs to refer to a type or well-known instance, uniquely identified within a namespace by the BrowseName. |

References can be to any other Node.

#### Additional sub-components

To provide information about sub-components, the format as shown in Table 5 is used.

Table 5 – <some>Type additional subcomponents

| BrowsePath | References | NodeClass | BrowseName | DataType | TypeDefinition | Others |
| --- | --- | --- | --- | --- | --- | --- |
| BrowsePath is always relative to the TypeDefinition. Multiple elements are defined as separate rows of a nested table | NOTE Same as for Table 2 |  |  |  |  |  |

#### Additional Attribute values

The type definition table provides columns to specify the values for required Node Attributes for InstanceDeclarations. To provide information about additional Attributes, the format as shown in Table 6 is used.

Table 6 – <some>Type Attribute values for child Nodes

| BrowsePath | <Attribute name> Attribute |
| --- | --- |
| BrowsePath is always relative to the TypeDefinition. Multiple elements are defined as separate rows of a nested table | The values of attributes are converted to text by adapting the reversible JSON encoding rules defined in OPC 10000-6. If the JSON encoding of a value is a JSON string or a JSON number then that value is entered in the value field. Double quotes are not included. If the DataType includes a NamespaceIndex (QualifiedNames, NodeIds or ExpandedNodeIds) then the notation used for BrowseNames is used. If the value is an Enumeration the name of the enumeration value is entered. If the value is a Structure then a sequence of name and value pairs is entered. Each pair is followed by a newline. The name is followed by a colon. The names are the names of the fields in the DataTypeDefinition. If the value is an array of non-structures then a sequence of values is entered where each value is followed by a newline. If the value is an array of Structures or a Structure with fields that are arrays or with nested Structures then the complete JSON array or JSON object is entered. |

There can be multiple columns to define more than one Attribute.

### NodeIds and BrowseNames

#### NodeIds

The NodeIds of all Nodes described in this standard are only symbolic names. Annex A defines the actual NodeIds.

The symbolic name of each Node defined in this document is its BrowseName, or, when it is part of another Node, the BrowseName of the other Node, a “.”, and the BrowseName of itself. In this case “part of” means that the whole has a HasProperty or HasComponent Reference to its part. Since all Nodes not being part of another Node have a unique name in this document, the symbolic name is unique.

The NamespaceUri for all NodeIds defined in this document is defined in Annex A. The NamespaceIndex for this NamespaceUri is vendor-specific and depends on the position of the NamespaceUri in the server namespace table.

Note that this document not only defines concrete Nodes, but also requires that some Nodes shall be generated, for example one for each Session running on the Server. The NodeIds of those Nodes are Server-specific, including the namespace. But the NamespaceIndex of those Nodes cannot be the NamespaceIndex used for the Nodes defined in this document, because they are not defined by this document but generated by the Server.

#### BrowseNames

The text part of the BrowseNames for all Nodes defined in this document is specified in the tables defining the Nodes. The NamespaceUri for all BrowseNames defined in this document is defined in 16.2.

For InstanceDeclarations of NodeClass Object and Variable that are placeholders (OptionalPlaceholder and MandatoryPlaceholder ModellingRule), the BrowseName and the DisplayName are enclosed in angle brackets (<>) as recommended in OPC 10000-3.

If a BrowseName is not defined by this document, a namespace index prefix is added to the BrowseName (e.g., prefix '0' leading to ‘0:EngineeringUnits’ or prefix '2' leading to ‘2:DeviceRevision’). This is typically necessary if a Property of another specification is overwritten or used in the OPC UA types defined in this document. Table 40 provides a list of namespaces and their indexes as used in this document.

### Common Attributes

#### General

The Attributes of Nodes, their DataTypes and descriptions are defined in OPC 10000-3. Attributes not marked as optional are mandatory and shall be provided by a Server. The following tables define if the Attribute value is defined by this document or if it is server-specific.

For all Nodes specified in this document, the Attributes named in Table 7 shall be set as specified in the table.

Table 7 – Common Node Attributes

| Attribute | Value |
| --- | --- |
| DisplayName | The DisplayName is a LocalizedText. Each Server shall provide the DisplayName identical to the BrowseName of the Node for the LocaleId “en” unless specified differently in the specification. Whether the Server provides translated names for other LocaleIds is server-specific. |
| Description | Optionally a server-specific description is provided. |
| NodeClass | Shall reflect the NodeClass of the Node. |
| NodeId | The NodeId is described by BrowseNames as defined in 3.4.2.1. |
| WriteMask | Optionally the WriteMask Attribute can be provided. If the WriteMask Attribute is provided, it shall set all non-server-specific Attributes to not writable. For example, the Description Attribute can be set to writable since a Server can provide a server-specific description for the Node. The NodeId shall not be writable, because it is defined for each Node in this document. |
| UserWriteMask | Optionally the UserWriteMask Attribute can be provided. The same rules as for the WriteMask Attribute apply. |
| RolePermissions | Optionally server-specific role permissions can be provided. |
| UserRolePermissions | Optionally the role permissions of the current Session can be provided. The value is server-specific and depends on the RolePermissions Attribute (if provided) and the current Session. |
| AccessRestrictions | Optionally server-specific access restrictions can be provided. |

#### Objects

For all Objects specified in this document, the Attributes named in Table 8 shall be set as specified in the Table 8. The definitions for the Attributes can be found in OPC 10000-3.

Table 8 – Common Object Attributes

| Attribute | Value |
| --- | --- |
| EventNotifier | Whether the Node can be used to subscribe to Events or not is server-specific. |

#### Variables

For all Variables specified in this document, the Attributes named in Table 9 shall be set as specified in the table. The definitions for the Attributes can be found in OPC 10000-3.

Table 9 – Common Variable Attributes

| Attribute | Value |
| --- | --- |
| MinimumSamplingInterval | Optionally, a server-specific minimum sampling interval is provided. |
| AccessLevel | The access level for Variables used for type definitions is server-specific, for all other Variables defined in this document, the access level shall allow reading; other settings are server-specific. |
| UserAccessLevel | The value for the UserAccessLevel Attribute is server-specific. It is assumed that all Variables can be accessed by at least one user. |
| Value | For Variables used as InstanceDeclarations, the value is server-specific; otherwise, it shall represent the value described in the text. |
| ArrayDimensions | If the ValueRank does not identify an array of a specific dimension (i.e., ValueRank <= 0) the ArrayDimensions can either be set to null or the Attribute is missing. This behaviour is server-specific. If the ValueRank specifies an array of a specific dimension (i.e., ValueRank > 0) then the ArrayDimensions Attribute shall be specified in the table defining the Variable. |
| Historizing | The value for the Historizing Attribute is server-specific. |
| AccessLevelEx | If the AccessLevelEx Attribute is provided, it shall have the bits 8, 9, and 10 set to 0, meaning that read and write operations on an individual Variable are atomic, and arrays can be partly written. |

#### VariableTypes

For all VariableTypes specified in this document, the Attributes named in Table 10 shall be set as specified in the table. The definitions for the Attributes can be found in OPC 10000-3.

Table 10 – Common VariableType Attributes

| Attributes | Value |
| --- | --- |
| Value | Optionally a server-specific default value can be provided. |
| ArrayDimensions | If the ValueRank does not identify an array of a specific dimension (i.e., ValueRank <= 0) the ArrayDimensions can either be set to null or the Attribute is missing. This behaviour is server-specific. If the ValueRank specifies an array of a specific dimension (i.e., ValueRank > 0) then the ArrayDimensions Attribute shall be specified in the table defining the VariableType. |

#### Methods

For all Methods specified in this document, the Attributes named in Table 11 shall be set as specified in the table. The definitions for the Attributes can be found in OPC 10000-3.

Table 11 – Common Method Attributes

| Attributes | Value |
| --- | --- |
| Executable | All Methods defined in this document shall be executable (Executable Attribute set to “True”), unless it is defined differently in the Method definition. |
| UserExecutable | The value of the UserExecutable Attribute is server-specific. It is assumed that all Methods can be executed by at least one user. |

### Structures

OPC 10000-3 differentiates between different kinds of Structures. The following conventions explain, how these Structures shall be defined.

The first kind are Structures without optional fields where none of the fields allows subtype (except fields with abstract DataTypes). Its definition is in Table 12.

Table 12 – Structures without optional fields where none of the fields allow subtypes

| Name | Type | Description |
| --- | --- | --- |
| <someStructure> | structure | Subtype of <someParentStructure> defined in … |
| SP1 | 0:Byte[] | Setpoint 1 |
| SP2 | 0:Byte[] | Setpoint 2 |

The second kind are Structures with optional fields where none of the fields allows subtypes (except fields with abstract DataTypes). Its definition is in Table 13.

Structures with fields that are optional have an “Optional” column. Fields that are optional have True set, otherwise False.

Table 13 – Structures with optional fields

| Name | Type | Description | Optional |
| --- | --- | --- | --- |
| <someStructure> | structure | Subtype of <someParentStructure> defined in … |  |
| SP1 | 0:Byte[] | Setpoint 1 | False |
| Optional Field_1 | 0:String | Some Text | True |

The third kind are Structures without optional fields where one or more of the fields allow subtypes. Its definition is in Table 14.

Structures with fields that allow subtypes have an “Allow Subtypes” column. Fields that allow subtypes have True set, otherwise False. Fields with abstract DataTypes can always be subtyped.

Table 14 – Structures where one or more of the fields allow subtypes

| Name | Type | Description | Allow Subtypes |
| --- | --- | --- | --- |
| <someStructure> | structure | Subtype of <someParentStructure> defined in … |  |
| SP1 | 0:Byte[] | Setpoint 1 | False |
| Allow Subtypes | 0:ByteString | Some Bytestring | True |

Fields with abstract DataTypes shall have True in the “Allow Subtypes” column.

It is not allowed to add both columns to combine optional fields and fields that allow subtypes in one structure.

# General information to Asset Administration Shell and OPC UA

## Introduction to Asset Administration Shell

An Asset Administration Shell is the standardized digital representation of an asset. It carries the asset's identity and references the submodels that describe it, such as a nameplate, technical data, a carbon footprint or a bill of material. Submodels are identified in their own right and may be referenced by more than one shell.

Shells, submodels and concept descriptions carry globally unique identifiers. Other elements are named within their parent by a short name and are addressed by the path of short names that leads to them.

OPC 30270 v1.00 models the AAS v1.x metamodel in

http://opcfoundation.org/UA/I4AAS/. This specification models the incompatible AAS V3 metamodel

in http://opcfoundation.org/UA/I4AAS/v3/. A model version does not change NodeId identity, so the

two models use distinct namespace URIs and a Server may load both without rebinding a published

NodeId.

AAS V3 has AssetInformation and SpecificAssetId in place of the v1.x Asset and View

shapes, carries identifiers as bare strings, and distinguishes SubmodelElementList from

SubmodelElementCollection. This specification also carries the information required for a

lossless round trip and defines the AAS registry as an xRegistry domain model. Annex C gives an

informative concept correspondence for migration tooling.

## Introduction to OPC Unified Architecture

This is an OPC UA introduction that can be used as is, shortened or enhanced as appropriate.

### What is OPC UA?

OPC UA is an open and royalty free set of standards designed as a universal communication protocol. While there are numerous communication solutions available, OPC UA has key advantages:

- A state of art security model (see OPC 10000-2).

- A fault tolerant communication protocol.

- An information modelling framework that allows application developers to represent their data in a way that makes sense to them.

OPC UA has a broad scope which delivers for economies of scale for application developers. This means that a larger number of high-quality applications at a reasonable cost are available. When combined with semantic models such as Asset Administration Shell, OPC UA makes it easier for end users to access data via generic commercial applications.

The OPC UA model is scalable from small devices to ERP systems. OPC UA Servers process information locally and then provide that data in a consistent format to any application requesting data - ERP, MES, PMS, Maintenance Systems, HMI, Smartphone, or a standard Browser, for examples. For a more complete overview see OPC 10000-1.

### Basics of OPC UA

As an open standard, OPC UA is based on standard internet technologies, like TCP/IP, HTTP, Web Sockets.

As an extensible standard, OPC UA provides a set of Services (see OPC 10000-4) and a basic information model framework. This framework provides an easy manner for creating and exposing vendor defined information in a standard way. More importantly all OPC UA Clients are expected to be able to discover and use vendor-defined information. This means OPC UA users can benefit from the economies of scale that come with generic visualisation and historian applications. This specification is an example of an OPC UA Information Model designed to meet the needs of developers and users.

OPC UA Clients can be any consumer of data from another device on the network to browser based thin clients and ERP systems. The full scope of OPC UA applications is shown in Figure 1.

Figure 1 – The scope of OPC UA within an enterprise

OPC UA provides a robust and reliable communication infrastructure having mechanisms for handling lost messages, failover, heartbeat, etc. With its binary encoded data, it offers a high-performing data exchange solution. Security is built into OPC UA as security requirements become more and more important especially since environments are connected to the office network or the internet and attackers are starting to focus on automation systems.

### Information modelling in OPC UA

#### Concepts

OPC UA provides a framework that can be used to represent complex information as Objects in an AddressSpace which can be accessed with standard services. These Objects consist of Nodes connected by References. Different classes of Nodes convey different semantics. For example, a Variable Node represents a value that can be read or written. The Variable Node has an associated DataType that can define the actual value, such as a string, float, structure etc. It can also describe the Variable value as a variant. A Method Node represents a function that can be called. Every Node has a number of Attributes including a unique identifier called a NodeId and non-localized name called as BrowseName. An Object representing a ‘Reservation’ is shown in Figure 2.

Figure 2 – A basic Object in an OPC UA Address Space

Object and Variable Nodes represent instances and they always reference a TypeDefinition (ObjectType or VariableType) Node which describes their semantics and structure. Figure 3 illustrates the relationship between an instance and its TypeDefinition.

The type Nodes are templates that define all of the children that can be present in an instance of the type. In the example in Figure 3 the PersonType ObjectType defines two children: First Name and Last Name. All instances of PersonType are expected to have the same children with the same BrowseNames. Within a type the BrowseNames uniquely identify the children. This means Client applications can be designed to search for children based on the BrowseNames from the type instead of NodeIds. This eliminates the need for manual reconfiguration of systems if a Client uses types that multiple Servers implement.

OPC UA also supports the concept of sub-typing. This allows a modeller to take an existing type and extend it. There are rules regarding sub-typing defined in OPC 10000-3, but in general they allow the extension of a given type or the restriction of a DataType. For example, the modeller can decide that the existing ObjectType in some cases needs an additional Variable. The modeller can create a subtype of the ObjectType and add the Variable. A Client that is expecting the parent type can treat the new type as if it was of the parent type. Regarding DataTypes, subtypes can only restrict. If a Variable is defined to have a numeric value, a subtype could restrict it to a float.

Figure 3 – The relationship between Type Definitions and Instances

References allow Nodes to be connected in ways that describe their relationships. All References have a ReferenceType that specifies the semantics of the relationship. References can be hierarchical or non-hierarchical. Hierarchical references are used to create the structure of Objects and Variables. Non-hierarchical are used to create arbitrary associations. Applications can define their own ReferenceType by creating subtypes of an existing ReferenceType. Subtypes inherit the semantics of the parent but can add additional restrictions. Figure 4 depicts several References, connecting different Objects.

Figure 4 – Examples of References between Objects

The figures above use a notation that was developed for the OPC UA specification. The notation is summarized in Figure 5. UML representations can also be used; however, the OPC UA notation is less ambiguous because there is a direct mapping from the elements in the figures to Nodes in the AddressSpace of an OPC UA Server.

Figure 5 – The OPC UA Information Model notation

A complete description of the different types of Nodes and References can be found in OPC 10000-3 and the base structure is described in OPC 10000-5.

The OPC UA specification defines a very wide range of functionality in its basic information model. It is not required that all Clients or Servers support all functionality in the OPC UA specifications. OPC UA includes the concept of Profiles, which segment the functionality into testable certifiable units. This allows the definition of functional subsets (that are expected to be implemented) within a companion specification. The Profiles do not restrict functionality, but generate requirements for a minimum set of functionalities (see OPC 10000-7).

#### Namespaces

OPC UA allows information from many different sources to be combined into a single coherent AddressSpace. Namespaces are used to make this possible by eliminating naming and id conflicts between information from different sources. Each namespace in OPC UA has a globally unique string called a NamespaceUri which identifies a naming authority and a locally unique integer called a NamespaceIndex, which is an index into the Server's table of NamespaceUris. The NamespaceIndex is unique only within the context of a Session between an OPC UA Client and an OPC UA Server- the NamespaceIndex can change between Sessions and still identify the same item even though the NamespaceUri's location in the table has changed. The Services defined for OPC UA use the NamespaceIndex to specify the Namespace for qualified values.

There are two types of structured values in OPC UA that are qualified with NamespaceIndexes: NodeIds and QualifiedNames. NodeIds are locally unique (and sometimes globally unique) identifiers for Nodes. The same globally unique NodeId can be used as the identifier in a node in many Servers – the node's instance data can vary but its semantic meaning is the same regardless of the Server it appears in. This means Clients can have built-in knowledge of of what the data means in these Nodes. OPC UA Information Models generally define globally unique NodeIds for the TypeDefinitions defined by the Information Model.

QualifiedNames are non-localized names qualified with a Namespace. They are used for the BrowseNames of Nodes and allow the same names to be used by different information models without conflict. TypeDefinitions are not allowed to have children with duplicate BrowseNames; however, instances do not have that restriction.

#### Companion Specifications

An OPC UA companion specification for an industry specific vertical market describes an Information Model by defining ObjectTypes, VariableTypes, DataTypes and ReferenceTypes that represent the concepts used in the vertical market, and potentially also well-defined Objects as entry points into the AddressSpace.

# Use cases

An AAS toolchain compiles an environment into a deterministic AASEnvironmentType subtree and serializes that subtree back without losing fields, identity, order, absence or xsd value semantics. This supports generated Servers and lossless interchange between document and AddressSpace forms.

An OPC UA Client browses live shells and submodels, reads or writes submodel element values with the standard Services, and invokes an AAS operation through AASOperationType.Invoke.

A registry Client discovers shells and submodels, retrieves versioned documents and AASX packages, and follows federated references while preserving entity identity and applying the target resource's disclosure and authorization policy.

A product-passport implementation exposes public and controlled AAS content through one registry, retrieves historical revisions, and serves an environment document filtered to the calling Session's permissions.

# Asset Administration Shell information model overview

## Mapping rules

### General

An AAS Environment materializes as an AASEnvironmentType folder holding the shells, submodels

and concept descriptions it contains. Submodels are held by the environment, not nested inside

shells, because a submodel is not owned by the shell that references it; a shell carries references

to its submodels, and those references are the link.

AASEnvironmentType is a FolderType and not a subtype of the xRegistry RegistryType. An

environment holds the metamodel objects of one serialization — the unit an AASX package carries and

a source generator compiles — and its membership is whatever that serialization contained. A

registry holds documents about shells, versioned and federatable, and its membership is what the

Server catalogues. A Server may serve one, the other, or both over the same shells. Making

AASEnvironmentType a RegistryType would require every Server that materializes an AAS to

implement the registry half.

Where both halves are present, clause 6.5.2 defines the link between a catalogued shell and its

materialized node tree, and clause 6.5.10 requires the registry to serve the materialized environment

as retrievable AAS and AASX documents.

Every metamodel field has exactly one representation in the AddressSpace. Annex B lists

them all, field by field. A field with no entry in that annex is a defect in this specification, not

a field an implementation may drop.

### Canonical value representation

AAS types values with an xsd type. Clause 6.3.1 assigns each of the thirty values of DataTypeDefXsd

one OPC UA DataType, and no DataType is assigned to two of them. Where a built-in DataType denotes

the xsd type on its own it is used; where two xsd types would otherwise share one built-in,

clause 6.3.1 defines a subtype for one of them.

A value materializes as one Value Variable, whose DataType is the one clause 6.3.1 assigns to its

declared xsd type. The declared type is read from that DataType.

ValueType is a Mandatory Variable. The metamodel makes Property.valueType mandatory and

Property.value optional, and the same holds for Range, so a Property that declares xs:decimal

and carries no value is conformant AAS; clause 6.1.5 gives an absent field no node, leaving no value

node to carry the declaration. Where both are present a Server shall keep them consistent: the

Value node's DataType shall be the one clause 6.3.1 assigns to ValueType.

A value is compared in the xsd value space, not the lexical space. XML Schema defines a

datatype's value space and its lexical space separately, defines identity on the value space,

and designates a canonical lexical representation for each type. AAS carries a value as a string

in the lexical space and defines no equality on it: no normalization, no canonical form, and no

requirement that a lexical form survive a round trip. The ValueOnly serialization of AAS Part 2

renders a value as a native JSON number or boolean and back, so "1" declared xs:boolean returns

as "true".

A Server therefore:

- shall materialize a value into the DataType of clause 6.3.1, and

- shall serialize it back as the XSD canonical lexical representation of that type.

The canonical lexical representation is the one defined by XSD 1.1 Part 2: Datatypes. For

xs:decimal, the decimalCanonicalMap omits the fractional part where the value is integral:

"1" therefore serializes as "1", not as the XSD 1.0 form "1.0". The same XSD 1.1

canonical mapping governs the exponent form serialized for xs:float and xs:double.

A Property authored as "1.500000" with ValueType xs:decimal therefore serializes as "1.5",

and one authored "+42" with xs:int serializes as "42". The documents are equivalent under

clause 6.4, not identical. An implementation that requires the authored lexical form to survive

verbatim expresses that in the metamodel, with a qualifier or an IEC 61360 valueFormat.

AASValueString is used only where a Structure field carries a value whose declared type is given

by a sibling field of the same Structure (clause 6.3.2). It is never the DataType of a Variable.

### NodeId and BrowseName assignment

Assignment is deterministic: two implementations materializing the same AAS produce the same nodes.

NodeId. Instance nodes use String identifiers in the Server's own namespace. The identifier is

a node-kind discriminator followed by decimal length prefixes and reversibly escaped source

components:

Table 1 – NodeId and BrowseName assignment

| Node | String identifier |
| --- | --- |
| Asset Administration Shell | i4aas3:A:<n>:E(<id>) |
| Submodel | i4aas3:S:<n>:E(<id>) |
| Concept Description | i4aas3:C:<n>:E(<id>) |
| Submodel Element | i4aas3:E:<n>:<m>:E(<owner-id>)E(<idShortPath>) |

E is applied independently to every raw identifier and path component. It scans the source

Unicode scalar values without normalization. A literal %, every C0 control from U+0000 through

U+001F, and every C1 control from U+007F through U+009F is encoded as its UTF-8 bytes, with each

byte written as %HH using uppercase hexadecimal digits. Every other scalar value is copied

unchanged. Decoding accepts only this canonical form: a raw C0 or C1 control, a malformed escape,

or an escape whose decoded value would not be escaped by E is invalid. This also makes a literal

percent unambiguous: % encodes as %25.

<n> and <m> are the numbers of Unicode code points in the encoded components that follow,

written with ASCII decimal digits and without leading zeroes. For an identifiable, <n> is the

length of E(<id>). For an element, <n> is the length of E(<owner-id>) and <m> is the length of

E(<idShortPath>); those lengths split the concatenated payload before either component is

decoded, without relying on a delimiter that a source string may contain. A, S, C and E

distinguish the four node kinds.

The idShortPath is the metamodel's own path convention: short names joined by ., with [n] for

a member of a list. The encoding is reversible and collision-free: for example, a shell whose

identifier is a#b encodes as i4aas3:A:3:a#b, while element path b beneath owner a encodes as

i4aas3:E:1:1:ab. An identifier containing LF between a and b encodes as

i4aas3:S:5:a%0Ab; NUL in the same position encodes as i4aas3:S:5:a%00b; U+0085 by itself

encodes as i4aas3:S:6:%C2%85; and the three source characters %0A encode as

i4aas3:S:5:%250A, not as a line feed.

An OperationVariable wrapper is not a node. Its value element has the path

<operation-path>.<role>[<index>], where <role> is exactly inputVariables,

outputVariables or inoutputVariables and <index> is its zero-based position in that role.

For example, the first input value of AnOperation has path

AnOperation.inputVariables[0]. The value element's own idShort remains its BrowseName and

IdShort Property; the role and index, rather than that short name, identify its containment

position.

OPC UA limits a String NodeId identifier to 4096 characters. Before creating any nodes for one

identifiable, a materializer shall derive every identifier in its subtree. If any derived

identifier, after escaping and adding its discriminator and length prefixes, exceeds that limit, it

shall reject that identifiable without exposing a partial subtree and shall report the

overlength path in its diagnostic. It shall not truncate, replace or hash the source strings,

because those operations would not implement the reversible encoding above.

BrowseName. A Referable that has a non-empty idShort uses that exact short name in the

Server's namespace. The three top-level Identifiables permit idShort to be absent. In that case

the BrowseName is <kind>_<digest>, where <kind> is exactly AssetAdministrationShell,

Submodel or ConceptDescription, and <digest> is the lowercase hexadecimal SHA-256 digest of

the exact, non-normalized UTF-8 bytes of id. The raw identifier is not included in the

BrowseName.

Before allocating derived BrowseNames, a materializer shall reserve every BrowseName supplied

by an idShort among the environment's top-level children. Identifiers that produce one derived

base name are processed in ascending lexicographic order of their UTF-8 bytes. The first available

name is used: the unsuffixed base where it is free, otherwise the base followed by _n, where n

is the smallest non-negative ASCII decimal integer that makes the name unused. This rule handles both

a SHA-256 collision and a collision with an authored idShort without making source array order

significant. Duplicate identifiers within one identifiable kind would produce the same NodeId and

shall be rejected.

An element inside a SubmodelElementList has no short name — the metamodel does not give it one —

so its BrowseName is its index rendered as a decimal string. Order is carried by the Index

Property, not by the BrowseName, because a BrowseName is a name and not a position.

DisplayName. The short name where one exists; otherwise the index for a list member or the

derived BrowseName for a top-level Identifiable.

### Ordering

SubmodelElementList.orderRelevant says whether the order of a list's members carries meaning; when

it is false the metamodel states that the list represents a set or a bag. OPC UA can say this in the

type system rather than in a Property, and it does:

- Where orderRelevant is true — including where it is absent, since its default is true — a SubmodelElementList shall reference its members with HasOrderedComponent (i=49), the subtype of HasComponent whose semantic is that the order of the references is meaningful.

- Where orderRelevant is false, it shall reference them with HasComponent.

- Every other parent/child relationship between Submodel Elements shall use HasComponent.

The ReferenceType carries orderRelevant; there is no OrderRelevant Property.

AASSubmodelElementListType.<Element> is declared with HasComponent; an ordered instance uses

the HasOrderedComponent subtype for that declaration.

Index carries the position. The Browse Service is not required to return references in any

particular order, and a NodeSet is a set of references rather than a sequence, so the order of a

Browse result is not a reliable source for it. Index is Optional on a list member and RECOMMENDED

wherever HasOrderedComponent is used; an implementation claiming AAS-LosslessRoundTrip shall

materialize it there. Where a Server materializes Index, the values shall be the positions

0 … n-1 without gaps or repeats, and a serializer emits members in Index order.

Where the members are referenced with HasComponent, a serializer may emit them in any order,

and clause 6.4 compares the collection as a bag.

The three Operation variable roles are separately ordered arrays. Their value elements are direct

HasComponent children, not HasOrderedComponent children. Each value element shall carry

Index equal to its zero-based position within its role. The array position is authoritative; the

Index Property and the role/index in the NodeId of clause 6.1.3 shall agree with it. Indices

start again at zero for each role.

### Absent versus empty

An optional field that is absent and one present but empty are distinct in the metamodel:

- An absent optional field has no node.

- A field present but empty has a node whose value is an empty array, or an Object with no children.

A Server shall not materialize a node for an absent field, and shall not omit one for a

present-but-empty field. A serializer distinguishes the two by the presence of the node, never by

its value.

The rule presupposes that the field has a node of its own, and five do not. submodelElements,

SubmodelElementCollection.value, SubmodelElementList.value, Entity.statements and

AnnotatedRelationshipElement.annotations materialize as components of the parent node rather than

as a node holding a collection (Annex B), so an absent one and a present-but-empty one both leave

the parent with no children of that kind and no marker distinguishes them. For these five fields

only, the distinction is therefore not preserved: a serializer shall emit the field as

absent where the parent has no corresponding children, and clause 6.4 compares them accordingly.

Collapsing the distinction is deliberate rather than an oversight. Preserving it would require a

marker node on every collection-bearing element, which would appear in every AddressSpace to record

a difference the metamodel draws but nothing observable depends on, and clause 6.4 would still have

to special-case it. Every other optional field keeps the distinction, because every other optional

field has a node.

### Instance materialization

Materializing an Environment is mechanical:

- Create an AASEnvironmentType folder.

- For each shell, submodel and concept description, create a node of the corresponding type, with the NodeId of clause 6.1.3 and the BrowseName of clause 6.1.3, organized by the environment folder.

- For each element within a submodel, recursively create a node of the type corresponding to its metamodel class, referenced by its parent with HasComponent.

- For each field present on the element, create the member node named in Annex B, with the value the metamodel carries. Omit members for absent fields, per clause 6.1.5.

- For a value-bearing element, set ValueType, and set Value with the DataType clause 6.3.1 assigns to it, per clause 6.1.2.

- Reference the members of a SubmodelElementList with HasOrderedComponent where its orderRelevant is true and HasComponent where it is false, and set each member's Index to its position, per clause 6.1.4.

No step in that sequence is implementation-defined. A generator that implements it compiles an AAS

into a loadable NodeSet, and a Server that loads the NodeSet serves the AAS.

## AAS metamodel ObjectTypes

The companion namespace is http://opcfoundation.org/UA/I4AAS/v3/, model version 3.00-draft3.

Draft numeric NodeIds use the 1001+ block; final NodeIds are assigned by the OPC Foundation. The

normative node reference is Annex A; this clause describes intent.

### Abstract bases

The abstract bases mirror the metamodel's own hierarchy, so that an element carries the members its

metamodel class gives it and no others: AASReferableType for everything with a short name,

AASIdentifiableType for the three classes with a globally unique identifier, and

AASHasSemanticsType, AASHasKindType, AASHasDataSpecificationType and AASQualifiableType for

the orthogonal aspects.

AASReferableType carries a Mandatory ModelType, the metamodel class name. It is redundant with

the ObjectType, and it is carried anyway: a serialization produced from the AddressSpace must be

byte-identical to the one that produced it, and the metamodel's serialization includes this

discriminator.

AASIdentifiableType carries the Id — up to 2048 characters of arbitrary text, which is why

identity lives in a Property and in the encoded String NodeId rather than in the BrowseName

(clause 6.1.3).

<!-- model-figure: root=ns=2;i=1001 require=mandatory external=BaseObjectType -->

Figure 2 – AASReferableType and the identity it gives every element

Figure 2 — AASReferableType, and the identity it gives every element

<!-- model-figure: root=ns=2;i=1003 require=mandatory external=BaseObjectType -->

Figure 3 – The orthogonal aspect bases

Figure 3 — The orthogonal aspect bases

### Environment, shell and asset information

AASEnvironmentType is the container and the root a generator materializes into. Shells, submodels

and concept descriptions are all held by it directly: a submodel is not owned by the shell that

references it, and one submodel may be referenced by several shells, so nesting them inside shells

would misrepresent the model.

AASType is a shell. It holds AssetInformation, references to its submodels, and the derivation

link from an instance to its type.

<!-- model-figure: root=ns=2;i=1010 require=mandatory external=FolderType -->

Figure 4 – AASEnvironmentType, the container a generator materializes

Figure 4 — AASEnvironmentType, the container a generator materializes

<!-- model-figure: root=ns=2;i=1011 require=mandatory -->

Figure 5 – AASType and the asset identity it carries

Figure 5 — AASType and the asset identity it carries

### Submodel and concept description

AASSubmodelType is a submodel, holding its elements. AASConceptDescriptionType is the definition

a semantic identifier resolves to — what makes two submodels from different vendors comparable.

<!-- model-figure: root=ns=2;i=1013 require=mandatory -->

Figure 6 – AASSubmodelType and AASConceptDescriptionType

Figure 6 — AASSubmodelType and AASConceptDescriptionType

### Submodel elements

The element types cover the metamodel's element set. Every one of them subtypes

AASSubmodelElementType, which carries the semantics, qualifiers and data specifications an element

may have, and the Index that gives a list member its position (clause 6.1.4).

<!-- model-figure: root=ns=2;i=1020 require=mandatory -->

Figure 7 – The submodel element hierarchy

Figure 7 — The submodel element hierarchy

Three element types deserve note, and they are the three the losslessness rules bear on.

AASPropertyType carries a value once, in a Value node whose DataType is the one clause 6.3.1

assigns to the declared ValueType (clause 6.1.2). ValueType is Mandatory because the metamodel

makes it mandatory while making the value itself optional. AASRangeType carries its bounds the

same way, and an absent bound means unbounded rather than zero.

<!-- model-figure: root=ns=2;i=1021 require=mandatory -->

Figure 8 – A value and the type it is declared as

Figure 8 — A value and the type it is declared as

AASSubmodelElementListType declares its member placeholder with HasComponent. An instance

specializes that reference to HasOrderedComponent where the list's order is relevant and retains

HasComponent where it is not; its members carry Index where the position has to be recoverable.

AASSubmodelElementCollectionType is unordered and its members are identified by their own short

names.

<!-- model-figure: root=ns=2;i=1031 require=mandatory -->

Figure 9 – Ordered and unordered collections

Figure 9 — Ordered and unordered collections

AASEntityType is a component of a composition; a self-managed entity carries the identifier of

its own shell, so a bill of material is traversable across organizations.

AASOperationType carries its variables as references to the element nodes that hold them,

rather than duplicating those elements, so an operation's variables round-trip as the elements they

are.

For every OperationVariable in a present role array, a materializer shall create the wrapper's

mandatory value as one direct HasComponent child of the AASOperationType node. It shall not

create a node for the wrapper itself. The corresponding InputVariables, OutputVariables or

InoutputVariables array entry shall be one AASOperationVariableDataType whose

ValueNodeId is the local NodeId of that child. A child shall occur in exactly one role entry.

The child uses its concrete AAS ObjectType and its own idShort as BrowseName; its NodeId and

Index follow clauses 6.1.3 and 6.1.4. An absent role has no corresponding Property instance, while a

present empty role has a Property whose value is an empty array.

<!-- model-figure: root=ns=2;i=1032 require=mandatory -->

Figure 10 – Composition, operations and events

Figure 10 — Composition, operations and events

### Invoking an operation

An Operation submodel element is invocable. AASOperationType carries the Method Invoke, whose

arguments correspond positionally to the element's InputVariables, OutputVariables and

InoutputVariables.

Table 2 – Invoking an operation

| Argument | Direction | Corresponds to |
| --- | --- | --- |
| InputValues | in | inputArguments of the AAS API request, in the order of InputVariables |
| InoutputValues | in | inoutputArguments of the request, in the order of InoutputVariables |
| ClientTimeout | in | clientTimeoutDuration; zero selects the Server's default |
| OutputValues | out | outputArguments of the result, in the order of OutputVariables |
| InoutputResults | out | inoutputArguments of the result |
| Success | out | success |
| Diagnostic | out | the message of a failed execution |

A Server shall return Bad_InvalidArgument where the number of values does not match the number

of variables the element declares.

Success reports the outcome of the operation, not of the Call. An operation that runs and reports

failure returns Good with Success false; a Call that could not be made at all returns a bad

StatusCode. Conflating the two would leave a Client unable to distinguish an unreachable Server from

a rejected workpiece.

Positional correspondence, rather than a name-keyed structure, is what the metamodel supports:

InputVariables is an ordered array and clause 6.1.4 already preserves that order, so the n-th value

belongs to the n-th variable in both directions.

The AAS API of IDTA-01002 Part 2 also defines an asynchronous form, InvokeOperationAsync with

GetOperationAsyncResult. This specification defines no counterpart: an OPC UA Method Call is

synchronous, and a Server whose operations outlive a Call implements the Program interface of

OPC 10000-10 on the element rather than a second Method here. Annex G records the correspondence.

## AAS DataTypes

### The xsd type mapping

Each of the thirty values of AASDataTypeDefXsdDataType is assigned one OPC UA DataType, and no

DataType is assigned to two of them. A serializer reads the declared xsd type from the DataType of

the value node.

Where a built-in DataType denotes the xsd type on its own, it is used. Where two xsd types would

otherwise share one built-in, a subtype is defined in this namespace, as OPC UA does in deriving

DecimalString, DurationString, DateString and TimeString from String. Ten such subtypes

are defined.

Table 3 – The xsd type mapping

| ValueType | OPC UA DataType | Note |
| --- | --- | --- |
| xs:boolean | Boolean (i=1) |  |
| xs:byte | SByte (i=2) | xsd byte is signed |
| xs:unsignedByte | Byte (i=3) |  |
| xs:short | Int16 (i=4) |  |
| xs:unsignedShort | UInt16 (i=5) |  |
| xs:int | Int32 (i=6) |  |
| xs:unsignedInt | UInt32 (i=7) |  |
| xs:long | Int64 (i=8) |  |
| xs:unsignedLong | UInt64 (i=9) |  |
| xs:float | Float (i=10) |  |
| xs:double | Double (i=11) |  |
| xs:decimal | Decimal (i=50) | arbitrary precision, and its Scale preserves the authored number of decimal places |
| xs:integer | Integer (i=27) |  |
| xs:nonNegativeInteger | UInteger (i=28) |  |
| xs:positiveInteger | AASPositiveInteger | subtype of UInteger, so it does not collide with xs:nonNegativeInteger |
| xs:nonPositiveInteger | AASNonPositiveInteger | subtype of Integer |
| xs:negativeInteger | AASNegativeInteger | subtype of AASNonPositiveInteger, mirroring the xsd restriction hierarchy |
| xs:string | String (i=12) |  |
| xs:anyURI | AASAnyUri | subtype of String, so it does not collide with xs:string |
| xs:dateTime | DateTime (i=13) |  |
| xs:date | DateString (i=12881) | a date is a day, not an instant |
| xs:time | TimeString (i=12880) | a time-of-day has no day |
| xs:duration | DurationString (i=12879) | the ISO 8601 duration form; not Duration (i=290), which is a count of milliseconds |
| xs:gYear | AASGYear | a Gregorian period; OPC UA has no DataType denoting one |
| xs:gYearMonth | AASGYearMonth |  |
| xs:gMonth | AASGMonth |  |
| xs:gMonthDay | AASGMonthDay |  |
| xs:gDay | AASGDay |  |
| xs:base64Binary | ByteString (i=15) |  |
| xs:hexBinary | AASHexBinary | subtype of ByteString; the octets are identical to a xs:base64Binary value's and only the written form differs |

Four rows are qualified further.

xs:duration is assigned DurationString, not Duration (i=290). Duration is a Double

counting milliseconds; xs:duration has year and month components that are not a fixed number of

milliseconds, as P1M is not thirty days. DurationString holds the ISO 8601 form, which is the

lexical space of xs:duration.

xs:date and xs:time are assigned DateString and TimeString, not DateTime. A DateTime is

an instant; a date is a day in some timezone and a time is a time-of-day with no day. Assigning

either DateTime would require a value for the missing component.

The five partial-date types have no OPC UA DataType, built-in or derived, that denotes a period.

Each is assigned its own String subtype.

Two assignments have a range narrower than the xsd type. Integer and UInteger are the abstract

unions of OPC UA's concrete integer types, so their range is that of Int64 and UInt64, whereas

xs:integer is unbounded; and DateTime begins in 1601, whereas xs:dateTime does not. A value

outside the representable range shall be rejected rather than truncated, as in clause 6.3.3.

### AASValueString

AASValueString is a subtype of String (i=12). It carries the xsd lexical form of a value whose

declared type is given by a sibling field of the same Structure.

A Structure field has one static DataType and cannot vary with a declared type.

AASQualifierDataType and AASExtensionDataType pair a value with a ValueType field.

AASDataSpecificationIec61360DataType pairs its value with a DataType field. In each case the

value field is lexical and the sibling field states how to read it. Where a Variable carries a

value, clause 6.3.1 assigns the DataType of its declared xsd type instead.

A Server shall not use AASValueString as the DataType of a Variable.

### Enumerations

The enumerations are closed. AASKeyTypesDataType, AASDataTypeDefXsdDataType and the rest

enumerate exactly the metamodel's values; a value outside the enumeration cannot round-trip, so an

implementation rejects it rather than dropping it silently.

### Structures

The structures carry the metamodel's value classes: references and their ordered keys,

language-tagged strings, specific asset identifiers, administrative information, qualifiers,

extensions, data specifications and their IEC 61360 content. AASQualifierDataType and

AASExtensionDataType pair a value with ValueType;

AASDataSpecificationIec61360DataType pairs it with DataType. Each carries the value as

AASValueString for the reason given in clause 6.3.2.

AASReferenceDataType carries its Keys as an ordered array. The order is part of the reference's

meaning — it is the path — so it is preserved exactly.

IEC 61360 permitted values are structured rather than flattened into strings.

AASValueReferencePairDataType has Mandatory Value and ValueId fields.

AASValueListDataType has a Mandatory, non-empty ValueReferencePairs array of those pairs.

AASLevelTypeDataType has the four Mandatory Boolean fields Min, Nom, Typ and Max.

AASDataSpecificationIec61360DataType.ValueList and .LevelType are Optional, so their absence

remains distinct from a present structured value.

## Round-trip conformance

An implementation claiming the AAS-LosslessRoundTrip conformance unit shall satisfy both

directions.

Materialize and serialize. For any conformant AAS environment, materializing it per clause 6.1.6

and serializing the result shall produce an environment equivalent to the original.

Serialize and materialize. For any AddressSpace subtree produced by clause 6.1.6, serializing it

and materializing the result shall produce a subtree with the same nodes, NodeIds, BrowseNames,

References and values.

Two environments are equivalent when, after canonical ordering of JSON object members:

- every field present in one is present in the other, and absent in one is absent in the other;

- every value is the same element of the xsd value space of its declared valueType, compared per XSD 1.1 Part 2: Datatypes. "1.500000" and "1.5" are equivalent as xs:decimal; "1" and "true" are equivalent as xs:boolean; "1.5" and "2.5" are not;

- every array to which the metamodel gives an order is compared in order: the keys of a reference, a multi-language value, an operation's variables, and a SubmodelElementList whose orderRelevant is true;

- a SubmodelElementList whose orderRelevant is false is compared as a bag: same members, same multiplicities, order disregarded;

- the five container fields of clause 6.1.5 that materialize as parent components rather than as a node of their own — submodelElements, SubmodelElementCollection.value, SubmodelElementList.value, Entity.statements and AnnotatedRelationshipElement.annotations — are compared with absent and present-but-empty treated alike, since the mapping does not distinguish them.

No further tolerance applies. A field that cannot be represented is a defect in this specification;

Annex B is the list against which that is checked.

runs both directions over it. The corpus covers every element type, nested and ordered lists,

elements without short names, multi-language values, non-canonical lexical forms of the xsd types,

the absent-versus-empty distinction, qualifiers, extensions, data specifications and multi-key

references.

value rather than re-writing it, canonicalizing an xs:decimal through a fixed working precision so

that digits are lost, restoring an ordered list in Browse order rather than by Index, conflating an

absent field with an empty one — and asserts that the comparison reports each. It also asserts that

re-writing a value into its canonical lexical form is not reported.

## The AAS Registry

### The registry is folders of files

The registry half projects a catalogue of shells onto the AddressSpace using the abstract

OPC UA — xRegistry base model: a registry and its groups are FolderType folders, and a resource

document is a FileType file, read with the File Transfer Methods of OPC 10000-20.

It answers different questions from the metamodel half:

Table 4 – The registry is folders of files

|  | Metamodel clauses | Registry clauses |
| --- | --- | --- |
| Unit | one shell's metamodel, in nodes | a catalogue of shells, as folders and files |
| Granularity | per element | per document |
| Access | Read, Write, Call | Open, Read, Close |
| Answers | what is this value now | which shells exist, at what versions, where else served |

A Server may implement either half or both. Where both are present the same shell appears twice, and

AASShellGroupType.ShellNode links the catalogue entry to the live node tree.

### Registry types

AASRegistryType is the registry root, exposed as a well-known AASRegistry Object under the

Server Object so that any Client reaching the standard Server object discovers it. Its group

folders hold shells, submodel template families, concept dictionaries and package stores; its

Methods answer the discovery question and provide a document fast path.

The UANodeSet representation of the concrete LookupShellsByAssetLink, GetSubmodel and

Materialize Methods on the well-known AASRegistry Object shall set the

MethodDeclarationId XML attribute to the NodeId of the same-named Method declaration on

AASRegistryType.

AASShellGroupType holds the submodel documents of one shell. It is a GroupType, and therefore a

folder of resource files, whereas AASType is the shell's metamodel object tree. They are separate

types because they are separate things, and three consequences follow that a single type could not

give:

- Either half can exist without the other. A registry that catalogues a shell served by another Server has no metamodel tree to attach to, and a Server that materializes an AAS from a package need not catalogue it. Conflating the two would make each imply the other.

- A group is a folder of files; a shell is an object. The base GroupType gives the catalogue entry its xRegistry attributes, its creation Methods and its file members. AASType gives the shell its AssetInformation and its submodel references. Neither set belongs on the other.

- They have different lifetimes. A catalogued shell has versions, and the current version of its submodel document need not be what the metamodel tree currently holds — that is the whole point of clause 6.5.4.

Where a Server implements both halves, AASShellGroupType.ShellNode points at the AASType node

for the same shell, and both carry the same identifiers, so a Client can move between the catalogue

and the live tree without re-resolving anything.

AASSubmodelFileType is one submodel document. AASConceptDescriptionFileType and

AASPackageFileType are the corresponding resources for concept definitions and packages. Every

package carries the strong integrity metadata defined in clause 6.5.4.

<!-- model-figure: root=ns=2;i=1100 require=mandatory external=RegistryType,GroupType,ResourceType,Server -->

Figure 11 – The registry root, shells and their submodel documents

Figure 11 — The registry root, shells and their submodel documents

The other three group types follow the same shape: a group folder holding resource files, each

naming the source identity its identifier is derived from.

<!-- model-figure: root=ns=2;i=1103 require=mandatory external=GroupType,ResourceType -->

Figure 12 – Templates, concept dictionaries and package stores, each with its source identity

Figure 12 — Templates, concept dictionaries and package stores, each with its source identity

### Identifiers

Base clause 6.9 requires every group and resource type to name exactly one source identity and to

expose it verbatim as a Mandatory Property, and derives the identifier from it by the symbolic

construction defined there. This clause names them:

Table 5 – Identifiers

| Type | Source identity |
| --- | --- |
| AASShellGroupType | AasIdentifier |
| AASSubmodelFileType | SubmodelIdentifier |
| AASSubmodelTemplateGroupType | TemplateNamespace |
| AASConceptDictionaryGroupType | DictionaryIdentifier |
| AASConceptDescriptionFileType | ConceptIdentifier |
| AASPackageStoreGroupType | StoreIdentifier |
| AASPackageFileType | PackageIdentifier |

These are the same source identities the xRegistry AAS model names, and the construction is the same

one. The same shell therefore receives the same identifier whether it is served over OPC UA or over

HTTP, so the two bindings address one registry.

An identifier is never derived from a document. A resource is a stable umbrella over its versions,

so its identifier is invariant while its document changes; a content digest is version-level

metadata and identifies bytes, not entities.

### Versioning and the lifecycle record

The AAS metamodel records a single current revision. AdministrativeInformation carries a version

label and a revision label, and nothing retains what a submodel previously said or distinguishes a

correction from a new observation.

The registry supplies what the metamodel does not: each version of an AASSubmodelFileType is one

revision, ordered in time, and a Client asking for a submodel as it stood at a given moment reads

the newest version not later than that moment. This is not a convenience. Where a regulation

requires a record retrievable as of a date, or an auditable and tamper-evident history of changes to

controlled data, a Server implementing only the metamodel half has nothing to answer from.

The AAS version labels are carried unchanged in Administration; they are not reflected into the

registry's version identifiers, which follow the base model's own rules.

Package integrity. Every Version of an AASPackageFileType shall instantiate Digest and

DigestAlg, and both Properties shall be immutable within that Version. Digest shall be

the lowercase hexadecimal digest of the exact package blob bytes returned by FileType.Read and

shall not contain an algorithm prefix. DigestAlg is case-sensitive and shall be exactly

Sha256, Sha384 or Sha512; every other algorithm or casing shall be rejected. An OCI

descriptor algorithm of sha256, sha384 or sha512 shall map respectively to Sha256,

Sha384 or Sha512.

Before publishing these Properties or making the package available for reading or materialization,

the Server shall verify the exact returned blob against them. Before parsing, materializing or

otherwise using a retrieved package, a Consumer shall independently recompute Digest over

the exact returned bytes using DigestAlg and compare it with the published value.

An OCI-backed package Version shall instantiate immutable ManifestDigest as the exact digest

of the OCI manifest bytes, including a lower-case algorithm prefix and lower-case hexadecimal

value, for example sha256:<lowercase-hex>. ManifestDigest verifies only those exact manifest

bytes and shall not be treated as verification of the returned package blob. Before publishing

or using the Version, respectively, the Server and Consumer shall perform all of these checks:

- recompute and compare ManifestDigest over the exact manifest bytes;

- parse the verified manifest and require exactly one package-layer descriptor;

- require that descriptor's algorithm and encoded digest, after the mapping above, equal DigestAlg and Digest; and

- retrieve the package blob and independently recompute and compare Digest using DigestAlg.

ManifestDigest shall be the sole authority for OCI Version identity, and VersionId shall be the always-hashed symbolic identifier of its exact value. A raw OCI tag may locate a current manifest only as a mutable Resource-level alias and shall never be a VersionId. The tag shall match [A-Za-z0-9_][A-Za-z0-9_.-]{0,127} and shall be preserved byte-for-byte, including case and a leading underscore. Moving a tag to a previously unseen manifest shall create and retain a distinct immutable Version and shall not mutate or replace the old Version. The xRegistry base ResourceType declares the Attestations attribute, and AASAttestationDataType is the AAS payload type for that attribute. An AASPackageFileType Version shall not instantiate Subject or Attestations: the prohibition is on the package Version, not on use of the DataType. An attestation or other OCI referrer shall be represented as a separate immutable Resource and shall not become a Version of the package Resource it refers to. Adding, removing or discovering a referrer shall not change that package Resource's Version collection, default Version, document, attributes, Epoch or ModifiedAt.

### Discovery and resolution

LookupShellsByAssetLink answers the discovery question — given an asset key such as a serial

number or a manufacturer part identifier, which shells describe it — without the Client browsing the

whole collection. GetSubmodel returns a document and enough metadata to parse it, for a Client that

holds an identifier rather than a node.

GetSubmodel shall first resolve the selected AASSubmodelFileType and then apply, for the

calling Session, the exact effective RolePermissions, UserRolePermissions, disclosure tier,

authorization policy and access decision that direct FileType.Open and FileType.Read apply to

that target. Permission to Call the registry Method shall not authorize the target resource.

Where target access is denied, the Method shall return Bad_UserAccessDenied, or

Bad_NotFound where the Server's policy conceals the target's existence. Before successful target

authorization it shall not return document bytes, Format, ContentType, size, digest or other

target metadata, and shall use the same externally observable failure behavior, including

response timing, for a concealed unauthorized target and a nonexistent target.

A Server should bound the results returned for an unauthenticated collection query. A registry

serving regulated product data is subject to requirements to prevent bulk extraction of its

contents, and an unbounded collection endpoint is exactly such an extraction surface.

### Federation

Federation follows the base model. A shell or submodel this registry describes but does not host

carries an ExternalReference — an ExpandedNodeId whose ServerUri identifies the hosting

endpoint and whose NamespaceUri and identifier identify the entity — or a ResourceUrl for a

registry served over a different protocol.

The identity rule is absolute: identity is carried by the AAS identifier attributes and the

identifier derived from them, never by an endpoint. A Server exposing a local proxy for a remote

entity shall retain the remote entity's identifier attributes and shall not treat its own

endpoint as part of that entity's identity. The external authority identifies the serving endpoint,

not the entity.

Every resolution of ExternalReference.ServerUri or ResourceUrl is an egress operation over

untrusted registry metadata. A resolver shall apply the fail-closed federation policy of the

AAS xRegistry binding before opening a connection: configured scheme, host and port allowlists;

DNS resolution and connected-address checks that reject loopback, link-local, private,

unique-local, unspecified, multicast, reserved and cloud-metadata destinations unless explicitly

trusted; revalidation on every redirect and after connection to prevent DNS rebinding; finite

redirect, time, response and decompressed-size bounds; and no ambient cookies, credentials,

authorization headers or proxy credentials. A credential configured for one peer shall not be

sent to another peer or redirect target.

For an OPC UA peer, the resolver shall validate the endpoint certificate against its configured

trust list and shall require the certificate ApplicationUri, the Server ApplicationUri returned

by discovery and the configured federation-peer identity to agree. A policy, DNS, redirect,

credential, certificate, identity or resource-bound failure shall terminate resolution without

returning or caching any bytes obtained from the rejected destination.

Because the construction of clause 6.5.3 is deterministic, the same shell has the same identifier in

every registry that describes it, and a Client moving between registries re-resolves nothing.

### Information disclosure tiers

Some assets carry data that cannot be shown to everyone: a product passport is public in part and

restricted in part, and a supplier's technical data is commercially sensitive.

An information disclosure tier is the class of caller an entity's content may be released to.

This specification defines two, Public and Controlled, carried by the DisclosureTier Property.

They classify the information, not the caller and not the transport: a tier says what kind of

release this entity's content is, and leaves who may obtain it to the authorization the entity

advertises. A regulation that names finer classes maps them onto these two by deciding, for each,

whether the content is readable without authentication.

Note that a disclosure tier is metadata about content, and is therefore itself disclosed. A Server

that must not reveal even the existence of controlled content omits those entries entirely rather

than marking them; see the mitigations below.

This specification expresses two of the three things tiering requires, and does not express the

third.

Segmentation is expressible: a registry serves public content as stored documents and represents

controlled content as entries carrying a ResourceUrl or an ExternalReference instead of bytes.

Advertisement is expressible: DisclosureTier records whether an entity is readable without

authentication, and Authorization describes the authorization options a Consumer may use — a type,

a mechanism, and the authority and resource URIs that say where authorization is obtained and what

for. Authorization is authorization configuration only and shall not carry credentials, keys

or tokens, which are supplied out of band.

Enforcement at element granularity is not expressible. Where a regulation requires access rights

to be enforced between two elements within one document, this model cannot express it: a registry

resource document is opaque bytes, and a decision that falls inside a document cannot be taken by a

model that addresses documents.

OPC UA's own access control does not close that gap. It operates on nodes, and the boundary in

question is inside a file. Two mitigations are conformant: a registry may publish tier-specific

resources whose documents are already the redacted projection appropriate to a tier, so that every

document is wholly one tier; and a registry may omit controlled entries entirely from responses

to unauthenticated Clients, since advertising that a controlled submodel exists is itself a

disclosure.

A registry that serves public data shall not require authentication to read it.

The same disclosure decision applies through every access path. In particular, the

GetSubmodel convenience Method is subject to the target-resource authorization and

existence-concealment rule of clause 6.5.5; its Method-level Call permission is never a substitute for

the target's disclosure and file-access controls.

### The xRegistry API over OPC UA

The registry subtree is simultaneously an xRegistry API server: the operations are realized natively

by OPC UA Services over the same nodes, as defined by the base model and its API binding. Annex D

gives the correspondence to the HTTP binding for readers who know that one.

### Updateable registry

Clauses 6.5.1 to 6.5.8 describe a registry that catalogues documents. A Server may additionally make

the registry updateable: a Client writes a document into the registry, and the AddressSpace of

clause 6.1 changes to match. This clause defines that profile. It is optional, it is declared by the

AAS-UpdateableRegistry conformance unit, and a Server that implements clauses 6.5.1 to 6.5.8 without

it is fully conformant.

The documents are canonical; the nodes are derived. A Server implementing this profile shall

be able to rebuild the entire materialized subtree from the stored documents and this specification

alone, with no additional state. A Client shall not assume that a materialized node survives a

change to the document it came from, except as the generation rules below allow.

Materialization is generational, and a switch is atomic. A Server shall not let a Client

observe a half-built subtree. It prepares the new nodes as a shadow generation beside the active

one, with the document's LoadState set to Loading; it validates them; and only then does it

switch, making them visible, setting LoadState to Active and incrementing

MaterializationGeneration. Nodes in a shadow generation are not browsable and raise no events.

A superseded generation is retired under one documented policy. After a switch the previous

generation becomes Superseded. A Server shall apply exactly one of two policies, deterministic

for a given call and documented by the implementation:

- Graceful. The superseded generation enters Retiring. Existing MonitoredItems, continuation points and in-flight requests continue to be served from it until they drain or are migrated; new requests use the active generation. Its nodes are removed only once the retained work has drained.

- Immediate. The superseded generation is retired without waiting. The Server shall invalidate every affected MonitoredItem: a data-change item reports BadNodeIdUnknown, an event item stops producing events, and subsequent monitored-item operations on either return BadNodeIdUnknown.

Graceful retirement preserves subscriptions across an update; immediate retirement applies where a

Server cannot hold two generations at once. A Server shall apply one of these two and **shall

not** apply a third behaviour.

A failed document does not destroy anything. Validation failure shall leave the stored

document in place, set its LoadState to Failed and record the reason. The generation that was

active shall keep serving, unchanged. DesiredVersionId records the version an operator wants

materialized and ActiveVersionId the version actually serving; they diverge transiently during a

switch and persistently when the desired version does not validate. That divergence is observable on

purpose, because an operator needs to see that the update they requested is not the one being

served. A Server shall not silently activate a version that failed validation.

A shell is materialized whole or not at all. A shell's document, its submodel documents and the

concept descriptions its SemanticIds resolve to form a closure. A Server shall materialize a

closure into one shadow generation and commit it as a unit: a closure with an unresolved or invalid

member shall not be partially activated, and no node of it becomes visible. A dependency is

resolved against the registry itself first and, failing that, through a configured federation

provider (clause 6.5.6). A Server shall not dereference an arbitrary URL found inside a document

while materializing it — an ExternalReference is federation metadata, not permission to fetch.

Re-materializing an unchanged document changes nothing. A document whose digest is unchanged

since it was last materialized shall be reported Unchanged and shall not be

re-materialized, unless Force is set. Re-running an unchanged call shall produce the same nodes

with the same NodeIds and shall not increment MaterializationGeneration. A Server should

compute the digest over canonical document bytes, so that reformatting a document that means the

same thing does not churn the AddressSpace.

A committed switch is announced. A switch changes the AddressSpace graph, so the Server

shall emit GeneralModelChangeEventType for the committed additions, removals and reference

changes, and shall stamp the affected nodes' NodeVersion so a Client can correlate a node with

the MaterializationGeneration that produced it. Events are emitted for committed changes only,

never for shadow-generation work.

Deleting a document retires its nodes first. Deleting a registry resource shall retire its

materialized generation before removing the stored document, under the same closure rules: a Server

shall refuse the deletion while another materialized document depends on it, unless the caller

asks for the dependents to be retired too, in which case those dependents are retired first and

reported.

AutoMaterialize states whether a stored-document change re-materializes without being asked. The

Materialize Method re-materializes a selection on demand whether or not AutoMaterialize is set,

and returns one AASMaterializationResultDataType per document it considered — so a caller learns

which documents were skipped, which were rebuilt and which failed, rather than only whether the call

returned good.

<!-- model-figure: root=ns=2;i=1100 require=none external=RegistryType,ResourceType -->

Figure 13 – The updateable registry profile

Figure 13 — The updateable registry profile. SubmodelNode holds the NodeId of the

AASSubmodelType node the document materialized into; it is a Property value, not a Reference.

The states a document passes through are those of AASLoadStateDataType, and the transitions are

the rules above rather than an implementation's choice:

Figure 14 – Materialization lifecycle of one stored document

Figure 14 — Materialization lifecycle of one stored document

Implementer notes. This subclause is informative.

- NodeId stability. The NodeIds of clause 6.1.3 are the escaped, length-prefixed encodings of the AAS identifier and idShortPath, not values allocated by the Server. Two generations of the same document therefore contain the same NodeIds. A shadow generation must be held in a separate node table until the switch, not merged into the live one, or the preparation itself becomes visible.

- Retirement and NodeId reuse together. Under graceful retirement two generations exist at once with overlapping NodeIds. A Session that resolved a NodeId before the switch must continue to reach the generation it started with; resolving NodeId to node per Session-and-generation rather than globally is the straightforward way to get this right.

- Digest before parse. Compute the digest and compare it before parsing. A registry of any size spends nearly all of a periodic re-materialization discovering that nothing changed.

- The closure is not the document. A submodel document that validates on its own can still fail a closure because a concept description it references is missing. Report the failure against the document the operator wrote, naming the unresolved member — reporting it against the missing document tells the operator nothing they can act on.

canonical-document-and-derived-projection discipline for a registry of Thing Descriptions. The rules

above are stated here in full rather than incorporated by reference.

### Environment documents

This clause applies to a Server that implements both halves — that is, one claiming AAS-Registry

together with AAS-InstanceMaterialization. Such a Server shall claim AAS-EnvironmentExport

and satisfy this clause.

For each AASEnvironmentType folder it materializes, the registry shall hold at least one

AASEnvironmentFileType resource whose file content is a serialization of that environment. The

Format attribute states which serialization; a Server shall offer at least the AAS JSON

environment document (aas/3.0+json) and the AASX package (aasx/3.0), and may offer the AAS

XML environment document (aas/3.0+xml). EnvironmentNode identifies the folder the document

serializes. The document is retrieved with the File Transfer Methods of clause 6.5.1, like any other

registry resource.

The document covers the whole environment. A serialization shall contain every shell,

submodel, concept description and submodel element materialized under that folder, serialized per

clauses 6.1 and 6.4. A Server shall not offer a document that covers part of an environment under

this type; a partial export is a submodel document (clause 6.5.2) or a package (clause 6.5.2), not an

environment document.

The document is filtered to the caller's permissions. The content served to a Session shall

contain only what that Session is permitted to read. A node the Session could not Browse or Read

shall be absent from the document, together with everything beneath it. Filtering shall be

applied at the point of retrieval, not at the point the document was written, so a Session never

obtains content through a document that the AddressSpace would have withheld from it.

Filtering interacts with three earlier rules, and the interaction is resolved in favour of the

permission check:

- Absent versus empty (clause 6.1.5) is not re-derived from filtering. A field removed by filtering shall be omitted as though absent. A Consumer shall not infer from a filtered document that a field was absent in the environment.

- A filtered document is not lossless. Clause 6.4 applies to an unfiltered serialization. A Server shall set Filtered on the resource to indicate that the content served to this Session omits content, and shall not publish a Digest for a document whose bytes depend on the caller.

- Disclosure tiers (clause 6.5.7) apply to the document itself. DisclosureTier and Authorization on the resource describe the document; they do not substitute for the per-node permission check above.

A Server may materialize the document on demand rather than storing it, provided the result is

identical to a stored document filtered the same way.

<!-- model-figure: root=ns=2;i=1100 require=none external=RegistryType,ResourceType -->

Figure 15 – The materialized environment served as a retrievable document

Figure 15 — The materialized environment served as a retrievable document

# OPC UA ObjectTypes

## AASReferableType

Abstract base of everything in the metamodel that can be referred to by a short name. Carries the identifying and descriptive attributes every element has.

Table 6 – AASReferableType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASReferableType |  |  |  |  |
| IsAbstract | True |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:BaseObjectType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | IdShort | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | Category | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | DisplayNameSet | AASLangStringDataType[] | 0:PropertyType | O |
| 0:HasProperty | Variable | DescriptionSet | AASLangStringDataType[] | 0:PropertyType | O |
| 0:HasProperty | Variable | Extensions | AASExtensionDataType[] | 0:PropertyType | O |
| 0:HasProperty | Variable | ModelType | 0:String | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| AAS-Metamodel |  |  |  |  |  |

## AASIdentifiableType

Abstract base of the metamodel elements that carry a globally unique identifier: shells, submodels and concept descriptions.

Table 7 – AASIdentifiableType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASIdentifiableType |  |  |  |  |
| IsAbstract | True |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the AASReferableType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | Id | 0:String | 0:PropertyType | M |
| 0:HasProperty | Variable | Administration | AASAdministrativeInformationDataType | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| AAS-Metamodel |  |  |  |  |  |

## AASHasSemanticsType

Abstract base of the elements that declare what concept they are an occurrence of.

Table 8 – AASHasSemanticsType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASHasSemanticsType |  |  |  |  |
| IsAbstract | True |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:BaseObjectType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | SemanticId | AASReferenceDataType | 0:PropertyType | O |
| 0:HasProperty | Variable | SupplementalSemanticIds | AASReferenceDataType[] | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| AAS-Metamodel |  |  |  |  |  |

## AASHasKindType

Abstract base of the elements that distinguish a template from an instance.

Table 9 – AASHasKindType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASHasKindType |  |  |  |  |
| IsAbstract | True |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:BaseObjectType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | Kind | AASModellingKindDataType | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| AAS-Metamodel |  |  |  |  |  |

## AASHasDataSpecificationType

Abstract base of the elements that carry data specifications.

Table 10 – AASHasDataSpecificationType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASHasDataSpecificationType |  |  |  |  |
| IsAbstract | True |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:BaseObjectType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | EmbeddedDataSpecifications | AASEmbeddedDataSpecificationDataType[] | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| AAS-Metamodel |  |  |  |  |  |

## AASQualifiableType

Abstract base of the elements that can be qualified.

Table 11 – AASQualifiableType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASQualifiableType |  |  |  |  |
| IsAbstract | True |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:BaseObjectType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | Qualifiers | AASQualifierDataType[] | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| AAS-Metamodel |  |  |  |  |  |

## AASEnvironmentType

The container of shells, submodels and concept descriptions - the unit an AAS serialization carries and the root a source generator materializes into a Server.

Table 12 – AASEnvironmentType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASEnvironmentType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:FolderType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:Organizes | Object | <AssetAdministrationShell> |  | AASType | OP |
| 0:Organizes | Object | <Submodel> |  | AASSubmodelType | OP |
| 0:Organizes | Object | <ConceptDescription> |  | AASConceptDescriptionType | OP |
| Conformance Units |  |  |  |  |  |
| AAS-Metamodel |  |  |  |  |  |
| AAS-InstanceMaterialization |  |  |  |  |  |

## AASType

An Asset Administration Shell: the digital representation of one asset, carrying the asset's identity and references to the submodels that describe it.

Table 13 – AASType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the AASIdentifiableType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasComponent | Object | AssetInformation |  | AASAssetInformationType | M |
| 0:HasProperty | Variable | SubmodelReferences | AASReferenceDataType[] | 0:PropertyType | O |
| 0:HasProperty | Variable | DerivedFrom | AASReferenceDataType | 0:PropertyType | O |
| 0:HasProperty | Variable | EmbeddedDataSpecifications | AASEmbeddedDataSpecificationDataType[] | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| AAS-Metamodel |  |  |  |  |  |
| AAS-InstanceMaterialization |  |  |  |  |  |

## AASAssetInformationType

The identity of the asset a shell represents, as distinct from the identity of the shell itself.

Table 14 – AASAssetInformationType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASAssetInformationType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:BaseObjectType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | AssetKind | AASAssetKindDataType | 0:PropertyType | M |
| 0:HasProperty | Variable | GlobalAssetId | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | AssetType | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | SpecificAssetIds | AASSpecificAssetIdDataType[] | 0:PropertyType | O |
| 0:HasProperty | Variable | DefaultThumbnail | AASResourceDataType | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| AAS-Metamodel |  |  |  |  |  |

## AASSubmodelType

One coherent aspect of an asset, identified in its own right and typed by its SemanticId: a nameplate, technical data, a carbon footprint, a bill of material.

Table 15 – AASSubmodelType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASSubmodelType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the AASIdentifiableType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | Kind | AASModellingKindDataType | 0:PropertyType | O |
| 0:HasProperty | Variable | SemanticId | AASReferenceDataType | 0:PropertyType | O |
| 0:HasProperty | Variable | SupplementalSemanticIds | AASReferenceDataType[] | 0:PropertyType | O |
| 0:HasProperty | Variable | Qualifiers | AASQualifierDataType[] | 0:PropertyType | O |
| 0:HasProperty | Variable | EmbeddedDataSpecifications | AASEmbeddedDataSpecificationDataType[] | 0:PropertyType | O |
| 0:HasComponent | Object | <SubmodelElement> |  | AASSubmodelElementType | OP |
| Conformance Units |  |  |  |  |  |
| AAS-Metamodel |  |  |  |  |  |
| AAS-InstanceMaterialization |  |  |  |  |  |

## AASConceptDescriptionType

The definition a SemanticId resolves to - what makes two submodels from different vendors comparable.

Table 16 – AASConceptDescriptionType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASConceptDescriptionType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the AASIdentifiableType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | IsCaseOf | AASReferenceDataType[] | 0:PropertyType | O |
| 0:HasProperty | Variable | EmbeddedDataSpecifications | AASEmbeddedDataSpecificationDataType[] | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| AAS-Metamodel |  |  |  |  |  |

## AASSubmodelElementType

Abstract base of every element that can appear inside a submodel.

Table 17 – AASSubmodelElementType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASSubmodelElementType |  |  |  |  |
| IsAbstract | True |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the AASReferableType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | SemanticId | AASReferenceDataType | 0:PropertyType | O |
| 0:HasProperty | Variable | SupplementalSemanticIds | AASReferenceDataType[] | 0:PropertyType | O |
| 0:HasProperty | Variable | Qualifiers | AASQualifierDataType[] | 0:PropertyType | O |
| 0:HasProperty | Variable | EmbeddedDataSpecifications | AASEmbeddedDataSpecificationDataType[] | 0:PropertyType | O |
| 0:HasProperty | Variable | Index | 0:UInt32 | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| AAS-SubmodelElements |  |  |  |  |  |

## AASPropertyType

A single typed value. The value node carries the OPC UA DataType clause 6.3.1 assigns to the declared xsd type, from which the declared type is read.

Table 18 – AASPropertyType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASPropertyType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the AASSubmodelElementType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | ValueType | AASDataTypeDefXsdDataType | 0:PropertyType | M |
| 0:HasProperty | Variable | Value | 0:BaseDataType | 0:PropertyType | O |
| 0:HasProperty | Variable | ValueId | AASReferenceDataType | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| AAS-SubmodelElements |  |  |  |  |  |
| AAS-ValueFidelity |  |  |  |  |  |

## AASMultiLanguagePropertyType

A value expressed in one or more languages. The array order is preserved, because the metamodel's serialization is ordered and a round trip that reordered it would not reproduce its input.

Table 19 – AASMultiLanguagePropertyType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASMultiLanguagePropertyType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the AASSubmodelElementType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | Value | AASLangStringDataType[] | 0:PropertyType | O |
| 0:HasProperty | Variable | ValueId | AASReferenceDataType | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| AAS-SubmodelElements |  |  |  |  |  |
| AAS-ValueFidelity |  |  |  |  |  |

## AASRangeType

A closed or half-open interval of a single typed value.

Table 20 – AASRangeType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASRangeType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the AASSubmodelElementType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | ValueType | AASDataTypeDefXsdDataType | 0:PropertyType | M |
| 0:HasProperty | Variable | Min | 0:BaseDataType | 0:PropertyType | O |
| 0:HasProperty | Variable | Max | 0:BaseDataType | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| AAS-SubmodelElements |  |  |  |  |  |
| AAS-ValueFidelity |  |  |  |  |  |

## AASBlobType

Binary content carried inline.

Table 21 – AASBlobType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASBlobType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the AASSubmodelElementType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | Value | 0:ByteString | 0:PropertyType | O |
| 0:HasProperty | Variable | ContentType | 0:String | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| AAS-SubmodelElements |  |  |  |  |  |
| AAS-ValueFidelity |  |  |  |  |  |

## AASFileType

A pointer to content held outside the element.

Table 22 – AASFileType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASFileType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the AASSubmodelElementType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | Value | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | ContentType | 0:String | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| AAS-SubmodelElements |  |  |  |  |  |
| AAS-ValueFidelity |  |  |  |  |  |

## AASReferenceElementType

An element whose value is a reference.

Table 23 – AASReferenceElementType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASReferenceElementType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the AASSubmodelElementType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | Value | AASReferenceDataType | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| AAS-SubmodelElements |  |  |  |  |  |

## AASRelationshipElementType

A directed relationship between two referenced things.

Table 24 – AASRelationshipElementType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASRelationshipElementType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the AASSubmodelElementType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | First | AASReferenceDataType | 0:PropertyType | M |
| 0:HasProperty | Variable | Second | AASReferenceDataType | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| AAS-SubmodelElements |  |  |  |  |  |

## AASAnnotatedRelationshipElementType

A relationship carrying data elements that annotate it, such as a quantity or a position.

Table 25 – AASAnnotatedRelationshipElementType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASAnnotatedRelationshipElementType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the AASRelationshipElementType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasComponent | Object | <Annotation> |  | AASSubmodelElementType | OP |
| Conformance Units |  |  |  |  |  |
| AAS-SubmodelElements |  |  |  |  |  |

## AASSubmodelElementCollectionType

An unordered set of elements, each identified by its own IdShort.

Table 26 – AASSubmodelElementCollectionType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASSubmodelElementCollectionType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the AASSubmodelElementType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasComponent | Object | <SubmodelElement> |  | AASSubmodelElementType | OP |
| Conformance Units |  |  |  |  |  |
| AAS-SubmodelElements |  |  |  |  |  |
| AAS-LosslessRoundTrip |  |  |  |  |  |

## AASSubmodelElementListType

A list of elements. Its members have no IdShort, so they are named by index. Whether the order carries meaning is stated by the ReferenceType on each instance, not by a Property: HasOrderedComponent where it does, HasComponent where the list is a set or a bag. The declaration uses HasComponent, the base of both legal instance forms.

Table 27 – AASSubmodelElementListType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASSubmodelElementListType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the AASSubmodelElementType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | TypeValueListElement | AASSubmodelElementsDataType | 0:PropertyType | M |
| 0:HasProperty | Variable | SemanticIdListElement | AASReferenceDataType | 0:PropertyType | O |
| 0:HasProperty | Variable | ValueTypeListElement | AASDataTypeDefXsdDataType | 0:PropertyType | O |
| 0:HasComponent | Object | <Element> |  | AASSubmodelElementType | OP |
| Conformance Units |  |  |  |  |  |
| AAS-SubmodelElements |  |  |  |  |  |
| AAS-LosslessRoundTrip |  |  |  |  |  |

## AASEntityType

A component of a composition. A self-managed entity carries the identifier of its own shell, so a bill of material is traversable across organizations.

Table 28 – AASEntityType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASEntityType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the AASSubmodelElementType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | EntityType | AASEntityTypeDataType | 0:PropertyType | M |
| 0:HasProperty | Variable | GlobalAssetId | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | SpecificAssetIds | AASSpecificAssetIdDataType[] | 0:PropertyType | O |
| 0:HasComponent | Object | <Statement> |  | AASSubmodelElementType | OP |
| Conformance Units |  |  |  |  |  |
| AAS-SubmodelElements |  |  |  |  |  |

## AASBasicEventElementType

An event source or sink.

Table 29 – AASBasicEventElementType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASBasicEventElementType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the AASSubmodelElementType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | Observed | AASReferenceDataType | 0:PropertyType | M |
| 0:HasProperty | Variable | Direction | AASDirectionDataType | 0:PropertyType | M |
| 0:HasProperty | Variable | State | AASStateOfEventDataType | 0:PropertyType | M |
| 0:HasProperty | Variable | MessageTopic | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | MessageBroker | AASReferenceDataType | 0:PropertyType | O |
| 0:HasProperty | Variable | LastUpdate | 0:DateTime | 0:PropertyType | O |
| 0:HasProperty | Variable | MinInterval | 0:DurationString | 0:PropertyType | O |
| 0:HasProperty | Variable | MaxInterval | 0:DurationString | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| AAS-SubmodelElements |  |  |  |  |  |

## AASOperationType

An invocable operation.

Table 30 – AASOperationType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASOperationType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the AASSubmodelElementType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | InputVariables | AASOperationVariableDataType[] | 0:PropertyType | O |
| 0:HasProperty | Variable | OutputVariables | AASOperationVariableDataType[] | 0:PropertyType | O |
| 0:HasProperty | Variable | InoutputVariables | AASOperationVariableDataType[] | 0:PropertyType | O |
| 0:HasComponent | Object | <Variable> |  | AASSubmodelElementType | OP |
| 0:HasComponent | Method | Invoke |  |  | O |
| Conformance Units |  |  |  |  |  |
| AAS-SubmodelElements |  |  |  |  |  |

### Invoke

Invoke the operation and return its results. The Call counterpart of InvokeOperation in the AAS API of IDTA-01002 Part 2: a Client that has browsed to the Operation element calls this rather than reaching for the HTTP interface, and the two carry the same arguments in the same order.

Signature

Invoke (

[in]  0:BaseDataType[] InputValues,

[in]  0:BaseDataType[] InoutputValues,

[in]  0:Duration     ClientTimeout,

[out] 0:BaseDataType[] OutputValues,

[out] 0:BaseDataType[] InoutputResults,

[out] 0:Boolean      Success,

[out] 0:String       Diagnostic);

Table 31 – Invoke Method Arguments

| Argument | Description |
| --- | --- |
| InputValues | Values for the operation's input variables, positionally matching InputVariables. |
| InoutputValues | Values for the operation's in-out variables, positionally matching InoutputVariables. |
| ClientTimeout | How long the caller will wait. Zero means the Server's default. Corresponds to clientTimeoutDuration of the AAS API request. |
| OutputValues | Results, positionally matching OutputVariables. |
| InoutputResults | The in-out variables after execution, positionally matching InoutputVariables. |
| Success | Whether the operation executed successfully. A false result is an executed operation that failed, not a failed Call. |
| Diagnostic | Why the operation failed, where it did. |

Table 32 – Invoke Method AddressSpace definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | Invoke |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | ModellingRule |
| 0:HasProperty | Variable | 0:InputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |
| 0:HasProperty | Variable | 0:OutputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |
| Conformance Units |  |  |  |  |  |
| AAS-OperationInvoke |  |  |  |  |  |

## AASCapabilityType

A declared capability of the asset. It carries no value of its own; the element's identity and semantics are the whole of its content.

Table 33 – AASCapabilityType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASCapabilityType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the AASSubmodelElementType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| AAS-SubmodelElements |  |  |  |  |  |

## AASRegistryType

The AAS Registry root - an xRegistry RegistryType, and therefore a FolderType - whose group folders hold shells, submodel templates, concept dictionaries and packages. Exposed as a well-known object under the Server object, so any Client that reaches the standard Server object discovers it.

Table 34 – AASRegistryType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASRegistryType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 1:RegistryType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:Organizes | Object | <ShellGroup> |  | AASShellGroupType | OP |
| 0:Organizes | Object | <SubmodelTemplateGroup> |  | AASSubmodelTemplateGroupType | OP |
| 0:Organizes | Object | <ConceptDictionaryGroup> |  | AASConceptDictionaryGroupType | OP |
| 0:Organizes | Object | <PackageStoreGroup> |  | AASPackageStoreGroupType | OP |
| 0:Organizes | Object | <Environment> |  | AASEnvironmentFileType | OP |
| 0:HasComponent | Method | LookupShellsByAssetLink |  |  | O |
| 0:HasComponent | Method | GetSubmodel |  |  | O |
| 0:HasProperty | Variable | AutoMaterialize | 0:Boolean | 0:PropertyType | O |
| 0:HasProperty | Variable | MaterializationGeneration | 0:UInt32 | 0:PropertyType | O |
| 0:HasComponent | Method | Materialize |  |  | O |
| Conformance Units |  |  |  |  |  |
| AAS-Registry |  |  |  |  |  |
| AAS-Discovery |  |  |  |  |  |
| AAS-UpdateableRegistry |  |  |  |  |  |
| AAS-EnvironmentExport |  |  |  |  |  |

### LookupShellsByAssetLink

Return the shells discoverable by an asset key. This is the discovery question - given a serial number or a part identifier, which shells describe it - answered without the caller browsing the whole collection.

Signature

LookupShellsByAssetLink (

[in]  0:String       Name,

[in]  0:String       Value,

[out] 0:NodeId[]     Shells);

Table 35 – LookupShellsByAssetLink Method Arguments

| Argument | Description |
| --- | --- |
| Name | The key name, for example serialNumber. |
| Value | The key value. |
| Shells | The shell group nodes matching the key. |

Table 36 – LookupShellsByAssetLink Method AddressSpace definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | LookupShellsByAssetLink |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | ModellingRule |
| 0:HasProperty | Variable | 0:InputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |
| 0:HasProperty | Variable | 0:OutputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |

### GetSubmodel

Resolve the selected AASSubmodelFileType before returning its document and enforce the same Session-specific effective RolePermissions, UserRolePermissions, DisclosureTier, Authorization and FileType Open/Read decision as direct access to that target. Call permission on this Method does not authorize the target. Return Bad_UserAccessDenied, or Bad_NotFound where policy conceals existence, without exposing controlled bytes, Format, ContentType, other target metadata or a distinguishable timing path.

Signature

GetSubmodel (

[in]  0:String       SubmodelIdentifier,

[out] 0:ByteString   Document,

[out] 0:String       Format,

[out] 0:String       ContentType);

Table 37 – GetSubmodel Method Arguments

| Argument | Description |
| --- | --- |
| SubmodelIdentifier | The submodel's authored identifier. |
| Document | The submodel document bytes. |
| Format | xRegistry format string. |
| ContentType | Document media type. |

Table 38 – GetSubmodel Method AddressSpace definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | GetSubmodel |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | ModellingRule |
| 0:HasProperty | Variable | 0:InputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |
| 0:HasProperty | Variable | 0:OutputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |

### Materialize

Re-materialize the AddressSpace from the stored documents. Part of the updateable registry profile: the documents are canonical and the nodes are derived, so this is the operation that makes the derived side agree with the canonical one.

Signature

Materialize (

[in]  0:String[]     Targets,

[in]  0:Boolean      Force,

[out] 0:UInt32       Generation,

[out] AASMaterializationResultDataType[] Results);

Table 39 – Materialize Method Arguments

| Argument | Description |
| --- | --- |
| Targets | The documents to consider, as registry-relative paths. An empty array means every document. |
| Force | Re-materialize even a document whose digest is unchanged. |
| Generation | The generation in force after the call. |
| Results | One result per document considered. |

Table 40 – Materialize Method AddressSpace definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | Materialize |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | ModellingRule |
| 0:HasProperty | Variable | 0:InputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |
| 0:HasProperty | Variable | 0:OutputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |

## AASShellGroupType

An xRegistry GroupType holding the submodel documents of one shell. Its source identity is the shell's authored identifier, from which the GroupId is constructed. It is distinct from AASType, which models the same shell as a live node tree rather than as a catalogue entry.

Table 41 – AASShellGroupType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASShellGroupType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 1:GroupType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | AasIdentifier | 0:String | 0:PropertyType | M |
| 0:HasProperty | Variable | AssetKind | AASAssetKindDataType | 0:PropertyType | M |
| 0:HasProperty | Variable | GlobalAssetId | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | AssetType | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | SpecificAssetIds | AASSpecificAssetIdDataType[] | 0:PropertyType | O |
| 0:HasProperty | Variable | Administration | AASAdministrativeInformationDataType | 0:PropertyType | O |
| 0:HasProperty | Variable | DerivedFrom | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | DisclosureTier | AASDisclosureTierDataType | 0:PropertyType | O |
| 0:HasProperty | Variable | Authorization | AASAuthorizationOptionDataType[] | 0:PropertyType | O |
| 0:HasProperty | Variable | EventEndpoint | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | ShellNode | 0:NodeId | 0:PropertyType | O |
| 0:Organizes | Object | <Submodel> |  | AASSubmodelFileType | OP |
| Conformance Units |  |  |  |  |  |
| AAS-Registry |  |  |  |  |  |
| AAS-RegistryIdentity |  |  |  |  |  |
| AAS-Discovery |  |  |  |  |  |
| AAS-DisclosureTiers |  |  |  |  |  |

## AASSubmodelFileType

An xRegistry ResourceType whose file content is one submodel document. Each version is one revision, which is what gives a shell the lifecycle history the metamodel does not itself provide.

Table 42 – AASSubmodelFileType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASSubmodelFileType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 1:ResourceType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | SubmodelIdentifier | 0:String | 0:PropertyType | M |
| 0:HasProperty | Variable | SemanticId | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | SupplementalSemanticIds | 0:String[] | 0:PropertyType | O |
| 0:HasProperty | Variable | Kind | AASModellingKindDataType | 0:PropertyType | O |
| 0:HasProperty | Variable | Template | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | Digest | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | DigestAlg | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | IsDefault | 0:Boolean | 0:PropertyType | O |
| 0:HasProperty | Variable | Ancestor | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | DisclosureTier | AASDisclosureTierDataType | 0:PropertyType | O |
| 0:HasProperty | Variable | Authorization | AASAuthorizationOptionDataType[] | 0:PropertyType | O |
| 0:HasProperty | Variable | SubmodelNode | 0:NodeId | 0:PropertyType | O |
| 0:HasProperty | Variable | LoadState | AASLoadStateDataType | 0:PropertyType | O |
| 0:HasProperty | Variable | DesiredVersionId | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | ActiveVersionId | 0:String | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| AAS-Registry |  |  |  |  |  |
| AAS-RegistryIdentity |  |  |  |  |  |
| AAS-RegistryVersioning |  |  |  |  |  |
| AAS-DisclosureTiers |  |  |  |  |  |
| AAS-UpdateableRegistry |  |  |  |  |  |

## AASSubmodelTemplateGroupType

An xRegistry GroupType holding one publisher's family of submodel templates. Templates are held in a group of their own so that a Consumer lists templates and instances separately.

Table 43 – AASSubmodelTemplateGroupType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASSubmodelTemplateGroupType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 1:GroupType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | TemplateNamespace | 0:String | 0:PropertyType | M |
| 0:HasProperty | Variable | Publisher | 0:String | 0:PropertyType | O |
| 0:Organizes | Object | <Submodel> |  | AASSubmodelFileType | OP |
| Conformance Units |  |  |  |  |  |
| AAS-Registry |  |  |  |  |  |
| AAS-RegistryIdentity |  |  |  |  |  |

## AASConceptDictionaryGroupType

An xRegistry GroupType holding one dictionary of concept definitions - the definitions a SemanticId elsewhere in the registry resolves to.

Table 44 – AASConceptDictionaryGroupType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASConceptDictionaryGroupType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 1:GroupType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | DictionaryIdentifier | 0:String | 0:PropertyType | M |
| 0:Organizes | Object | <ConceptDescription> |  | AASConceptDescriptionFileType | OP |
| Conformance Units |  |  |  |  |  |
| AAS-Registry |  |  |  |  |  |
| AAS-RegistryIdentity |  |  |  |  |  |

## AASConceptDescriptionFileType

An xRegistry ResourceType whose file content is one concept description document.

Table 45 – AASConceptDescriptionFileType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASConceptDescriptionFileType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 1:ResourceType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | ConceptIdentifier | 0:String | 0:PropertyType | M |
| 0:HasProperty | Variable | IsCaseOf | 0:String[] | 0:PropertyType | O |
| 0:HasProperty | Variable | ConceptNode | 0:NodeId | 0:PropertyType | O |
| 0:HasProperty | Variable | LoadState | AASLoadStateDataType | 0:PropertyType | O |
| 0:HasProperty | Variable | DesiredVersionId | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | ActiveVersionId | 0:String | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| AAS-Registry |  |  |  |  |  |
| AAS-RegistryIdentity |  |  |  |  |  |
| AAS-UpdateableRegistry |  |  |  |  |  |

## AASPackageStoreGroupType

An xRegistry GroupType holding packages - one store, or one namespace within one.

Table 46 – AASPackageStoreGroupType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASPackageStoreGroupType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 1:GroupType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | StoreIdentifier | 0:String | 0:PropertyType | M |
| 0:HasProperty | Variable | RegistryUrl | 0:String | 0:PropertyType | O |
| 0:Organizes | Object | <Package> |  | AASPackageFileType | OP |
| Conformance Units |  |  |  |  |  |
| AAS-Packages |  |  |  |  |  |
| AAS-RegistryIdentity |  |  |  |  |  |

## AASPackageFileType

An xRegistry ResourceType whose file content is one package. Every package carries mandatory strong integrity metadata for the exact returned blob; an OCI-backed version also carries the immutable manifest digest that is its version identity. Mutable tags are Resource-level discovery aliases, never Version identity, and OCI referrers are separate Resources rather than package Versions and cannot affect the package default Version.

Table 47 – AASPackageFileType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASPackageFileType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 1:ResourceType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | PackageIdentifier | 0:String | 0:PropertyType | M |
| 0:HasProperty | Variable | ArtifactType | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | Digest | 0:String | 0:PropertyType | M |
| 0:HasProperty | Variable | DigestAlg | 0:String | 0:PropertyType | M |
| 0:HasProperty | Variable | AasIdentifiers | 0:String[] | 0:PropertyType | O |
| 0:HasProperty | Variable | ManifestDigest | 0:String | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| AAS-Packages |  |  |  |  |  |
| AAS-RegistryIdentity |  |  |  |  |  |
| AAS-PackageIntegrity |  |  |  |  |  |

## AASEnvironmentFileType

An xRegistry ResourceType whose file content is one serialization of a materialized environment: an AAS JSON or XML environment document, or an AASX package. It is the retrievable form of an AASEnvironmentType folder, and its content is filtered to what the calling Session is permitted to read.

Table 48 – AASEnvironmentFileType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASEnvironmentFileType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 1:ResourceType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | EnvironmentIdentifier | 0:String | 0:PropertyType | M |
| 0:HasProperty | Variable | Format | 0:String | 0:PropertyType | M |
| 0:HasProperty | Variable | EnvironmentNode | 0:NodeId | 0:PropertyType | M |
| 0:HasProperty | Variable | Digest | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | DigestAlg | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | Filtered | 0:Boolean | 0:PropertyType | M |
| 0:HasProperty | Variable | DisclosureTier | AASDisclosureTierDataType | 0:PropertyType | O |
| 0:HasProperty | Variable | Authorization | AASAuthorizationOptionDataType[] | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| AAS-EnvironmentExport |  |  |  |  |  |
| AAS-RegistryIdentity |  |  |  |  |  |

# OPC UA DataTypes

The DataTypes defined by this document are enumerations. Each is formally defined in the NodeSet and listed in Annex A.

## AASAnyUri

An xs:anyURI value. A subtype of String, since String carries xs:string.

Table 49 – AASAnyUri definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASAnyUri |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:String defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| AAS-ValueFidelity |  |  |  |  |  |

## AASHexBinary

An xs:hexBinary value. ByteString carries xs:base64Binary, whose octets are the same, so the hexadecimal form is carried by this subtype.

Table 50 – AASHexBinary definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASHexBinary |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:ByteString defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| AAS-ValueFidelity |  |  |  |  |  |

## AASNonPositiveInteger

An xs:nonPositiveInteger value: an integer at most zero.

Table 51 – AASNonPositiveInteger definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASNonPositiveInteger |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Integer defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| AAS-ValueFidelity |  |  |  |  |  |

## AASNegativeInteger

An xs:negativeInteger value: an integer below zero. A subtype of AASNonPositiveInteger, following the xsd restriction hierarchy.

Table 52 – AASNegativeInteger definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASNegativeInteger |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the AASNonPositiveInteger defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| AAS-ValueFidelity |  |  |  |  |  |

## AASPositiveInteger

An xs:positiveInteger value: an integer above zero. A subtype of UInteger, which carries xs:nonNegativeInteger.

Table 53 – AASPositiveInteger definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASPositiveInteger |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:UInteger defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| AAS-ValueFidelity |  |  |  |  |  |

## AASGYear

An xs:gYear value, such as 2026. A Gregorian year denotes a period, for which OPC UA has no DataType, so the value is its lexical form.

Table 54 – AASGYear definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASGYear |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:String defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| AAS-ValueFidelity |  |  |  |  |  |

## AASGYearMonth

An xs:gYearMonth value, such as 2026-08.

Table 55 – AASGYearMonth definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASGYearMonth |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:String defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| AAS-ValueFidelity |  |  |  |  |  |

## AASGMonth

An xs:gMonth value, such as --08.

Table 56 – AASGMonth definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASGMonth |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:String defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| AAS-ValueFidelity |  |  |  |  |  |

## AASGMonthDay

An xs:gMonthDay value, such as --08-07.

Table 57 – AASGMonthDay definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASGMonthDay |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:String defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| AAS-ValueFidelity |  |  |  |  |  |

## AASGDay

An xs:gDay value, such as ---07.

Table 58 – AASGDay definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASGDay |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:String defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| AAS-ValueFidelity |  |  |  |  |  |

## AASValueString

The xsd lexical form of a value whose declared type is carried in a sibling field of the same Structure. A Structure field has one static DataType and cannot vary with a declared type, so a qualifier, an extension or a data specification carries its value lexically and its sibling ValueType or DataType field states how to read it. A subtype of String, as OPC UA defines DecimalString and DurationString. It is never the DataType of a Variable; a value node carries the DataType clause 6.3.1 assigns to its declared xsd type.

Table 59 – AASValueString definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASValueString |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:String defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| AAS-ValueFidelity |  |  |  |  |  |

## AASAssetKindDataType

Whether a shell describes a product model, an individual item, a batch, a role, or none of these. The three granularity levels a product passport is issued at map onto Type, Instance and Batch.

Table 60 – AASAssetKindDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Type | 0 | The shell describes a product model rather than an individual item. |
| Instance | 1 | The shell describes one individual physical item. |
| Batch | 2 | The shell describes a production lot. |
| Role | 3 | The shell describes a role rather than a physical asset. |
| NotApplicable | 4 | Asset kind does not apply. |

Table 61 – AASAssetKindDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASAssetKindDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASAssetKindDataType |  |  |  |  |  |

## AASModellingKindDataType

Whether an element defines a shape or carries values.

Table 62 – AASModellingKindDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Template | 0 | Defines the shape other elements are built from; carries no values for an individual asset. |
| Instance | 1 | Carries values for one asset. |

Table 63 – AASModellingKindDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASModellingKindDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASModellingKindDataType |  |  |  |  |  |

## AASEntityTypeDataType

Whether a composition entity is managed within its parent or has a shell of its own.

Table 64 – AASEntityTypeDataType Items

| Name | Value | Description |
| --- | --- | --- |
| CoManagedEntity | 0 | The entity has no shell of its own and is managed within its parent. |
| SelfManagedEntity | 1 | The entity has its own shell, identified by GlobalAssetId, so a bill of material is traversable across organizations. |

Table 65 – AASEntityTypeDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASEntityTypeDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASEntityTypeDataType |  |  |  |  |  |

## AASDirectionDataType

The direction of an event element.

Table 66 – AASDirectionDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Input | 0 | The event is consumed by the element. |
| Output | 1 | The event is produced by the element. |

Table 67 – AASDirectionDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASDirectionDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASDirectionDataType |  |  |  |  |  |

## AASStateOfEventDataType

Whether an event element is currently active.

Table 68 – AASStateOfEventDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Off | 0 | The event source is inactive. |
| On | 1 | The event source is active. |

Table 69 – AASStateOfEventDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASStateOfEventDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASStateOfEventDataType |  |  |  |  |  |

## AASQualifierKindDataType

What a qualifier qualifies, and therefore whether it may change.

Table 70 – AASQualifierKindDataType Items

| Name | Value | Description |
| --- | --- | --- |
| ValueQualifier | 0 | Qualifies the value and may change during the element's lifetime. |
| ConceptQualifier | 1 | Qualifies the concept and is invariant. |
| TemplateQualifier | 2 | Qualifies the template the element was built from. |

Table 71 – AASQualifierKindDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASQualifierKindDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASQualifierKindDataType |  |  |  |  |  |

## AASReferenceTypesDataType

Whether a reference addresses something inside the model or outside it.

Table 72 – AASReferenceTypesDataType Items

| Name | Value | Description |
| --- | --- | --- |
| ExternalReference | 0 | Points at something outside the metamodel. |
| ModelReference | 1 | Points at a node within the model, navigated key by key. |

Table 73 – AASReferenceTypesDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASReferenceTypesDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASReferenceTypesDataType |  |  |  |  |  |

## AASKeyTypesDataType

The kind of thing a reference key addresses. The enumeration is closed: a value outside it cannot round-trip, so an implementation rejects it rather than dropping it.

Table 74 – AASKeyTypesDataType Items

| Name | Value | Description |
| --- | --- | --- |
| AnnotatedRelationshipElement | 0 |  |
| AssetAdministrationShell | 1 |  |
| BasicEventElement | 2 |  |
| Blob | 3 |  |
| Capability | 4 |  |
| ConceptDescription | 5 |  |
| DataElement | 6 |  |
| Entity | 7 |  |
| EventElement | 8 |  |
| File | 9 |  |
| FragmentReference | 10 |  |
| GlobalReference | 11 |  |
| Identifiable | 12 |  |
| MultiLanguageProperty | 13 |  |
| Operation | 14 |  |
| Property | 15 |  |
| Range | 16 |  |
| Referable | 17 |  |
| ReferenceElement | 18 |  |
| RelationshipElement | 19 |  |
| Submodel | 20 |  |
| SubmodelElement | 21 |  |
| SubmodelElementCollection | 22 |  |
| SubmodelElementList | 23 |  |

Table 75 – AASKeyTypesDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASKeyTypesDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASKeyTypesDataType |  |  |  |  |  |

## AASDataTypeDefXsdDataType

The xsd type a value is expressed in. All thirty of the metamodel's values are listed. Clause 6.3.1 assigns each one OPC UA DataType, and no DataType to two of them.

Table 76 – AASDataTypeDefXsdDataType Items

| Name | Value | Description |
| --- | --- | --- |
| AnyUri | 0 |  |
| Base64Binary | 1 |  |
| Boolean | 2 |  |
| Byte | 3 |  |
| Date | 4 |  |
| DateTime | 5 |  |
| Decimal | 6 |  |
| Double | 7 |  |
| Duration | 8 |  |
| Float | 9 |  |
| GDay | 10 |  |
| GMonth | 11 |  |
| GMonthDay | 12 |  |
| GYear | 13 |  |
| GYearMonth | 14 |  |
| HexBinary | 15 |  |
| Int | 16 |  |
| Integer | 17 |  |
| Long | 18 |  |
| NegativeInteger | 19 |  |
| NonNegativeInteger | 20 |  |
| NonPositiveInteger | 21 |  |
| PositiveInteger | 22 |  |
| Short | 23 |  |
| String | 24 |  |
| Time | 25 |  |
| UnsignedByte | 26 |  |
| UnsignedInt | 27 |  |
| UnsignedLong | 28 |  |
| UnsignedShort | 29 |  |

Table 77 – AASDataTypeDefXsdDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASDataTypeDefXsdDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASDataTypeDefXsdDataType |  |  |  |  |  |

## AASDataTypeIec61360DataType

The data type of a concept definition expressed in the IEC 61360 data specification.

Table 78 – AASDataTypeIec61360DataType Items

| Name | Value | Description |
| --- | --- | --- |
| Blob | 0 |  |
| Boolean | 1 |  |
| Date | 2 |  |
| File | 3 |  |
| Html | 4 |  |
| IntegerCount | 5 |  |
| IntegerCurrency | 6 |  |
| IntegerMeasure | 7 |  |
| Irdi | 8 |  |
| Iri | 9 |  |
| Rational | 10 |  |
| RationalMeasure | 11 |  |
| RealCount | 12 |  |
| RealCurrency | 13 |  |
| RealMeasure | 14 |  |
| String | 15 |  |
| StringTranslatable | 16 |  |
| Time | 17 |  |
| Timestamp | 18 |  |

Table 79 – AASDataTypeIec61360DataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASDataTypeIec61360DataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASDataTypeIec61360DataType |  |  |  |  |  |

## AASSubmodelElementsDataType

The element kind a SubmodelElementList constrains its members to.

Table 80 – AASSubmodelElementsDataType Items

| Name | Value | Description |
| --- | --- | --- |
| AnnotatedRelationshipElement | 0 |  |
| BasicEventElement | 1 |  |
| Blob | 2 |  |
| Capability | 3 |  |
| DataElement | 4 |  |
| Entity | 5 |  |
| EventElement | 6 |  |
| File | 7 |  |
| MultiLanguageProperty | 8 |  |
| Operation | 9 |  |
| Property | 10 |  |
| Range | 11 |  |
| ReferenceElement | 12 |  |
| RelationshipElement | 13 |  |
| SubmodelElement | 14 |  |
| SubmodelElementCollection | 15 |  |
| SubmodelElementList | 16 |  |

Table 81 – AASSubmodelElementsDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASSubmodelElementsDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASSubmodelElementsDataType |  |  |  |  |  |

## AASDisclosureTierDataType

Whether an entity is readable without authentication. It advertises the tier so a Consumer can discover it; it does not enforce it.

Table 82 – AASDisclosureTierDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Public | 0 | Readable without authentication. |
| Controlled | 1 | Requires an authenticated role. |

Table 83 – AASDisclosureTierDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASDisclosureTierDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASDisclosureTierDataType |  |  |  |  |  |

## AASLoadStateDataType

The materialization state of one stored document under the updateable registry profile.

Table 84 – AASLoadStateDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Unloaded | 0 | The document is stored but not materialized. |
| Loading | 1 | A shadow generation is being prepared and is not yet visible. |
| Active | 2 | The materialized nodes are the ones a Client sees. |
| Superseded | 3 | A newer generation has been switched in; this one still serves retained work. |
| Retiring | 4 | The superseded generation is draining and its nodes will be removed. |
| Retired | 5 | The generation's nodes have been removed. |
| Failed | 6 | The document did not validate or did not materialize. The stored document is kept and the previously active generation, where there was one, keeps serving. |

Table 85 – AASLoadStateDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASLoadStateDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| AAS-UpdateableRegistry |  |  |  |  |  |

## AASMaterializationOutcomeDataType

What a Materialize call did to one document.

Table 86 – AASMaterializationOutcomeDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Unchanged | 0 | The document's digest was unchanged, so it was not re-materialized. |
| Materialized | 1 | A new generation was prepared and switched in. |
| Retired | 2 | The document's projection was removed. |
| Failed | 3 | The document did not validate or did not materialize. Diagnostic says why. |

Table 87 – AASMaterializationOutcomeDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASMaterializationOutcomeDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| AAS-UpdateableRegistry |  |  |  |  |  |

## AASKeyDataType

One step of a reference path. Keys are ordered, and the order is part of the reference's meaning.

Table 88 – AASKeyDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Type |  | The kind of thing this key addresses. |
| Value |  | The identifier value at this key. |

Table 89 – AASKeyDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASKeyDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| 0:HasEncoding | Object | Default JSON |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASKeyDataType |  |  |  |  |  |

## AASReferenceDataType

A reference, external or model-navigating, expressed as an ordered key path.

Table 90 – AASReferenceDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Type |  | Whether the reference is external or navigates the model. |
| ReferredSemanticId |  | The semantic identifier of the thing referred to, where known. |
| Keys |  | The ordered key path. At least one key is present. |

Table 91 – AASReferenceDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASReferenceDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| 0:HasEncoding | Object | Default JSON |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASReferenceDataType |  |  |  |  |  |

## AASLangStringDataType

One language-tagged string. A multi-language value is an array of these, and the array order is preserved.

Table 92 – AASLangStringDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Language |  | BCP 47 language tag. |
| Text |  | The text in that language. |

Table 93 – AASLangStringDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASLangStringDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| 0:HasEncoding | Object | Default JSON |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASLangStringDataType |  |  |  |  |  |

## AASSpecificAssetIdDataType

A domain-specific key an asset is discoverable by.

Table 94 – AASSpecificAssetIdDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Name |  | The key name, for example serialNumber or manufacturerPartId. |
| Value |  | The key value. |
| ExternalSubjectId |  | The subject this key is disclosed to, where the key is not public. |
| SemanticId |  | The concept this key is an occurrence of. |
| SupplementalSemanticIds |  | Further concepts this key corresponds to. |

Table 95 – AASSpecificAssetIdDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASSpecificAssetIdDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| 0:HasEncoding | Object | Default JSON |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASSpecificAssetIdDataType |  |  |  |  |  |

## AASAdministrativeInformationDataType

Administrative information. It records a single current revision: the entity's history is carried by the registry, which the metamodel has no equivalent of.

Table 96 – AASAdministrativeInformationDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Version |  | Version label. |
| Revision |  | Revision label; only meaningful when Version is present. |
| Creator |  | The party that created the entity. |
| TemplateId |  | The template the entity was built from. |
| EmbeddedDataSpecifications |  | Data specifications carried by this administrative information. |

Table 97 – AASAdministrativeInformationDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASAdministrativeInformationDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| 0:HasEncoding | Object | Default JSON |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASAdministrativeInformationDataType |  |  |  |  |  |

## AASQualifierDataType

A qualifier constraining or annotating an element.

Table 98 – AASQualifierDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Kind |  | What the qualifier qualifies. |
| Type |  | The qualifier type name. |
| ValueType |  | The xsd type the value is expressed in. |
| Value |  | The value in the xsd lexical form of the type declared in the sibling ValueType field, because a Structure field has one static DataType and cannot vary with a declared type. |
| ValueId |  | A reference to the value, where it is itself an identified concept. |
| SemanticId |  | The concept this qualifier is an occurrence of. |
| SupplementalSemanticIds |  | Further concepts this qualifier corresponds to. |

Table 99 – AASQualifierDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASQualifierDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| 0:HasEncoding | Object | Default JSON |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASQualifierDataType |  |  |  |  |  |

## AASEmbeddedDataSpecificationDataType

A data specification carried by an element, paired with its content.

Table 100 – AASEmbeddedDataSpecificationDataType Items

| Name | Value | Description |
| --- | --- | --- |
| DataSpecification |  | Reference to the data specification template. |
| DataSpecificationContent |  | The content, in the IEC 61360 data specification. |

Table 101 – AASEmbeddedDataSpecificationDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASEmbeddedDataSpecificationDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| 0:HasEncoding | Object | Default JSON |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASEmbeddedDataSpecificationDataType |  |  |  |  |  |

## AASDataSpecificationIec61360DataType

The IEC 61360 data specification content of a concept definition.

Table 102 – AASDataSpecificationIec61360DataType Items

| Name | Value | Description |
| --- | --- | --- |
| PreferredName |  | Preferred name per language. |
| ShortName |  | Short name per language. |
| Unit |  | Unit symbol. |
| UnitId |  | Reference to the unit concept. |
| SourceOfDefinition |  | Where the definition comes from. |
| Symbol |  | Symbol for the concept. |
| DataType |  | The IEC 61360 data type. |
| Definition |  | Definition per language. |
| ValueFormat |  | Format of the value. |
| ValueList |  | Permitted values and the references identifying their meanings. |
| Value |  | The value in the xsd lexical form of the type declared in the sibling DataType field, because a Structure field has one static DataType and cannot vary with a declared type. |
| LevelType |  | Which of min, nom, typ and max apply. |

Table 103 – AASDataSpecificationIec61360DataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASDataSpecificationIec61360DataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| 0:HasEncoding | Object | Default JSON |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASDataSpecificationIec61360DataType |  |  |  |  |  |

## AASExtensionDataType

A proprietary extension carried on a Referable. Extensions round-trip verbatim; a reader that does not understand one preserves it unchanged.

Table 104 – AASExtensionDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Name |  | Extension name. |
| ValueType |  | The xsd type the value is expressed in. |
| Value |  | The value in the xsd lexical form of the type declared in the sibling ValueType field, because a Structure field has one static DataType and cannot vary with a declared type. |
| RefersTo |  | What the extension refers to. |
| SemanticId |  | The concept this extension is an occurrence of. |
| SupplementalSemanticIds |  | Further concepts this extension corresponds to. |

Table 105 – AASExtensionDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASExtensionDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| 0:HasEncoding | Object | Default JSON |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASExtensionDataType |  |  |  |  |  |

## AASResourceDataType

A pointer to external content, such as a thumbnail.

Table 106 – AASResourceDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Path |  | Path or URL to the resource. |
| ContentType |  | Media type of the resource. |

Table 107 – AASResourceDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASResourceDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| 0:HasEncoding | Object | Default JSON |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASResourceDataType |  |  |  |  |  |

## AASOperationVariableDataType

One input, output or in-out variable of an operation, carried as a reference to the element node that holds it so that the element's own representation is not duplicated.

Table 108 – AASOperationVariableDataType Items

| Name | Value | Description |
| --- | --- | --- |
| ValueNodeId |  | The direct HasComponent child of the Operation that carries this variable's value. |

Table 109 – AASOperationVariableDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASOperationVariableDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| 0:HasEncoding | Object | Default JSON |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASOperationVariableDataType |  |  |  |  |  |

## AASAuthorizationOptionDataType

One authorization option a Consumer may use. It is authorization configuration only and never carries credentials, which are supplied out of band.

Table 110 – AASAuthorizationOptionDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Type |  | Authorization type, for example OAuth2, Plain, SASL, X509Cert or APIKey. |
| Mechanism |  | SASL mechanism name, used only when Type is SASL. |
| ResourceUri |  | The resource authorization is requested for. |
| AuthorityUri |  | The authority authorization is obtained from. |

Table 111 – AASAuthorizationOptionDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASAuthorizationOptionDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| 0:HasEncoding | Object | Default JSON |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASAuthorizationOptionDataType |  |  |  |  |  |

## AASAttestationDataType

A non-authoritative discovery hint for a separate attestation or OCI referrer Resource. It never represents a package Version, and its presence is not verification: a Consumer retrieves and verifies the separate artifact itself.

Table 112 – AASAttestationDataType Items

| Name | Value | Description |
| --- | --- | --- |
| ArtifactType |  | Media type identifying what kind of attestation this is. |
| Digest |  | Digest of the attestation artifact. |
| Signer |  | The party that produced the attestation. |

Table 113 – AASAttestationDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASAttestationDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| 0:HasEncoding | Object | Default JSON |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASAttestationDataType |  |  |  |  |  |

## AASMaterializationResultDataType

The result of materializing one document. A call returns one of these per document it considered, reporting per document whether it was unchanged, materialized, retired or failed.

Table 114 – AASMaterializationResultDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Xid |  | The registry-relative path of the document this result is about. |
| Outcome |  | What the call did to it. |
| VersionId |  | The version that is now active for this document, where one is. |
| MaterializedNode |  | The root node of the generation now serving this document, where it materialized. |
| Diagnostic |  | Why the document failed, where it did. Empty otherwise. |

Table 115 – AASMaterializationResultDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASMaterializationResultDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| 0:HasEncoding | Object | Default JSON |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| AAS-UpdateableRegistry |  |  |  |  |  |

## AASValueReferencePairDataType

One permitted value paired with the reference identifying its meaning.

Table 116 – AASValueReferencePairDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Value |  | One permitted IEC 61360 value. |
| ValueId |  | The reference identifying the meaning of Value. |

Table 117 – AASValueReferencePairDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASValueReferencePairDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| 0:HasEncoding | Object | Default JSON |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASValueReferencePairDataType |  |  |  |  |  |

## AASValueListDataType

The non-empty list of permitted values for an IEC 61360 data specification.

Table 118 – AASValueListDataType Items

| Name | Value | Description |
| --- | --- | --- |
| ValueReferencePairs |  | The permitted values. At least one pair is present. |

Table 119 – AASValueListDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASValueListDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| 0:HasEncoding | Object | Default JSON |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASValueListDataType |  |  |  |  |  |

## AASLevelTypeDataType

The four IEC 61360 level flags. Every flag is explicit.

Table 120 – AASLevelTypeDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Min |  | Whether a minimum value applies. |
| Nom |  | Whether a nominal value applies. |
| Typ |  | Whether a typical value applies. |
| Max |  | Whether a maximum value applies. |

Table 121 – AASLevelTypeDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASLevelTypeDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| 0:HasEncoding | Object | Default JSON |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| OpenUSD AASLevelTypeDataType |  |  |  |  |  |

# Instances

## AASRegistry

Server-wide AAS Registry, a well-known component of the Server object.

Table 122 – AASRegistry definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 2:AASRegistry |  |  |  |  |
| TypeDefinition | AASRegistryType |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| 0:HasComponent | Method | LookupShellsByAssetLink |  |  |  |
| 0:HasComponent | Method | GetSubmodel |  |  |  |
| 0:HasComponent | Method | Materialize |  |  |  |
| Conformance Units |  |  |  |  |  |
| AAS-Registry |  |  |  |  |  |

### LookupShellsByAssetLink

Return the shells discoverable by an asset key. The functional method on the well-known AASRegistry object.

Signature

LookupShellsByAssetLink (

[in]  0:String       Name,

[in]  0:String       Value,

[out] 0:NodeId[]     Shells);

Table 123 – LookupShellsByAssetLink Method Arguments

| Argument | Description |
| --- | --- |
| Name | The key name, for example serialNumber. |
| Value | The key value. |
| Shells | The shell group nodes matching the key. |

Table 124 – LookupShellsByAssetLink Method AddressSpace definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | LookupShellsByAssetLink |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | ModellingRule |
| 0:HasProperty | Variable | 0:InputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |
| 0:HasProperty | Variable | 0:OutputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |
| Conformance Units |  |  |  |  |  |
| AAS-Discovery |  |  |  |  |  |

### GetSubmodel

Resolve the selected AASSubmodelFileType before returning its document and enforce the same Session-specific effective RolePermissions, UserRolePermissions, DisclosureTier, Authorization and FileType Open/Read decision as direct access to that target. Call permission on this Method does not authorize the target. Return Bad_UserAccessDenied, or Bad_NotFound where policy conceals existence, without exposing controlled bytes, Format, ContentType, other target metadata or a distinguishable timing path.

Signature

GetSubmodel (

[in]  0:String       SubmodelIdentifier,

[out] 0:ByteString   Document,

[out] 0:String       Format,

[out] 0:String       ContentType);

Table 125 – GetSubmodel Method Arguments

| Argument | Description |
| --- | --- |
| SubmodelIdentifier | The submodel's authored identifier. |
| Document | The submodel document bytes. |
| Format | xRegistry format string. |
| ContentType | Document media type. |

Table 126 – GetSubmodel Method AddressSpace definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | GetSubmodel |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | ModellingRule |
| 0:HasProperty | Variable | 0:InputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |
| 0:HasProperty | Variable | 0:OutputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |
| Conformance Units |  |  |  |  |  |
| AAS-Discovery |  |  |  |  |  |

### Materialize

Re-materialize the AddressSpace from the stored documents. The functional method on the well-known AASRegistry object.

Signature

Materialize (

[in]  0:String[]     Targets,

[in]  0:Boolean      Force,

[out] 0:UInt32       Generation,

[out] AASMaterializationResultDataType[] Results);

Table 127 – Materialize Method Arguments

| Argument | Description |
| --- | --- |
| Targets | The documents to consider, as registry-relative paths. An empty array means every document. |
| Force | Re-materialize even a document whose digest is unchanged. |
| Generation | The generation in force after the call. |
| Results | One result per document considered. |

Table 128 – Materialize Method AddressSpace definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | Materialize |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | ModellingRule |
| 0:HasProperty | Variable | 0:InputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |
| 0:HasProperty | Variable | 0:OutputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |
| Conformance Units |  |  |  |  |  |
| AAS-UpdateableRegistry |  |  |  |  |  |

# Profiles and conformance units

An implementation conforms to this specification if it implements at least one of the two halves and

declares the corresponding conformance units.

Table 129 – Profiles and conformance units

| Unit | Requires |
| --- | --- |
| AAS-Metamodel | Shells, submodels and concept descriptions as typed nodes. |
| AAS-SubmodelElements | The submodel element types. |
| AAS-ValueFidelity | The xsd type assignment of clauses 6.1.2 and 6.3.1. |
| AAS-InstanceMaterialization | Materialization per clause 6.1.6. |
| AAS-LosslessRoundTrip | Both directions of clause 6.4. |
| AAS-Registry | The registry root, groups and submodel documents. |
| AAS-RegistryIdentity | Source identities and derived identifiers per clause 6.5.3. |
| AAS-RegistryVersioning | Versions as the lifecycle record, clause 6.5.4. |
| AAS-Discovery | LookupShellsByAssetLink and GetSubmodel. |
| AAS-OperationInvoke | AASOperationType.Invoke, clause 6.2.5. |
| AAS-Federation | External references and the identity rule of clause 6.5.6. |
| AAS-DisclosureTiers | DisclosureTier and Authorization, clause 6.5.7. |
| AAS-UpdateableRegistry | Generational materialization from stored documents, clause 6.5.9. |
| AAS-EnvironmentExport | The materialized environment served as filtered AAS and AASX documents, clause 6.5.10. Required of a Server claiming both AAS-Registry and AAS-InstanceMaterialization. |
| AAS-Packages | Package stores and package resources; requires AAS-PackageIntegrity. |
| AAS-PackageIntegrity | Mandatory package-blob digest and algorithm, OCI manifest-to-blob binding, immutable identity and tag rules, referrer separation and verification requirements of clause 6.5.4. |

AAS-Metamodel and AAS-SubmodelElements together are the baseline for the metamodel half;

AAS-Registry and AAS-RegistryIdentity for the registry half. AAS-ValueFidelity is required by

AAS-LosslessRoundTrip, which is the unit that makes source generation possible. An implementation

claiming AAS-Packages shall also claim AAS-PackageIntegrity.

# Namespaces

## Namespace metadata

The namespace metadata provide standardized information about the elements of this namespace, which an aggregating Server relies on. All Nodes defined by this document are static: they are identical in every Server, including the Value Attribute.

The information is provided as an Object of type NamespaceMetadataType, a component of the Namespaces Object of the Server Object. The NamespaceMetadataType ObjectType and its Properties are defined in OPC 10000-5. The same version information is carried by the ModelTableEntry of the UANodeSet XML file.

Table 130 – NamespaceMetadata Object for this document

| Property | DataType | Value |
| --- | --- | --- |
| NamespaceUri | String | http://opcfoundation.org/UA/I4AAS/v3/ |
| NamespaceVersion | String | 3.00-draft3 |
| NamespacePublicationDate | DateTime | 2026-08-11 |
| IsNamespaceSubset | Boolean | False |
| StaticNodeIdTypes | IdType[] | 0 (Numeric) |
| StaticNumericNodeIdRange | NumericRange[] | 1001:5193 |
| StaticStringNodeIdPattern | String | -- |

The IsNamespaceSubset Property is False because the UANodeSet XML file contains the complete namespace. A Server exposing only a subset sets it to True.

## Handling of OPC UA namespaces

Namespaces are used by OPC UA to create unique identifiers across different naming authorities. The NodeId and BrowseName Attributes are identifiers. A Node in the AddressSpace is unambiguously identified by its NodeId; a BrowseName is not unique and is used to build a browse path or to name a standard Property.

Table 131 – Namespaces used by an Asset Administration Shell Server

| NamespaceURI | Description |
| --- | --- |
| http://opcfoundation.org/UA/ | Namespace for NodeIds and BrowseNames defined in the OPC UA specification (OPC 10000-3, OPC 10000-5). |
| Local Server URI | Namespace for Nodes defined in the local Server. This namespace shall have index 1. |
| http://opcfoundation.org/UA/xRegistry/ | Namespace of the RequiredModel that carries the abstract registry base types this document extends. |
| http://opcfoundation.org/UA/I4AAS/v3/ | Namespace for NodeIds and BrowseNames defined in this document. |
| Vendor specific types | A Server may provide vendor-specific types derived from the types defined in this document, in one or more additional namespaces. |

The following table lists the namespaces and their indexes used for BrowseNames in this document. The default namespace of this document is not listed, because every BrowseName without a prefix uses it.

Table 132 – Namespaces used in this document

| NamespaceURI | Namespace Index | Example |
| --- | --- | --- |
| http://opcfoundation.org/UA/ | 0 | 0:EngineeringUnits |
| http://opcfoundation.org/UA/xRegistry/ | 1 | 1:ResourceType |

(normative) 

Asset Administration Shell namespace and mappings

NodeSet and supplementary files for Asset Administration Shell Information Model

An Information Model is formally defined in an XML file called a NodeSet This file conforms to the standard syntax defined in the Annex “Information Model XML Schema” OPC 10000-6. It can be read and processed by a computer program.

An Information Model is identified by a URI – the so-called NamespaceUri.

A NamespaceUri follows one of these conventions:

http://opcfoundation.org/UA/I4AAS/

tag:opcfoundation.org,yyyy-MM:UA:I4AAS

Where I4AAS is described in 15 and yyyy-MM is the date when the NamespaceUri was first published. NamespaceUris are not network accessible URLs and the text should not suggest they are. The tag URI syntax allows authors to choose a URI that cannot be used as URL by mistake. Note that the date in the tag syntax is not the same as the PublicationDate for the NodeSet. It is set once when the URI is created and never changed.

The Online Reference provides a summary page for every NamespaceUri of released Information Models which has the form:

https://reference.opcfoundation.org/nodesets?u=http://opcfoundation.org/UA/I4AAS/v3/

The Asset Administration Shell Information Model is identified by the following URI:

http://opcfoundation.org/UA/I4AAS/

Documentation for the NamespaceUri can be found here.

“here” is a hyperlink to the summary page of the Online Reference.

In the hyperlink, the http://opcfoundation.org/UA/I4AAS/v3/ has to be replaced with the concrete URI for this specification.

The NodeSet associated with this version of specification can be found here:

https://reference.opcfoundation.org/nodesets/?u=http://opcfoundation.org/UA/I4AAS/v3/&v=3.00-draft3&i=1

The NodeSet associated with the latest version of the specification can be found here:

https://reference.opcfoundation.org/nodesets/?u=http://opcfoundation.org/UA/I4AAS/v3/&i=1

http://opcfoundation.org/UA/I4AAS/v3/ is the NamespaceUri for the Information Model.

The 3.00-draft3 is the string in the NamespaceVersion from the Namespace Metadata (see 16.1). This value is also the value of the Version attribute in the NodeSet.

Supplementary files for the Asset Administration Shell Information Model can be found here:

https://reference.opcfoundation.org/nodesets/?u=http://opcfoundation.org/UA/I4AAS/v3/&v=3.00-draft3&i=2

The files associated with the latest version of the specification can be found here:

https://reference.opcfoundation.org/nodesets/?u=http://opcfoundation.org/UA/I4AAS/v3/&i=2

Supplementary files should be provided when appropriate (i.e., IRDI NodeSets or examples)

File Names

NodeIds: 	Opc.Ua.I4AAS.NodeIds.csv

NodeSet: 	Opc.Ua.I4AAS.NodeSet2.xml;

Any other files should have a prefix that provides context when the file is downloaded in a browser.

When DataTypes are defined in the NodeSet:

Opc.ua.I4AAS.Types.xsd –

The XML schema for the DataTypes defined by the specification;

Opc.ua.I4AAS.Types.bsd –

The OPC Binary schema for the DataTypes defined by the specification;

In case there is the need to reference individual files the following syntax should be added to the template:

https://reference.opcfoundation.org/files/<file>?u=<namespaceUri>&v=<version>
https://reference.opcfoundation.org/files/<file>?u=<namespaceUri>

for example:

https://reference.opcfoundation.org/files/StatusCode.csv?u=http://opcfoundation.org/UA/&v=1.05.02
https://reference.opcfoundation.org/files/StatusCode.csv?u=http://opcfoundation.org/UA/
https://reference.opcfoundation.org/files/Opc.Ua.Types.xsd?u=http://opcfoundation.org/UA/&v=1.05.02
https://reference.opcfoundation.org/files/Opc.Ua.Types.xsd?u=http://opcfoundation.org/UA/

Capability identifier

ServerCapabilityIdentifiers are defined in OPC 10000-12. They can be used for features, like certain information models, which are likely to be useful during the discovery process. The identifiers shall be short because of length restrictions for fields used in the mDNS specification.

The identifier shall be up to 6 characters. It is recommended to use the I4AAS introduced in the guideline, at the beginning of clause 15 if that meets the requirement of up to 6 characters.

Note, that such identifiers are not required. If not needed, this Annex section shall be deleted.

The capability identifier for this document shall be:

I4AAS

Node reference

The following table lists every Node this document defines, with the provisional NodeId assigned by the generator. It is generated from the UANodeSet and is informative for navigation; the UANodeSet is authoritative.

Table 143 – Nodes defined by this document

| NodeId | BrowseName | NodeClass | Description |
| --- | --- | --- | --- |
| ns=2;i=1180 | AASAnyUri | DataType | An xs:anyURI value. A subtype of String, since String carries xs:string. |
| ns=2;i=1181 | AASHexBinary | DataType | An xs:hexBinary value. ByteString carries xs:base64Binary, whose octets are the same, so the hexadecimal form is carried by this subtype. |
| ns=2;i=1182 | AASNonPositiveInteger | DataType | An xs:nonPositiveInteger value: an integer at most zero. |
| ns=2;i=1183 | AASNegativeInteger | DataType | An xs:negativeInteger value: an integer below zero. A subtype of AASNonPositiveInteger, following the xsd restriction hierarchy. |
| ns=2;i=1184 | AASPositiveInteger | DataType | An xs:positiveInteger value: an integer above zero. A subtype of UInteger, which carries xs:nonNegativeInteger. |
| ns=2;i=1185 | AASGYear | DataType | An xs:gYear value, such as 2026. A Gregorian year denotes a period, for which OPC UA has no DataType, so the value is its lexical form. |
| ns=2;i=1186 | AASGYearMonth | DataType | An xs:gYearMonth value, such as 2026-08. |
| ns=2;i=1187 | AASGMonth | DataType | An xs:gMonth value, such as --08. |
| ns=2;i=1188 | AASGMonthDay | DataType | An xs:gMonthDay value, such as --08-07. |
| ns=2;i=1189 | AASGDay | DataType | An xs:gDay value, such as ---07. |
| ns=2;i=1199 | AASValueString | DataType | The xsd lexical form of a value whose declared type is carried in a sibling field of the same Structure. A Structure field has one static DataType and cannot vary with a declared type, so a qualifier, an extension or a data specification carries its value lexically and its sibling ValueType or DataType field states how to read it. A subtype of String, as OPC UA defines DecimalString and DurationString. It is never the DataType of a Variable; a value node carries the DataType clause 6.3.1 assigns to its declared xsd type. |
| ns=2;i=1200 | AASAssetKindDataType | DataType | Whether a shell describes a product model, an individual item, a batch, a role, or none of these. The three granularity levels a product passport is issued at map onto Type, Instance and Batch. |
| ns=2;i=1201 | AASModellingKindDataType | DataType | Whether an element defines a shape or carries values. |
| ns=2;i=1202 | AASEntityTypeDataType | DataType | Whether a composition entity is managed within its parent or has a shell of its own. |
| ns=2;i=1203 | AASDirectionDataType | DataType | The direction of an event element. |
| ns=2;i=1204 | AASStateOfEventDataType | DataType | Whether an event element is currently active. |
| ns=2;i=1205 | AASQualifierKindDataType | DataType | What a qualifier qualifies, and therefore whether it may change. |
| ns=2;i=1206 | AASReferenceTypesDataType | DataType | Whether a reference addresses something inside the model or outside it. |
| ns=2;i=1207 | AASKeyTypesDataType | DataType | The kind of thing a reference key addresses. The enumeration is closed: a value outside it cannot round-trip, so an implementation rejects it rather than dropping it. |
| ns=2;i=1208 | AASDataTypeDefXsdDataType | DataType | The xsd type a value is expressed in. All thirty of the metamodel's values are listed. Clause 6.3.1 assigns each one OPC UA DataType, and no DataType to two of them. |
| ns=2;i=1209 | AASDataTypeIec61360DataType | DataType | The data type of a concept definition expressed in the IEC 61360 data specification. |
| ns=2;i=1210 | AASSubmodelElementsDataType | DataType | The element kind a SubmodelElementList constrains its members to. |
| ns=2;i=1211 | AASDisclosureTierDataType | DataType | Whether an entity is readable without authentication. It advertises the tier so a Consumer can discover it; it does not enforce it. |
| ns=2;i=1212 | AASLoadStateDataType | DataType | The materialization state of one stored document under the updateable registry profile. |
| ns=2;i=1213 | AASMaterializationOutcomeDataType | DataType | What a Materialize call did to one document. |
| ns=2;i=1220 | AASKeyDataType | DataType | One step of a reference path. Keys are ordered, and the order is part of the reference's meaning. |
| ns=2;i=5000 | Default Binary | Object |  |
| ns=2;i=5001 | Default JSON | Object |  |
| ns=2;i=1221 | AASReferenceDataType | DataType | A reference, external or model-navigating, expressed as an ordered key path. |
| ns=2;i=5002 | Default Binary | Object |  |
| ns=2;i=5003 | Default JSON | Object |  |
| ns=2;i=1222 | AASLangStringDataType | DataType | One language-tagged string. A multi-language value is an array of these, and the array order is preserved. |
| ns=2;i=5004 | Default Binary | Object |  |
| ns=2;i=5005 | Default JSON | Object |  |
| ns=2;i=1223 | AASSpecificAssetIdDataType | DataType | A domain-specific key an asset is discoverable by. |
| ns=2;i=5006 | Default Binary | Object |  |
| ns=2;i=5007 | Default JSON | Object |  |
| ns=2;i=1224 | AASAdministrativeInformationDataType | DataType | Administrative information. It records a single current revision: the entity's history is carried by the registry, which the metamodel has no equivalent of. |
| ns=2;i=5008 | Default Binary | Object |  |
| ns=2;i=5009 | Default JSON | Object |  |
| ns=2;i=1225 | AASQualifierDataType | DataType | A qualifier constraining or annotating an element. |
| ns=2;i=5010 | Default Binary | Object |  |
| ns=2;i=5011 | Default JSON | Object |  |
| ns=2;i=1226 | AASEmbeddedDataSpecificationDataType | DataType | A data specification carried by an element, paired with its content. |
| ns=2;i=5012 | Default Binary | Object |  |
| ns=2;i=5013 | Default JSON | Object |  |
| ns=2;i=1227 | AASDataSpecificationIec61360DataType | DataType | The IEC 61360 data specification content of a concept definition. |
| ns=2;i=5014 | Default Binary | Object |  |
| ns=2;i=5015 | Default JSON | Object |  |
| ns=2;i=1228 | AASExtensionDataType | DataType | A proprietary extension carried on a Referable. Extensions round-trip verbatim; a reader that does not understand one preserves it unchanged. |
| ns=2;i=5016 | Default Binary | Object |  |
| ns=2;i=5017 | Default JSON | Object |  |
| ns=2;i=1229 | AASResourceDataType | DataType | A pointer to external content, such as a thumbnail. |
| ns=2;i=5018 | Default Binary | Object |  |
| ns=2;i=5019 | Default JSON | Object |  |
| ns=2;i=1230 | AASOperationVariableDataType | DataType | One input, output or in-out variable of an operation, carried as a reference to the element node that holds it so that the element's own representation is not duplicated. |
| ns=2;i=5020 | Default Binary | Object |  |
| ns=2;i=5021 | Default JSON | Object |  |
| ns=2;i=1231 | AASAuthorizationOptionDataType | DataType | One authorization option a Consumer may use. It is authorization configuration only and never carries credentials, which are supplied out of band. |
| ns=2;i=5022 | Default Binary | Object |  |
| ns=2;i=5023 | Default JSON | Object |  |
| ns=2;i=1232 | AASAttestationDataType | DataType | A non-authoritative discovery hint for a separate attestation or OCI referrer Resource. It never represents a package Version, and its presence is not verification: a Consumer retrieves and verifies the separate artifact itself. |
| ns=2;i=5024 | Default Binary | Object |  |
| ns=2;i=5025 | Default JSON | Object |  |
| ns=2;i=1233 | AASMaterializationResultDataType | DataType | The result of materializing one document. A call returns one of these per document it considered, reporting per document whether it was unchanged, materialized, retired or failed. |
| ns=2;i=5026 | Default Binary | Object |  |
| ns=2;i=5027 | Default JSON | Object |  |
| ns=2;i=1001 | AASReferableType | ObjectType | Abstract base of everything in the metamodel that can be referred to by a short name. Carries the identifying and descriptive attributes every element has. |
| ns=2;i=5028 | IdShort | Variable | The short name, unique only within its parent. It is never an identifier: two elements from different publishers routinely share one. Absent for an element inside a SubmodelElementList, which is addressed by index instead. |
| ns=2;i=5029 | Category | Variable | Deprecated in the metamodel and retained only so that a document carrying it round-trips unchanged. |
| ns=2;i=5030 | DisplayNameSet | Variable | Display name per language. |
| ns=2;i=5031 | DescriptionSet | Variable | Description per language. |
| ns=2;i=5032 | Extensions | Variable | Proprietary extensions, preserved verbatim. |
| ns=2;i=5033 | ModelType | Variable | The metamodel class name of this element. It is redundant with the ObjectType and is carried so that a serialization produced from the AddressSpace is byte-identical to the one that produced it. |
| ns=2;i=1002 | AASIdentifiableType | ObjectType | Abstract base of the metamodel elements that carry a globally unique identifier: shells, submodels and concept descriptions. |
| ns=2;i=5034 | Id | Variable | The globally unique identifier, up to 2048 characters. It is arbitrary text and can never be a BrowseName, so it is carried here and the node is named by the derived identifier instead. |
| ns=2;i=5035 | Administration | Variable | Administrative information: a single current revision, with no history. |
| ns=2;i=1003 | AASHasSemanticsType | ObjectType | Abstract base of the elements that declare what concept they are an occurrence of. |
| ns=2;i=5036 | SemanticId | Variable | The concept this element is an occurrence of, by which an element is discoverable by meaning rather than by name. |
| ns=2;i=5037 | SupplementalSemanticIds | Variable | Further concepts this element corresponds to, which is how one element is made discoverable through more than one dictionary. |
| ns=2;i=1004 | AASHasKindType | ObjectType | Abstract base of the elements that distinguish a template from an instance. |
| ns=2;i=5038 | Kind | Variable | Whether this element defines a shape or carries values. |
| ns=2;i=1005 | AASHasDataSpecificationType | ObjectType | Abstract base of the elements that carry data specifications. |
| ns=2;i=5039 | EmbeddedDataSpecifications | Variable | Data specifications carried by this element. |
| ns=2;i=1006 | AASQualifiableType | ObjectType | Abstract base of the elements that can be qualified. |
| ns=2;i=5040 | Qualifiers | Variable | Qualifiers constraining or annotating this element. |
| ns=2;i=1010 | AASEnvironmentType | ObjectType | The container of shells, submodels and concept descriptions - the unit an AAS serialization carries and the root a source generator materializes into a Server. |
| ns=2;i=5041 | <AssetAdministrationShell> | Object | A shell held by this environment. |
| ns=2;i=5042 | <Submodel> | Object | A submodel held by this environment. Submodels are top-level: one submodel may be referenced by several shells, which is why they are not nested inside them. |
| ns=2;i=5043 | <ConceptDescription> | Object | A concept description held by this environment. |
| ns=2;i=1011 | AASType | ObjectType | An Asset Administration Shell: the digital representation of one asset, carrying the asset's identity and references to the submodels that describe it. |
| ns=2;i=5044 | AssetInformation | Object | The identity of the asset this shell represents. |
| ns=2;i=5045 | SubmodelReferences | Variable | References to the submodels describing this asset. A submodel is not owned by the shell that references it. |
| ns=2;i=5046 | DerivedFrom | Variable | The Type shell this Instance shell was derived from, so an individual item can be traced to its product model. |
| ns=2;i=5047 | EmbeddedDataSpecifications | Variable | Data specifications carried by this shell. |
| ns=2;i=1012 | AASAssetInformationType | ObjectType | The identity of the asset a shell represents, as distinct from the identity of the shell itself. |
| ns=2;i=5048 | AssetKind | Variable | Whether the asset is a product model, an individual item, a batch, a role, or none of these. |
| ns=2;i=5049 | GlobalAssetId | Variable | The globally unique identifier of the asset itself. Where the asset carries an identification link, that link is this value, and it is what connects a code scanned from a physical product to this Server. |
| ns=2;i=5050 | AssetType | Variable | The identifier of the asset type this asset is an occurrence of. |
| ns=2;i=5051 | SpecificAssetIds | Variable | The additional keys the asset is discoverable by. |
| ns=2;i=5052 | DefaultThumbnail | Variable | A pointer to a representative image of the asset. |
| ns=2;i=1013 | AASSubmodelType | ObjectType | One coherent aspect of an asset, identified in its own right and typed by its SemanticId: a nameplate, technical data, a carbon footprint, a bill of material. |
| ns=2;i=5053 | Kind | Variable | Whether this submodel carries values or defines a shape other submodels are built from. |
| ns=2;i=5054 | SemanticId | Variable | The concept this submodel is an occurrence of. |
| ns=2;i=5055 | SupplementalSemanticIds | Variable | Further concepts this submodel corresponds to. |
| ns=2;i=5056 | Qualifiers | Variable | Qualifiers on this submodel. |
| ns=2;i=5057 | EmbeddedDataSpecifications | Variable | Data specifications carried by this submodel. |
| ns=2;i=5058 | <SubmodelElement> | Object | An element of this submodel. |
| ns=2;i=1030 | AASConceptDescriptionType | ObjectType | The definition a SemanticId resolves to - what makes two submodels from different vendors comparable. |
| ns=2;i=5059 | IsCaseOf | Variable | Concepts in other dictionaries this concept corresponds to, which is how a Server bridges two classification systems without asserting that either is canonical. |
| ns=2;i=5060 | EmbeddedDataSpecifications | Variable | The data specifications defining this concept. |
| ns=2;i=1020 | AASSubmodelElementType | ObjectType | Abstract base of every element that can appear inside a submodel. |
| ns=2;i=5061 | SemanticId | Variable | The concept this element is an occurrence of. |
| ns=2;i=5062 | SupplementalSemanticIds | Variable | Further concepts this element corresponds to. |
| ns=2;i=5063 | Qualifiers | Variable | Qualifiers on this element. |
| ns=2;i=5064 | EmbeddedDataSpecifications | Variable | Data specifications carried by this element. |
| ns=2;i=5065 | Index | Variable | The element's zero-based position within an ordered containing construct: its parent SubmodelElementList or one variable role of its parent Operation. For an Operation variable value it is mandatory and the role array position is authoritative. For a list member it is optional and recommended wherever the list's order is relevant, because Browse is not required to return references in order. |
| ns=2;i=1021 | AASPropertyType | ObjectType | A single typed value. The value node carries the OPC UA DataType clause 6.3.1 assigns to the declared xsd type, from which the declared type is read. |
| ns=2;i=5066 | ValueType | Variable | The xsd type the value is expressed in. Mandatory: the metamodel makes it mandatory and the value optional, so a Property with no value has no value node whose DataType could carry it. |
| ns=2;i=5067 | Value | Variable | The value. Declared as BaseDataType here because the concrete DataType depends on ValueType; a materialized node carries the specific DataType clause 6.3.1 assigns. |
| ns=2;i=5068 | ValueId | Variable | A reference to the value, where the value is itself an identified concept. |
| ns=2;i=1022 | AASMultiLanguagePropertyType | ObjectType | A value expressed in one or more languages. The array order is preserved, because the metamodel's serialization is ordered and a round trip that reordered it would not reproduce its input. |
| ns=2;i=5069 | Value | Variable | The language-tagged values, in order. |
| ns=2;i=5070 | ValueId | Variable | A reference to the value, where the value is itself an identified concept. |
| ns=2;i=1023 | AASRangeType | ObjectType | A closed or half-open interval of a single typed value. |
| ns=2;i=5071 | ValueType | Variable | The xsd type the bounds are expressed in. Mandatory: both bounds are optional and the declared type is not. |
| ns=2;i=5072 | Min | Variable | The lower bound, carrying the DataType clause 6.3.1 assigns to ValueType. Absent means unbounded below, which is different from a bound of zero. |
| ns=2;i=5073 | Max | Variable | The upper bound. Absent means unbounded above. |
| ns=2;i=1024 | AASBlobType | ObjectType | Binary content carried inline. |
| ns=2;i=5074 | Value | Variable | The content bytes. |
| ns=2;i=5075 | ContentType | Variable | Media type of the content. |
| ns=2;i=1025 | AASFileType | ObjectType | A pointer to content held outside the element. |
| ns=2;i=5076 | Value | Variable | Path or URL to the content. |
| ns=2;i=5077 | ContentType | Variable | Media type of the content. |
| ns=2;i=1026 | AASReferenceElementType | ObjectType | An element whose value is a reference. |
| ns=2;i=5078 | Value | Variable | The reference. |
| ns=2;i=1027 | AASRelationshipElementType | ObjectType | A directed relationship between two referenced things. |
| ns=2;i=5079 | First | Variable | The first, or source, end of the relationship. |
| ns=2;i=5080 | Second | Variable | The second, or target, end of the relationship. |
| ns=2;i=1028 | AASAnnotatedRelationshipElementType | ObjectType | A relationship carrying data elements that annotate it, such as a quantity or a position. |
| ns=2;i=5081 | <Annotation> | Object | A data element annotating this relationship. |
| ns=2;i=1029 | AASSubmodelElementCollectionType | ObjectType | An unordered set of elements, each identified by its own IdShort. |
| ns=2;i=5082 | <SubmodelElement> | Object | An element of this collection. |
| ns=2;i=1031 | AASSubmodelElementListType | ObjectType | A list of elements. Its members have no IdShort, so they are named by index. Whether the order carries meaning is stated by the ReferenceType on each instance, not by a Property: HasOrderedComponent where it does, HasComponent where the list is a set or a bag. The declaration uses HasComponent, the base of both legal instance forms. |
| ns=2;i=5083 | TypeValueListElement | Variable | The element kind every member is constrained to. |
| ns=2;i=5084 | SemanticIdListElement | Variable | The concept every member is an occurrence of, where they share one. |
| ns=2;i=5085 | ValueTypeListElement | Variable | The xsd type every member's value is expressed in, where they share one. Mandatory in the metamodel when the members are Properties or Ranges. |
| ns=2;i=5086 | <Element> | Object | A member of this list, named by its index. The declaration uses HasComponent; an instance uses HasOrderedComponent where the list's order is relevant and HasComponent where it is not. |
| ns=2;i=1032 | AASEntityType | ObjectType | A component of a composition. A self-managed entity carries the identifier of its own shell, so a bill of material is traversable across organizations. |
| ns=2;i=5087 | EntityType | Variable | Whether the component has its own shell or is managed within its parent. |
| ns=2;i=5088 | GlobalAssetId | Variable | The identifier of the component's own asset, for a self-managed entity. |
| ns=2;i=5089 | SpecificAssetIds | Variable | Additional keys the component is discoverable by. |
| ns=2;i=5090 | <Statement> | Object | A statement about the component. |
| ns=2;i=1033 | AASBasicEventElementType | ObjectType | An event source or sink. |
| ns=2;i=5091 | Observed | Variable | What the event observes. |
| ns=2;i=5092 | Direction | Variable | Whether the event is produced or consumed. |
| ns=2;i=5093 | State | Variable | Whether the event source is active. |
| ns=2;i=5094 | MessageTopic | Variable | The topic events are delivered on. Where the delivery endpoint is itself catalogued, the registry entry points at it. |
| ns=2;i=5095 | MessageBroker | Variable | The broker delivering the events. |
| ns=2;i=5096 | LastUpdate | Variable | When the event last fired. The metamodel types this xs:dateTime, which clause 6.3.1 assigns DateTime. |
| ns=2;i=5097 | MinInterval | Variable | Minimum interval between events. The metamodel types this xs:duration, which clause 6.3.1 assigns DurationString. |
| ns=2;i=5098 | MaxInterval | Variable | Maximum interval between events. The metamodel types this xs:duration, which clause 6.3.1 assigns DurationString. |
| ns=2;i=1034 | AASOperationType | ObjectType | An invocable operation. |
| ns=2;i=5099 | InputVariables | Variable | The operation's input variables, in order. Each entry points to one direct operation-variable child and the array position is authoritative. |
| ns=2;i=5100 | OutputVariables | Variable | The operation's output variables, in order. Each entry points to one direct operation-variable child and the array position is authoritative. |
| ns=2;i=5101 | InoutputVariables | Variable | The operation's in-out variables, in order. Each entry points to one direct operation-variable child and the array position is authoritative. |
| ns=2;i=5102 | <Variable> | Object | A direct HasComponent child carrying one operation variable value. Its role array entry points to it by ValueNodeId, and its Index equals its position within that role. |
| ns=2;i=5103 | Invoke | Method | Invoke the operation and return its results. The Call counterpart of InvokeOperation in the AAS API of IDTA-01002 Part 2: a Client that has browsed to the Operation element calls this rather than reaching for the HTTP interface, and the two carry the same arguments in the same order. |
| ns=2;i=5104 | InputArguments | Variable |  |
| ns=2;i=5105 | OutputArguments | Variable |  |
| ns=2;i=1035 | AASCapabilityType | ObjectType | A declared capability of the asset. It carries no value of its own; the element's identity and semantics are the whole of its content. |
| ns=2;i=1100 | AASRegistryType | ObjectType | The AAS Registry root - an xRegistry RegistryType, and therefore a FolderType - whose group folders hold shells, submodel templates, concept dictionaries and packages. Exposed as a well-known object under the Server object, so any Client that reaches the standard Server object discovers it. |
| ns=2;i=5106 | <ShellGroup> | Object | A shell folder held by the registry. |
| ns=2;i=5107 | <SubmodelTemplateGroup> | Object | A submodel template family held by the registry. |
| ns=2;i=5108 | <ConceptDictionaryGroup> | Object | A concept dictionary held by the registry. |
| ns=2;i=5109 | <PackageStoreGroup> | Object | A package store held by the registry. |
| ns=2;i=5110 | <Environment> | Object | A serialization of one materialized environment, held by the registry as a retrievable document. |
| ns=2;i=5111 | LookupShellsByAssetLink | Method | Return the shells discoverable by an asset key. This is the discovery question - given a serial number or a part identifier, which shells describe it - answered without the caller browsing the whole collection. |
| ns=2;i=5112 | InputArguments | Variable |  |
| ns=2;i=5113 | OutputArguments | Variable |  |
| ns=2;i=5114 | GetSubmodel | Method | Resolve the selected AASSubmodelFileType before returning its document and enforce the same Session-specific effective RolePermissions, UserRolePermissions, DisclosureTier, Authorization and FileType Open/Read decision as direct access to that target. Call permission on this Method does not authorize the target. Return Bad_UserAccessDenied, or Bad_NotFound where policy conceals existence, without exposing controlled bytes, Format, ContentType, other target metadata or a distinguishable timing path. |
| ns=2;i=5115 | InputArguments | Variable |  |
| ns=2;i=5116 | OutputArguments | Variable |  |
| ns=2;i=5117 | AutoMaterialize | Variable | Whether a change to a stored document re-materializes the AddressSpace without being asked. Part of the updateable registry profile. |
| ns=2;i=5118 | MaterializationGeneration | Variable | Increments once on each committed switch. A Client correlates a node's NodeVersion with the generation that produced it. |
| ns=2;i=5119 | Materialize | Method | Re-materialize the AddressSpace from the stored documents. Part of the updateable registry profile: the documents are canonical and the nodes are derived, so this is the operation that makes the derived side agree with the canonical one. |
| ns=2;i=5120 | InputArguments | Variable |  |
| ns=2;i=5121 | OutputArguments | Variable |  |
| ns=2;i=1101 | AASShellGroupType | ObjectType | An xRegistry GroupType holding the submodel documents of one shell. Its source identity is the shell's authored identifier, from which the GroupId is constructed. It is distinct from AASType, which models the same shell as a live node tree rather than as a catalogue entry. |
| ns=2;i=5122 | AasIdentifier | Variable | The shell's authored identifier, verbatim. It is the group's source identity: the GroupId is the symbolic identifier constructed from it, and Name is this identifier. |
| ns=2;i=5123 | AssetKind | Variable | Whether the shell describes a product model, an individual item or a batch. |
| ns=2;i=5124 | GlobalAssetId | Variable | The identifier of the asset itself, as distinct from the shell describing it. |
| ns=2;i=5125 | AssetType | Variable | The identifier of the asset type this asset is an occurrence of. |
| ns=2;i=5126 | SpecificAssetIds | Variable | The keys this shell is discoverable by. |
| ns=2;i=5127 | Administration | Variable | Administrative information carried by the shell. |
| ns=2;i=5128 | DerivedFrom | Variable | The identifier of the Type shell this Instance shell was derived from. |
| ns=2;i=5129 | DisclosureTier | Variable | Whether this entity is readable without authentication. |
| ns=2;i=5130 | Authorization | Variable | The authorization options a Consumer may use to obtain access. |
| ns=2;i=5131 | EventEndpoint | Variable | The catalogued endpoint delivering change events for this shell, where one is published. |
| ns=2;i=5132 | ShellNode | Variable | The AASType node modelling this same shell as a live node tree, where the Server also implements the metamodel half. The catalogue entry and the node tree are different nodes for the same shell, and this is the link between them. |
| ns=2;i=5133 | <Submodel> | Object | A submodel document held by this shell. |
| ns=2;i=1102 | AASSubmodelFileType | ObjectType | An xRegistry ResourceType whose file content is one submodel document. Each version is one revision, which is what gives a shell the lifecycle history the metamodel does not itself provide. |
| ns=2;i=5134 | SubmodelIdentifier | Variable | The submodel's authored identifier, verbatim. It is the resource's source identity, from which the ResourceId is constructed, and it is invariant across the submodel's versions. |
| ns=2;i=5135 | SemanticId | Variable | The concept this submodel is an occurrence of - the attribute a Consumer filters on to find, for example, every carbon footprint submodel in a registry. |
| ns=2;i=5136 | SupplementalSemanticIds | Variable | Further concepts this submodel corresponds to. |
| ns=2;i=5137 | Kind | Variable | Whether the submodel carries values or defines a shape. |
| ns=2;i=5138 | Template | Variable | The identifier of the template this submodel was built from. It is an identifier and not a pointer, so it resolves identically whether or not this registry also serves the template. |
| ns=2;i=5139 | Digest | Variable | Digest of the exact document bytes a Consumer retrieves. A registry does not publish one for bytes it has not itself seen. |
| ns=2;i=5140 | DigestAlg | Variable | The algorithm used to compute Digest. Present whenever Digest is. |
| ns=2;i=5141 | IsDefault | Variable | Whether this is the version served when none is selected. |
| ns=2;i=5142 | Ancestor | Variable | The version this one derives from. A root version's ancestor is itself. |
| ns=2;i=5143 | DisclosureTier | Variable | Whether this document is readable without authentication. A document is wholly one tier or the other: a boundary falling between elements inside a document cannot be expressed here. |
| ns=2;i=5144 | Authorization | Variable | The authorization options a Consumer may use to obtain access. |
| ns=2;i=5145 | SubmodelNode | Variable | The AASSubmodelType node modelling this same submodel as a live node tree, where the Server also implements the metamodel half. |
| ns=2;i=5146 | LoadState | Variable | The materialization state of this document. Part of the updateable registry profile. |
| ns=2;i=5147 | DesiredVersionId | Variable | The version an operator wants materialized. Part of the updateable registry profile. |
| ns=2;i=5148 | ActiveVersionId | Variable | The version currently materialized. It differs from DesiredVersionId while a switch is in flight, and persistently when the desired version failed to validate. |
| ns=2;i=1103 | AASSubmodelTemplateGroupType | ObjectType | An xRegistry GroupType holding one publisher's family of submodel templates. Templates are held in a group of their own so that a Consumer lists templates and instances separately. |
| ns=2;i=5149 | TemplateNamespace | Variable | The publisher's template namespace, verbatim. It is the group's source identity. |
| ns=2;i=5150 | Publisher | Variable | The organization publishing this template family. |
| ns=2;i=5151 | <Submodel> | Object | A submodel template held by this family. |
| ns=2;i=1104 | AASConceptDictionaryGroupType | ObjectType | An xRegistry GroupType holding one dictionary of concept definitions - the definitions a SemanticId elsewhere in the registry resolves to. |
| ns=2;i=5152 | DictionaryIdentifier | Variable | The dictionary's identifier, verbatim. It is the group's source identity. |
| ns=2;i=5153 | <ConceptDescription> | Object | A concept definition held by this dictionary. |
| ns=2;i=1105 | AASConceptDescriptionFileType | ObjectType | An xRegistry ResourceType whose file content is one concept description document. |
| ns=2;i=5154 | ConceptIdentifier | Variable | The concept's authored identifier, verbatim, which is the value that appears as a SemanticId elsewhere. It is the resource's source identity. Dictionary identifiers frequently use a syntax unrelated to any URI scheme, so the authored identifier is carried here and the node is named by the derived one. |
| ns=2;i=5155 | IsCaseOf | Variable | Concepts in other dictionaries this concept corresponds to. |
| ns=2;i=5156 | ConceptNode | Variable | The AASConceptDescriptionType node modelling this same concept as a live node tree, where the Server also implements the metamodel half. |
| ns=2;i=5157 | LoadState | Variable | The materialization state of this document. Part of the updateable registry profile. |
| ns=2;i=5158 | DesiredVersionId | Variable | The version an operator wants materialized. Part of the updateable registry profile. |
| ns=2;i=5159 | ActiveVersionId | Variable | The version currently materialized. |
| ns=2;i=1106 | AASPackageStoreGroupType | ObjectType | An xRegistry GroupType holding packages - one store, or one namespace within one. |
| ns=2;i=5160 | StoreIdentifier | Variable | The store's identifier, verbatim. It is the group's source identity. |
| ns=2;i=5161 | RegistryUrl | Variable | Base URL of the backing package store. |
| ns=2;i=5162 | <Package> | Object | A package held by this store. |
| ns=2;i=1107 | AASPackageFileType | ObjectType | An xRegistry ResourceType whose file content is one package. Every package carries mandatory strong integrity metadata for the exact returned blob; an OCI-backed version also carries the immutable manifest digest that is its version identity. Mutable tags are Resource-level discovery aliases, never Version identity, and OCI referrers are separate Resources rather than package Versions and cannot affect the package default Version. |
| ns=2;i=5163 | PackageIdentifier | Variable | The package's name as held by the backing store, verbatim. It is the resource's source identity. |
| ns=2;i=5164 | ArtifactType | Variable | The media type identifying what the artifact is, where the backing store carries one. |
| ns=2;i=5165 | Digest | Variable | Immutable lower-case hexadecimal digest, without an algorithm prefix, of the exact package blob bytes returned by FileType Read. It is Mandatory on every Version. The Server verifies it before publication, and a Consumer recomputes it before parsing, materializing or otherwise using the package. |
| ns=2;i=5166 | DigestAlg | Variable | Immutable case-sensitive algorithm used to compute Digest. Only the exact spellings Sha256, Sha384 and Sha512 are valid. OCI descriptor algorithms sha256, sha384 and sha512 map respectively to those values; all other algorithms or casing are rejected. |
| ns=2;i=5167 | AasIdentifiers | Variable | The shell identifiers this package contains, so a Consumer can tell what it holds without retrieving and opening it. |
| ns=2;i=5168 | Subject | Variable | Reserved Variable NodeId. It is not an InstanceDeclaration of AASPackageFileType; an attestation or OCI referrer is a separate immutable Resource and never a package Version. |
| ns=2;i=5169 | Attestations | Variable | Reserved Variable NodeId. It is not an InstanceDeclaration of AASPackageFileType; attestations and OCI referrers are separate immutable Resources and never affect the package default Version. |
| ns=2;i=1108 | AASEnvironmentFileType | ObjectType | An xRegistry ResourceType whose file content is one serialization of a materialized environment: an AAS JSON or XML environment document, or an AASX package. It is the retrievable form of an AASEnvironmentType folder, and its content is filtered to what the calling Session is permitted to read. |
| ns=2;i=5170 | EnvironmentIdentifier | Variable | The environment's identifier, verbatim. It is the resource's source identity, from which the ResourceId is constructed. |
| ns=2;i=5171 | Format | Variable | The serialization of the document: an xRegistry format string such as aas/3.0+json, aas/3.0+xml or aasx/3.0. |
| ns=2;i=5172 | EnvironmentNode | Variable | The AASEnvironmentType folder this document serializes. |
| ns=2;i=5173 | Digest | Variable | Digest of the exact document bytes a Consumer retrieves. A Server does not publish one for a document whose content depends on the caller's permissions. |
| ns=2;i=5174 | DigestAlg | Variable | The algorithm used to compute Digest. Present whenever Digest is. |
| ns=2;i=5175 | Filtered | Variable | Whether the document served to this Session omits content the Session is not permitted to read. |
| ns=2;i=5176 | DisclosureTier | Variable | Whether this document is readable without authentication. |
| ns=2;i=5177 | Authorization | Variable | The authorization options a Consumer may use to obtain access. |
| ns=2;i=1150 | AASRegistry | Object | Server-wide AAS Registry, a well-known component of the Server object. |
| ns=2;i=5178 | LookupShellsByAssetLink | Method | Return the shells discoverable by an asset key. The functional method on the well-known AASRegistry object. |
| ns=2;i=5179 | InputArguments | Variable |  |
| ns=2;i=5180 | OutputArguments | Variable |  |
| ns=2;i=5181 | GetSubmodel | Method | Resolve the selected AASSubmodelFileType before returning its document and enforce the same Session-specific effective RolePermissions, UserRolePermissions, DisclosureTier, Authorization and FileType Open/Read decision as direct access to that target. Call permission on this Method does not authorize the target. Return Bad_UserAccessDenied, or Bad_NotFound where policy conceals existence, without exposing controlled bytes, Format, ContentType, other target metadata or a distinguishable timing path. |
| ns=2;i=5182 | InputArguments | Variable |  |
| ns=2;i=5183 | OutputArguments | Variable |  |
| ns=2;i=5184 | Materialize | Method | Re-materialize the AddressSpace from the stored documents. The functional method on the well-known AASRegistry object. |
| ns=2;i=5185 | InputArguments | Variable |  |
| ns=2;i=5186 | OutputArguments | Variable |  |
| ns=2;i=1234 | AASValueReferencePairDataType | DataType | One permitted value paired with the reference identifying its meaning. |
| ns=2;i=5187 | Default Binary | Object |  |
| ns=2;i=5188 | Default JSON | Object |  |
| ns=2;i=1235 | AASValueListDataType | DataType | The non-empty list of permitted values for an IEC 61360 data specification. |
| ns=2;i=5189 | Default Binary | Object |  |
| ns=2;i=5190 | Default JSON | Object |  |
| ns=2;i=1236 | AASLevelTypeDataType | DataType | The four IEC 61360 level flags. Every flag is explicit. |
| ns=2;i=5191 | Default Binary | Object |  |
| ns=2;i=5192 | Default JSON | Object |  |
| ns=2;i=5193 | ManifestDigest | Variable | Immutable exact OCI manifest digest with its lower-case algorithm prefix and lower-case hexadecimal value. It is Mandatory for every OCI-backed Version, is the sole authority for that Version's identity, and produces its always-hashed symbolic VersionId; a mutable tag is never identity. It verifies only the exact manifest bytes, never the returned package blob. The verified manifest has exactly one package-layer descriptor whose algorithm and encoded digest map to DigestAlg and Digest; the Server verifies this chain before publication and a Consumer repeats it before use. |

(normative)

Field coverage

<a id="annex-b"></a>

This annex is normative. It is the losslessness argument in table form: every field of the AAS V3

metamodel, and where it lives in the AddressSpace. A field absent from this table is a defect.

Referable and Identifiable

Table 133 – Referable and Identifiable

| Metamodel field | Address space |
| --- | --- |
| idShort | AASReferableType.IdShort; absent for a list member, which is addressed by index |
| category | AASReferableType.Category |
| displayName | AASReferableType.DisplayNameSet |
| description | AASReferableType.DescriptionSet |
| extensions | AASReferableType.Extensions |
| modelType | AASReferableType.ModelType, Mandatory |
| id | AASIdentifiableType.Id, Mandatory, and input to the String NodeId encoding |
| administration | AASIdentifiableType.Administration |

Shell and asset information

Table 134 – Shell and asset information

| Metamodel field | Address space |
| --- | --- |
| assetInformation | AASType.AssetInformation, Mandatory |
| submodels | AASType.SubmodelReferences |
| derivedFrom | AASType.DerivedFrom |
| embeddedDataSpecifications | AASType.EmbeddedDataSpecifications |
| assetKind | AASAssetInformationType.AssetKind, Mandatory |
| globalAssetId | AASAssetInformationType.GlobalAssetId |
| assetType | AASAssetInformationType.AssetType |
| specificAssetIds | AASAssetInformationType.SpecificAssetIds |
| defaultThumbnail | AASAssetInformationType.DefaultThumbnail |

Submodel and concept description

Table 135 – Submodel and concept description

| Metamodel field | Address space |
| --- | --- |
| kind | AASSubmodelType.Kind |
| semanticId | AASSubmodelType.SemanticId |
| supplementalSemanticIds | AASSubmodelType.SupplementalSemanticIds |
| qualifiers | AASSubmodelType.Qualifiers |
| embeddedDataSpecifications | AASSubmodelType.EmbeddedDataSpecifications |
| submodelElements | AASSubmodelType components, one node per element |
| isCaseOf | AASConceptDescriptionType.IsCaseOf |
| embeddedDataSpecifications (concept) | AASConceptDescriptionType.EmbeddedDataSpecifications |

Submodel elements

Every element carries the AASReferableType fields of B.1 and, from AASSubmodelElementType,

SemanticId, SupplementalSemanticIds, Qualifiers, EmbeddedDataSpecifications and — inside a

list — Index. Element-specific fields:

Table 136 – Submodel elements

| Element and field | Address space |
| --- | --- |
| Property.valueType | AASPropertyType.ValueType, Mandatory |
| Property.value | AASPropertyType.Value, typed per clause 6.3.1 |
| Property.valueId | AASPropertyType.ValueId |
| MultiLanguageProperty.value | AASMultiLanguagePropertyType.Value, order preserved |
| MultiLanguageProperty.valueId | AASMultiLanguagePropertyType.ValueId |
| Range.valueType | AASRangeType.ValueType, Mandatory |
| Range.min, Range.max | AASRangeType.Min, Max, typed per clause 6.3.1; absent means unbounded |
| Blob.value | AASBlobType.Value |
| Blob.contentType | AASBlobType.ContentType, Mandatory |
| File.value | AASFileType.Value |
| File.contentType | AASFileType.ContentType, Mandatory |
| ReferenceElement.value | AASReferenceElementType.Value |
| RelationshipElement.first, .second | AASRelationshipElementType.First, Second, Mandatory |
| AnnotatedRelationshipElement.annotations | AASAnnotatedRelationshipElementType components |
| SubmodelElementCollection.value | AASSubmodelElementCollectionType components |
| SubmodelElementList.orderRelevant | the ReferenceType its members are referenced with: HasOrderedComponent when true, HasComponent when false (clause 6.1.4) |
| SubmodelElementList.typeValueListElement | AASSubmodelElementListType.TypeValueListElement, Mandatory |
| SubmodelElementList.semanticIdListElement | AASSubmodelElementListType.SemanticIdListElement |
| SubmodelElementList.valueTypeListElement | AASSubmodelElementListType.ValueTypeListElement |
| SubmodelElementList.value | AASSubmodelElementListType members, ordered by Index |
| Entity.entityType | AASEntityType.EntityType, Mandatory |
| Entity.globalAssetId | AASEntityType.GlobalAssetId |
| Entity.specificAssetIds | AASEntityType.SpecificAssetIds |
| Entity.statements | AASEntityType components |
| BasicEventElement.observed | AASBasicEventElementType.Observed, Mandatory |
| BasicEventElement.direction | AASBasicEventElementType.Direction, Mandatory |
| BasicEventElement.state | AASBasicEventElementType.State, Mandatory |
| BasicEventElement.messageTopic | AASBasicEventElementType.MessageTopic |
| BasicEventElement.messageBroker | AASBasicEventElementType.MessageBroker |
| BasicEventElement.lastUpdate | AASBasicEventElementType.LastUpdate, typed per clause 6.3.1 |
| BasicEventElement.minInterval, .maxInterval | AASBasicEventElementType.MinInterval, MaxInterval, typed per clause 6.3.1 |
| Operation.inputVariables | AASOperationType.InputVariables; each ordered AASOperationVariableDataType.ValueNodeId references one direct <Variable> child |
| Operation.outputVariables | AASOperationType.OutputVariables, with the same child contract |
| Operation.inoutputVariables | AASOperationType.InoutputVariables, with the same child contract |
| Capability | AASCapabilityType; the element has no own fields |

Value classes

Table 137 – Value classes

| Metamodel class and field | Address space |
| --- | --- |
| Reference.type, .referredSemanticId, .keys | AASReferenceDataType, keys ordered |
| Key.type, .value | AASKeyDataType |
| LangString.language, .text | AASLangStringDataType |
| SpecificAssetId fields | AASSpecificAssetIdDataType |
| AdministrativeInformation fields | AASAdministrativeInformationDataType |
| Qualifier fields | AASQualifierDataType, value as AASValueString |
| Extension fields | AASExtensionDataType, value as AASValueString |
| Resource.path, .contentType | AASResourceDataType |
| EmbeddedDataSpecification fields | AASEmbeddedDataSpecificationDataType |
| DataSpecificationIec61360 fields | AASDataSpecificationIec61360DataType |

<a id="annex-c"></a>

(informative)

OPC 30270 v1.00 correspondence

This annex is informative.

Table 138 – OPC 30270 v1.00 correspondence

| OPC 30270 v1.00 | This specification |
| --- | --- |
| AASAssetType | No direct counterpart. The asset's identity is AASAssetInformationType, a component of the shell. |
| AASViewType | No counterpart in the AAS V3 metamodel. |
| Identifier with a type discriminator | AASIdentifiableType.Id, a bare String |
| AASSubmodelElementCollectionType with ordering flags | Split into AASSubmodelElementCollectionType, unordered, and AASSubmodelElementListType, whose members are referenced with HasOrderedComponent and carry Index |
| One DataType shared by several xsd types | Clause 6.3.1: each of the thirty DataTypeDefXsd values is assigned its own OPC UA DataType |
| Data specification references | EmbeddedDataSpecifications |
| No catalogue | The registry half, clause 6.5 |

The two models have distinct namespace URIs. A Server may load both, and a Client identifies the

model by NamespaceUri rather than treating the model version as NodeId identity.

<a id="annex-d"></a>

(informative)

Correspondence to the xRegistry HTTP binding

This annex is informative. The registry half of this specification and the xRegistry AAS model

mirrored beside it describe one registry; this table maps the OPC UA realization onto the HTTP one.

Table 139 – Correspondence to the xRegistry HTTP binding

| xRegistry over HTTP | This specification |
| --- | --- |
| GET /shells | Browse the registry's shell group folder |
| GET /shells/<ID> | Browse or Read the AASShellGroupType node |
| GET /shells/<ID>/submodels/<SM> | Open, Read, Close on the AASSubmodelFileType file |
| GET …$details | Read the resource node's Properties |
| POST /shells | CreateGroup on the registry root |
| DELETE /shells/<ID> | Delete on the group node |
| ?filter=specificassetids[*].value=… | LookupShellsByAssetLink |
| ?filter=semanticid=… | Browse the shell's submodels and Read SemanticId |
| Versions collection | The resource's version files |
| <RESOURCE>url | ResourceUrl, or ExternalReference as an ExpandedNodeId |

<a id="annex-e"></a>

(informative)

Federation resolution

This annex is informative and follows the base model's own resolution algorithm.

- Read the entity's ExternalReference.

- If its ServerUri is empty or identifies the local Server, resolve the target in the local AddressSpace.

- Otherwise obtain the endpoint URL for ServerUri, open a secure channel and session to it, translate the NamespaceUri to the remote namespace index, and read the referenced entity there with the same Methods used locally.

- Where only ResourceUrl is present, use it as the external locator and treat the remote bytes and metadata as the representation of the same entity identity carried by the identifier attributes.

Before step 3 or 4 opens a connection, apply the federation egress and peer-identity policy of

6.5.6 to the initial target and to every resolution, redirect and connected address. The metadata is

an input to policy, never authorization to contact the target.

Because identifiers are stable across registries while the endpoint identifies only where an entity

is served, an entity federated from several registries keeps one identity and can be de-duplicated

by identifier even though it is reachable through several links.

<a id="annex-f"></a>

(informative)

Correspondence to a Thing Description projection

This annex is informative, and is pinned to the OPC UA — WoT Connectivity and *OPC UA — WoT

and a change there can invalidate what follows.

A Thing Description carrying the terms below, loaded through a WoT Connectivity registry,

materializes the nodes clause 6.1.6 defines. An author therefore writes an AAS once, as a WoT

document, and obtains both the Thing and the AddressSpace of this specification; and by clause 5A of

the JSON-LD mapping the same content exports as AAS JSON, XML and AASX through a registry.

projection rules of this annex, and compares the result against the node set the reference

materializer produces from the same environment. Both sides of that comparison are implemented

alongside this document, so it establishes that the rules below are self-consistent and complete for

the reference fixtures. It is not a test of a WoT Connectivity implementation.

absent-versus-empty, every-element-type, identifiable-without-idshort,

non-canonical-lexical-forms and ordering-and-nesting. Every file in a bundle is one valid Thing

Description object.

Scope of the claim

The claim covers the projection subgraph: the nodes clause 6.1.6 materializes for the submodels

of one environment and their submodel elements, with their NodeIds, BrowseNames, TypeDefinitions

and the ReferenceType each is reached by.

Shells and concept descriptions are outside it. They project by the same rules, but the

correspondence has not been exercised for them and this annex does not claim it.

Also outside it are the nodes a registry adds on its own account — the document resource, its

versions, the reference from a document to its projection. Those exist because a registry is

present, not because an AAS is being mapped.

Granularity

The projection emits one Thing Description per projected OPC UA Object: one for the Submodel

and one for every submodel element Object below it. The TDs form one publication bundle.

This granularity follows the OPC UA WoT Binding term domains. uav:object is a Thing-level type,

uav:variable is a property-affordance type and uav:method is an action-affordance type. A

submodel element that maps to an OPC UA Object therefore cannot be represented as a member of the

TD properties map. Sibling TD links carry containment between the Objects.

The AAS RDF graph is distributed across the bundle. The union of the sibling TD datasets is the

same AAS graph that clause 1 of the JSON-LD mapping defines, with the ordering occurrences that

clause 3 requires. A blank-node identifier is scoped to its TD document when those datasets are

combined.

Terms

Table 140 – Terms

| Fact of this specification | Term | Value |
| --- | --- | --- |
| The AAS itself | the AAS vocabulary | aas:Referable/idShort, aas:Property/value, aas:Submodel/submodelElements and the rest, on the node they belong to |
| AAS subject | TD id | the AAS identifier itself where it is an ordinary absolute IRI; otherwise the reserved encoded subject of the JSON-LD mapping; @id is not also written |
| TD document identity | a self link | a sibling TD document IRI distinct from the AAS subject |
| NodeId, clause 6.1.3 | uav:id | an ExpandedNodeId naming its namespace by URI |
| BrowseName, clause 6.1.3 | uav:browseName | the portable QualifiedName form |
| TypeDefinition, clause 6.2, readably | a member of @type | the prefix-qualified BrowseName of the ObjectType, for example i4aas:AASPropertyType |
| TypeDefinition, clause 6.2, definitively | a link with rel ua:HasTypeDefinition | the ObjectType's portable ExpandedNodeId, for example nsu=http://opcfoundation.org/UA/I4AAS/v3/;i=1021 |
| Protocol address | Thing-level forms[].href | an opc.tcp URL whose id query value decodes to uav:id |
| Child-to-parent containment | uav:componentOf and a uav:componentOf link | the parent ExpandedNodeId and parent sibling TD IRI |
| Parent-to-child containment | uav:hasComponent and a typed link | the child ExpandedNodeId and child sibling TD IRI |
| ReferenceType, clause 6.1.4 | typed link rel, uav:refId and optional uav:refName | ua:HasOrderedComponent with i=49, or ua:HasComponent with i=47; the reference name is the target BrowseName local name |
| Position, clause 6.1.4 | uav:index | the zero-based position |
| Modelling rule | uav:modellingRule | Mandatory, Optional, MandatoryPlaceholder or OptionalPlaceholder |
| Semantic identifier | uav:semanticId | the AAS semanticId as an IRI |

The AAS vocabulary is the first row because it is most of the document. A Thing Description that

carried only the uav terms would describe a tree of empty nodes: it would name what to create and

not what any of it is. The uav terms carry what the metamodel has no field for — a NodeId, a

BrowseName, an ObjectType, a ReferenceType, a modelling rule — and nothing else.

The type binding is §5.2.1 of OPC UA — Web of Things Binding. It admits two forms: the

prefix-qualified model name alongside the node-class term in @type, and a

ua:HasTypeDefinition link whose href is the ObjectType's portable ExpandedNodeId. A document

may carry either or both; the published examples carry both because the name is readable and the

link is definitive. A converter shall reject a document in which both resolve and disagree.

Every sibling TD root carries Thing and the uav:object node-class annotation. A projected Object

is never placed in properties. If a TD publishes actual OPC UA Variable or Method affordances,

each property carries uav:variable and each action carries uav:method.

Every projected Object carries one Thing-level readallproperties form. The form is the Object's

protocol address, not a property affordance. Its href has the WoT Binding form

opc.tcp://<host>:<port>[/<resourcePath>]/?id=<ExpandedNodeId>. URI decoding the id parameter

produces the TD root's uav:id. A percent escape that is part of the ExpandedNodeId is itself

percent-encoded in the URL.

The parent is the source of each typed containment link, so the link has no anchor. A

uav:refName on that link is the target BrowseName local name; the generated examples carry it.

Where a target TD declares the BrowseName, a publication can instead omit the duplicate

uav:refName. An Operation role index is not a BrowseName and is never used as one.

The @type form is resolved in the local model context defined by the WoT Binding. The link form

uses nsu=http://opcfoundation.org/UA/I4AAS/v3/;i=<id>, not the session-local ns=<index> form: these ObjectType NodeIds

this model. @type may also carry semantic annotations; only a model name that resolves in the

local context is a type binding. A TD carries at most one such name and one

ua:HasTypeDefinition link because an OPC UA Node has one HasTypeDefinition.

The context contains the published W3C TD 1.1 context, relative references to the bundled AAS and

OPC UA WoT Binding contexts, and an inline id: "@id" alias plus the i4aas prefix bound to

http://opcfoundation.org/UA/I4AAS/v3/.

A worked Thing Description

The ordering-and-nesting fixture contains a SubmodelElementList whose order is relevant, holding

SubmodelElementCollection members. It projects as a bundle. The following abridged root TD contains no Object-valued property

affordance:

{

"@context": [

"https://www.w3.org/2022/wot/td/v1.1",

"../../aas.context.jsonld",

"../../tools/jsonld/vendor/opc-ua-wot-binding.context.jsonld",

{ "id": "@id", "i4aas": "http://opcfoundation.org/UA/I4AAS/v3/" }

],

"@type": ["Thing", "uav:object", "aas:Submodel", "i4aas:AASSubmodelType"],

"id": "https://fabrikam.com/ids/sm/ordering",

"title": "Ordering",

"uav:id": "nsu=https://example.com/aas/instances/;s=i4aas3:S:36:https://fabrikam.com/ids/sm/ordering",

"uav:browseName": "nsu=https://example.com/aas/instances/;Ordering",

"uav:hasComponent": [

"nsu=https://example.com/aas/instances/;s=i4aas3:E:36:22:https://fabrikam.com/ids/sm/orderingCollectionsInsideAList"

],

"aas:Submodel/submodelElements": [

{

"@id": "https://w3id.org/aas-jsonld/node/v1/aHR0cHM6Ly9mYWJyaWthbS5jb20vaWRzL3NtL29yZGVyaW5n/Q29sbGVjdGlvbnNJbnNpZGVBTGlzdA"

}

],

"forms": [{

"href": "opc.tcp://example.com:4840/?id=nsu=https://example.com/aas/instances/;s=i4aas3:S:36:https://fabrikam.com/ids/sm/ordering",

"contentType": "application/octet-stream",

"op": "readallproperties"

}],

"links": [

{

"rel": "self",

"href": "https://w3id.org/aas-jsonld/td/v1/aHR0cHM6Ly9mYWJyaWthbS5jb20vaWRzL3NtL29yZGVyaW5n",

"type": "application/td+json"

},

{

"rel": "ua:HasTypeDefinition",

"href": "nsu=http://opcfoundation.org/UA/I4AAS/v3/;i=1013"

},

{

"rel": "ua:HasComponent",

"href": "https://w3id.org/aas-jsonld/td/v1/aHR0cHM6Ly9mYWJyaWthbS5jb20vaWRzL3NtL29yZGVyaW5n/node/Q29sbGVjdGlvbnNJbnNpZGVBTGlzdA",

"type": "application/td+json",

"uav:refId": "i=47",

"uav:refName": "CollectionsInsideAList"

}

]

}

The list Object is the root of a sibling TD. It names its parent in both forms required for

connectivity and names its ordered child from the parent side:

{

"@type": ["Thing", "uav:object", "aas:SubmodelElementList", "i4aas:AASSubmodelElementListType"],

"id": "https://w3id.org/aas-jsonld/node/v1/aHR0cHM6Ly9mYWJyaWthbS5jb20vaWRzL3NtL29yZGVyaW5n/Q29sbGVjdGlvbnNJbnNpZGVBTGlzdA",

"uav:id": "nsu=https://example.com/aas/instances/;s=i4aas3:E:36:22:https://fabrikam.com/ids/sm/orderingCollectionsInsideAList",

"uav:componentOf": [

"nsu=https://example.com/aas/instances/;s=i4aas3:S:36:https://fabrikam.com/ids/sm/ordering"

],

"uav:hasComponent": [

"nsu=https://example.com/aas/instances/;s=i4aas3:E:36:25:https://fabrikam.com/ids/sm/orderingCollectionsInsideAList[0]"

],

"aas:SubmodelElementList/value": [

{

"@id": "https://w3id.org/aas-jsonld/node/v1/aHR0cHM6Ly9mYWJyaWthbS5jb20vaWRzL3NtL29yZGVyaW5n/Q29sbGVjdGlvbnNJbnNpZGVBTGlzdFswXQ"

}

],

"links": [

{

"rel": "ua:HasTypeDefinition",

"href": "nsu=http://opcfoundation.org/UA/I4AAS/v3/;i=1031"

},

{

"rel": "uav:componentOf",

"href": "https://w3id.org/aas-jsonld/td/v1/aHR0cHM6Ly9mYWJyaWthbS5jb20vaWRzL3NtL29yZGVyaW5n",

"type": "application/td+json"

},

{

"rel": "ua:HasOrderedComponent",

"href": "https://w3id.org/aas-jsonld/td/v1/aHR0cHM6Ly9mYWJyaWthbS5jb20vaWRzL3NtL29yZGVyaW5n/node/Q29sbGVjdGlvbnNJbnNpZGVBTGlzdFswXQ",

"type": "application/td+json",

"uav:refId": "i=49",

"uav:refName": "0"

}

]

}

The list member is another sibling TD. Its position is explicit and its uav:componentOf link

points to the list TD, not to either node's AAS RDF subject:

{

"@type": ["Thing", "uav:object", "aas:SubmodelElementCollection", "i4aas:AASSubmodelElementCollectionType"],

"id": "https://w3id.org/aas-jsonld/node/v1/aHR0cHM6Ly9mYWJyaWthbS5jb20vaWRzL3NtL29yZGVyaW5n/Q29sbGVjdGlvbnNJbnNpZGVBTGlzdFswXQ",

"uav:id": "nsu=https://example.com/aas/instances/;s=i4aas3:E:36:25:https://fabrikam.com/ids/sm/orderingCollectionsInsideAList[0]",

"uav:browseName": "nsu=https://example.com/aas/instances/;0",

"uav:componentOf": [

"nsu=https://example.com/aas/instances/;s=i4aas3:E:36:22:https://fabrikam.com/ids/sm/orderingCollectionsInsideAList"

],

"uav:index": 0,

"forms": [{

"href": "opc.tcp://example.com:4840/?id=nsu=https://example.com/aas/instances/;s=i4aas3:E:36:25:https://fabrikam.com/ids/sm/orderingCollectionsInsideAList%5B0%5D",

"contentType": "application/octet-stream",

"op": "readallproperties"

}],

"links": [

{

"rel": "ua:HasTypeDefinition",

"href": "nsu=http://opcfoundation.org/UA/I4AAS/v3/;i=1029"

},

{

"rel": "uav:componentOf",

"href": "https://w3id.org/aas-jsonld/td/v1/aHR0cHM6Ly9mYWJyaWthbS5jb20vaWRzL3NtL29yZGVyaW5n/node/Q29sbGVjdGlvbnNJbnNpZGVBTGlzdA",

"type": "application/td+json"

}

]

}

The bundle carries the Asset Administration Shell, not a description of one. Every TD root

carries its own AAS content in the AAS vocabulary — aas:Referable/idShort,

aas:Property/value, aas:HasSemantics/semanticId — and the containment properties of the

metamodel, aas:Submodel/submodelElements and aas:SubmodelElementList/value, reference the sibling

subjects. Discarding every triple outside the AAS namespace from the union leaves the graph of

clause 1 of the JSON-LD mapping. The independent final-byte validator performs that comparison in

both directions and checks every ordering occurrence.

The uav terms carry only what the AAS does not say. The AAS gives the tree and the order; the

uav terms give the NodeId, the BrowseName, the ObjectType, the ReferenceType and the modelling

rule, none of which the metamodel has a field for.

The NamespaceUri is a real one. uav:id is an ExpandedNodeId in the string form of OPC 10000-6

and the WoT drafts define no placeholder syntax for a namespace. https://example.com/aas/instances/

above is the Server's instance namespace, written out. A document that cannot know the target

namespace omits uav:id instead; see F.5.

Subject, TD document and protocol address are separate. For an ordinary absolute AAS identifier,

the TD id is that identifier unchanged. An identifier that cannot be used directly, or one inside

a namespace reserved for generated subjects, uses the collision-free encoded form of the JSON-LD

mapping. The self link identifies the sibling TD document, and the opc.tcp form identifies the

OPC UA Node. A converter does not derive one from another. It parses the ExpandedNodeId from the

form, resolves containment through sibling TD links and uses the AAS subject only for RDF edges.

A list member has no short name, so its BrowseName is its index — the rule clause 6.1.3 states.

Implementer notes

This subclause is informative.

- The reference type is where the order lives, and the index is what recovers it. The link rel states whether the collection is a sequence; uav:index states where each member sits. A converter that emits one without the other produces a list a serializer cannot restore, which clause 6.1.4 is about.

- uav:componentOf is directional and complete. Every non-root Object TD names the parent ExpandedNodeId and links to the parent sibling TD. The parent independently lists the child in uav:hasComponent and links to it with the exact ReferenceType. Reversing either direction or replacing either sibling TD target builds a different tree.

- A containment link does not rename its target. If the typed parent-to-child link carries uav:refName, it is the target TD's BrowseName local name. It may be omitted where the target TD supplies that name. An Operation role and array index belong to the AAS role array, NodeId path and uav:index; they are not substituted for the child's own BrowseName.

- A form address is not an RDF subject. Every form is an opc.tcp URL with an explicit endpoint and an id query parameter that decodes to the same TD root's uav:id. Characters significant to a URI query and percent escapes inside the ExpandedNodeId are encoded at the URI layer.

- Do not synthesise a type when the @type binding resolves. The types of this specification are loaded from Opc.Ua.I4AAS.NodeSet2.xml. A converter that generates its own type of the same name leaves the Server holding two type hierarchies, and a Client written against this specification recognises neither.

- Mandatory members of the resolved type are populated, not duplicated. AASPropertyType declares ValueType as Mandatory. A document that also declares ValueType populates that declaration; a converter that adds a sibling produces a node carrying the member twice.

- NodeIds are derived, not allocated. Clause 6.1.3 fixes the node-kind-discriminated, escaped, length-prefixed encoding of the AAS identifier and idShortPath, so two documents describing the same submodel produce the same NodeIds. A converter that generates NodeIds from the browse path instead — which WoT Connectivity §6.5.4 permits where a document supplies none — produces a subtree that no longer matches this specification. Supply uav:id on every node.

- A document that cannot know the instance namespace omits uav:id. An ExpandedNodeId names a namespace, and the namespace a Server materializes instances into is the Server's to choose. An author writing for a known Server writes it out. An author writing a portable document omits uav:id and accepts browse-path NodeIds, or publishes the document per Server. There is no placeholder: nothing in the WoT drafts defines one, and a document carrying an invented one produces ExpandedNodeIds that name a namespace no Server has.

Type-binding conformance

The type binding is part of the published WoT Binding. The examples carry both forms of §5.2.1 and

the validation runs honour them independently:

Table 141 – Type-binding conformance

| Form honoured | Required outcome |
| --- | --- |
| compact model name in @type only | every projected Object has the clause 6.2 ObjectType |
| ua:HasTypeDefinition link only | the same nodes have the same ObjectTypes |
| both forms | both resolve to the same Node; disagreement invalidates the document |
| neither form | the projection falls back to BaseObjectType unless a Thing Model supplies a type |

uav:congruentType remains reconciliation metadata and shall not substitute for either form.

The compact name is resolved through the local model context of WoT Binding §5.1.5. The link is

resolved by exact ExpandedNodeId. A converter shall not manufacture a second ObjectType with the

same name: the target is the ObjectType already loaded from Opc.Ua.I4AAS.NodeSet2.xml.

<a id="annex-g"></a>

(informative)

Correspondence to the AAS API of IDTA-01002 Part 2

This annex is informative.

IDTA-01002 Part 2 defines an HTTP API over the same metamodel this specification maps. The two are

different bindings of one model, not two models: a Client that has a node reaches the same content a

Client that has a URL reaches. This annex says which OPC UA Service answers each operation, so that

a Server implementing both does not implement them twice.

The general rule is that a resource of the AAS API is a node of this specification, and the

operations on it are the OPC UA Services on that node. Read replaces GET, Write replaces PATCH with

the $value modifier, and Browse replaces the collection endpoints.

Table 142 – Correspondence to the AAS API of IDTA-01002 Part 2

| AAS API operation | OPC UA equivalent |
| --- | --- |
| GetAllAssetAdministrationShells | Browse the registry root, clause 6.5.2 |
| GetAssetAdministrationShellById | Browse to the shell group whose identifier matches, clause 6.5.3 |
| GetAllSubmodels, GetAllSubmodelReferences | Browse the shell's submodel references |
| GetSubmodelById | Read the AASSubmodelType subtree, or call GetSubmodel for the document form |
| GetSubmodelById-ValueOnly ($value) | Read the value Variables of the subtree; clause 6.1.2 assigns each its xsd type |
| GetSubmodelById-Metadata ($metadata) | Read the subtree with the value Variables excluded |
| GetSubmodelById-Reference ($reference) | Read the node's AASReferenceDataType form |
| GetSubmodelElementByPath | Read the node whose NodeId clause 6.1.3 derives from that idShortPath |
| PatchSubmodelElementValueByPath | Write that node's Value |
| GetFileByPath, PutFileByPath | Open, Read/Write, Close on the AASFileType node |
| InvokeOperation | Call Invoke on the AASOperationType node, clause 6.2.5 |
| InvokeOperationAsync, GetOperationAsyncResult | no counterpart; see clause 6.2.5 |
| SearchAllAssetAdministrationShellIdsByAssetLink | Call LookupShellsByAssetLink, clause 6.5.5 |
| GenerateSerializationByIds | The environment documents of clause 6.5.10 |
| GetSelfDescription | Read Server.ServerCapabilities.ServerProfileArray |

Three differences are structural rather than incidental, and an implementation should not try to

paper over them.

The path is a NodeId, not a string. The AAS API addresses an element by a base64url-encoded

identifier and an idShortPath. Clause 6.1.3 derives its escaped, length-prefixed String NodeId from

the same two parts, so the mapping is mechanical, but the encoding is not the same and a gateway

converts rather than passes through.

The level and extent parameters have no counterpart. level=core|deep and

extent=withBlobValue|withoutBlobValue shape one response document. OPC UA shapes a response by

what the caller asked to Read and by the disclosure tier of clause 6.5.7, which is a different

mechanism with a different granularity.

Paging is a continuation point. The AAS API returns a cursor; OPC UA returns a

ContinuationPoint from Browse. Both are opaque to the caller and neither is convertible into the

other.

A Server that implements both bindings should publish the IDTA profile identifier it satisfies —

for example https://admin-shell.io/aas/API/3/0/SubmodelServiceSpecification/SSP-001 — in

ServerProfileArray alongside the OPC UA profile URIs, so that one array answers the conformance

question for either kind of Client.

_____________

—————————