| Logo of OPC Foundation, only if this is a joint work. Both logos shall be side by side.. |  | Logo of other organization. Both logos shall be side by side.. | OPC UA Companion Specification |
| --- | --- | --- | --- |
| OPC nnnnn-m |  |  |  |
| OPC UA for Robot Intent Part 1: Robot Intent Draft 0.1.0 YYYY-MM-DD Edit the advanced properties to complete the title page. Boxes are used to provide context or guidelines on expected content. Any words or phrases written in red font colour shall be replaced by actual values as appropriate for the companion specification. |  |  |  |

| Specification Type: | Industry Standard Specification | Comments: |  |
| --- | --- | --- | --- |
| Doc-Number | OPC nnnnn-m |  |  |
| Title: | OPC UA for Robot IntentPart 1 :Robot Intent | Date: | YYYY-MM-DD |
| Version: | Draft 0.1.0 | Software: | MS-Word |
|  |  | Source: | OPC 20020 - UA Companion Specification Template v1.01.18.docx |
| Author: | <organization> | Status: | Draft |

Template Revisions

| Version | Date | Description |
| --- | --- | --- |
| 1.01.09 | Mar 09, 2020 | M-4665: Link to NodeSet indicates that also ERRATA, Amendments or Revisions are applied. Fixed various styles. |
| 1.01.10 | Apr 29, 2020 | Minor update for the Value attribute (M-5314). |
| 1.01.11 | July 09, 2020 | M-5595: Added better wording and recommendations for NamespaceMetadata as suggested. M-5520: Fixed quotation marks. M-5781: Title "Additional Variable Attributes" misleading. M-5673: Added HasInterface Reference to figure. M-5468: Use of Namespace Index also in Method signatures. M-5789: Clarify use of NamespaceIndex for Profiles or ConformanceUnits in other specifications. |
| 1.01.12 | Nov 16, 2020 | M-5904: BrowseName convention for placeholders. M-6184: The "additional attributes" table shall not be restricted to Value and Description. M-6183: Fix heading names for columns with a browse path. M-6112: Add an example state machine. M-6185: Add new table types to conventions. |
| 1.01.13 | Feb 17, 2021 | M-6302: Updated format for NamespaceMetadata table to make it validatable. M-6317: Added guideline for uncaptioned embedded figures. |
| 1.01.14 | Nov 15, 2021 | M-6264: GeneratesEvent example added. M-7147: Added abbreviated terms used in template. M-6265: No HasSubtype References. M-7265: Added max size for ConformanceUnit description. M-6612: Added definition of capability ID (Annex A.2). M-6150: Extended tables with Conformance Units row. |
| 1.01.15 | Oct 01, 2022 | M-7528: Namespace index also for the “Subtype of” rows. M-8025: Added missing BrowsePath tables for subcomponents. M-6776: Changed the examples for Structure and Union to start with an uppercase character. M-7341: Added extended definition for structures. |
| 1.01.16 | Mar 14, 2023 | M-8638: Updated URL format for schema files (Annex A). Updated URL format for normative references to documents (Clause 2). |
| 1.01.17 | Nov 17, 2023 | M-9196: Conformance Units and Profiles should be in profile database. M-8765: Added URL syntax for individual files. M-9170: Casing of UANodeSet wrong. M-9182: Added reference to IEC specifications. |
| 1.01.18 | Dec 03, 2024 | M-9275: Missing value rank 0. M-9270: Added guidelines to make figures smaller. M-8648: Well-Known BrowseNames. M-9723: Hyperlinks to online reference. |
| 1.01.19 | Jan 21, 2025 | M-9833: Added description on issue reporting to the “Agreement of Use”. |

CONTENTS

Right-click and choose Update Field.

Figures

Right-click and choose Update Field.

Tables

Right-click and choose Update Field.

OPC FOUNDATION, VDMA and the Alliance for OpenUSD

____________

This work (OPC UA Companion Specification Template, by OPC Foundation), identified by OPC Foundation, is free of known copyright restrictions.

<Remove the above Public Domain notice and add a copyright date in the following Agreement of use for the completed specification. If possible (but not legally required), provide attribution to the original author in the following way:

“This specification is a derivative work of the public domain document OPC UA Companion Specification Template, by OPC Foundation”

>

AGREEMENT OF USE

<For joint work, this is the agreement accepted by OPC Foundation and used by most if not all joint companion specifications.

Changes to this agreement will require OPC Foundation review. For non-joint work replace this section with appropriate agreement of use and copyright verbiage.>

COPYRIGHT RESTRICTIONS

- This document is provided "as is" by the OPC Foundation and the VDMA and the Alliance for OpenUSD.

- Right of use for this specification is restricted to this specification and does not grant rights of use for referred documents.

- Right of use for this specification will be granted without cost.

- This document may be distributed through computer systems, printed or copied as long as the content remains unchanged and the document is not modified.

- OPC Foundation and VDMA and the Alliance for OpenUSD do not guarantee usability for any purpose and shall not be made liable for any case using the content of this document.

- The user of the document agrees to indemnify OPC Foundation and VDMA and the Alliance for OpenUSD and their officers, directors and agents harmless from all demands, claims, actions, losses, damages (including damages from personal injuries), costs and expenses (including attorneys' fees) which are in any way related to activities associated with its use of content from this specification.

- The document shall not be used in conjunction with company advertising, shall not be sold or licensed to any party.

- The intellectual property and copyright is solely owned by the OPC Foundation and the VDMA and the Alliance for OpenUSD.

PATENTS

The attention of adopters is directed to the possibility that compliance with or adoption of OPC or VDMA and the Alliance for OpenUSD specifications may require use of an invention covered by patent rights. OPC Foundation or VDMA and the Alliance for OpenUSD shall not be responsible for identifying patents for which a license may be required by any OPC or VDMA and the Alliance for OpenUSD specification, or for conducting legal inquiries into the legal validity or scope of those patents that are brought to its attention. OPC or VDMA and the Alliance for OpenUSD specifications are prospective and advisory only. Prospective users are responsible for protecting themselves against liability for infringement of patents.

WARRANTY AND LIABILITY DISCLAIMERS

WHILE THIS PUBLICATION IS BELIEVED TO BE ACCURATE, IT IS PROVIDED "AS IS" AND MAY CONTAIN ERRORS OR MISPRINTS. THE OPC FOUDATION NOR VDMA and the Alliance for OpenUSD MAKES NO WARRANTY OF ANY KIND, EXPRESSED OR IMPLIED, WITH REGARD TO THIS PUBLICATION, INCLUDING BUT NOT LIMITED TO ANY WARRANTY OF TITLE OR OWNERSHIP, IMPLIED WARRANTY OF MERCHANTABILITY OR WARRANTY OF FITNESS FOR A PARTICULAR PURPOSE OR USE. IN NO EVENT SHALL THE OPC FOUNDATION NOR VDMA and the Alliance for OpenUSD BE LIABLE FOR ERRORS CONTAINED HEREIN OR FOR DIRECT, INDIRECT, INCIDENTAL, SPECIAL, CONSEQUENTIAL, RELIANCE OR COVER DAMAGES, INCLUDING LOSS OF PROFITS, REVENUE, DATA OR USE, INCURRED BY ANY USER OR ANY THIRD PARTY IN CONNECTION WITH THE FURNISHING, PERFORMANCE, OR USE OF THIS MATERIAL, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

The entire risk as to the quality and performance of software developed using this specification is borne by you.

RESTRICTED RIGHTS LEGEND

This Specification is provided with Restricted Rights. Use, duplication, or disclosure by the U.S. government is subject to restrictions as set forth in (a) this Agreement pursuant to DFARs 227.7202-3(a); (b) subparagraph (c)(1)(i) of the Rights in Technical Data and Computer Software clause at DFARs 252.227-7013; or (c) the Commercial Computer Software Restricted Rights clause at FAR 52.227-19 subdivision (c)(1) and (2), as applicable. Contractor / manufacturer are the OPC Foundation, 16101 N. 82nd Street, Suite 3B, Scottsdale, AZ, 85260-1830

COMPLIANCE

The combination of VDMA and the Alliance for OpenUSD and OPC Foundation shall at all times be the sole entities that may authorize developers, suppliers and sellers of hardware and software to use certification marks, trademarks, or other special designations to indicate compliance with these materials as specified within this document. Products developed using this specification may claim compliance or conformance with this specification if and only if the software satisfactorily meets the certification requirements set by VDMA and the Alliance for OpenUSD or the OPC Foundation. Products that do not meet these requirements may claim only that the product was based on this specification and must not claim compliance or conformance with this specification.

Trademarks

Most computer and software brand names have trademarks or registered trademarks. The individual trademarks have not been listed here.

GENERAL PROVISIONS

Should any provision of this Agreement be held to be void, invalid, unenforceable, or illegal by a court, the validity and enforceability of the other provisions shall not be affected thereby.

This Agreement shall be governed by and construed under the laws of Germany.

This Agreement embodies the entire understanding between the parties with respect to, and supersedes any prior understanding or agreement (oral or written) relating to, this specification.

ISSUE REPORTING

If an error or problem is found in this specification, the UaNodeSet, or any associated supplementary files, it should be reported as an issue.

The reporting process can be found here: https://opcfoundation.org/resources/issue-tracking/

The Link to the issue tracking project for this document is here:

https://mantis.opcfoundation.org/set_project.php?project_id=<nnn>&make_default=no

<nnn> is the project_id in Mantis which is created for any document when requested by the working group. Example: https://mantis.opcfoundation.org/set_project.php?project_id=142&make_default=no is the Link for OPC 40001-* (Machinery).

If you have no Mantis Project or do not know the project_id, please send a request to TechnicalDirector@opcfoundation.org.

Release 0.1.0 highlights

<Revision Highlights specify the interesting changes to the previous revision. Such a "Revision Highlights" clause will not exist in the first revision.>

The table below follows the design and process used for the OPC UA specification. All "interesting" changes need to be reported in the problem tracking tool used for the specification. The OPC Foundation uses Mantis and provides it for companion specs as well. If a different problem tracking tool is used, "Mantis" needs to be replaced.

<The two rows in the following table are just examples. Note that the "Mantis IDs" are real hyperlinks – when clicked the reader opens the Mantis page.>

Scope shall specify the type of change. The three options are Errata (corrected something that was wrong), Clarification (make something clearer that was not wrong), and Feature (added new functionality).

The following table includes the Mantis issues resolved with this revision.

| Mantis ID | Scope | Summary | Resolution |
| --- | --- | --- | --- |
| 3165 | Errata | Wrong variable data types in Program example. | Fixed datatypes and replaced Table A.14 by description of the variables. |
| 3521 | Clarification | Modelling rules for states and transitions. | Removed the modelling rules from state and transition objects as they are never created (multiple tables). |

OPC UA FOR ROBOT INTENT

Part 1: Task-level commanding, missions and safety awareness

# Scope

This specification defines an OPC UA information model that lets a Server describe:

- what a robot can be asked to do — as a declared, machine-readable set of intents rather than a convention a client must know in advance;

- how to ask it — one submission per intent, or an ordered mission of them;

- what happens next — a lifecycle a client can observe from admission through queueing, execution and cancellation to a terminal result;

- what the request refers to — the frames, tools, locations, axes, signals and programs that give a pose or a place its meaning;

- when the robot will refuse — the operational modes, the command authority, and the boundary between this interface and the safety system that is never crossed.

## Motivation

OPC 40010-1 describes robot topology in detail — the motion device system, its axes, its power trains, its controller, its safety states. It defines no motion verbs at all. Its entire actuation surface is Start, Stop and the loading of a named program over two state machines. A conformant client can discover everything about a robot's construction and cannot ask it to move anywhere.

The consequence is that every robot integration above the level of "run program 7" is bespoke. Two Servers can be fully conformant to OPC 40010-1 and still share no way to express move the tool to this pose. The verbs exist — every vendor has joint, linear and circular moves, a speed, an acceleration, a blend, a tool frame and a work frame — but they exist only in vendor languages that do not interoperate.

This specification supplies the verbs, and nothing else, so that the two compose rather than compete.

There is a second gap, and it is the harder one. A motion takes seconds; a pick takes a minute. An OPC UA Call cannot stay open that long — Session timeouts, SecureChannel re-keying and transport timeouts all bound it, and OPC 10000-4 is explicit that if the Session ends the result is discarded "independent of the task actually performed at the Server". A synchronous method that commands a robot is therefore not merely inelegant; it loses the outcome of work that has already physically happened. Clause 6 addresses this, and it is the reason the model is shaped the way it is.

## What this specification does not do

Two of these are permanent boundaries. Neither is a deferral.

- It is not a real-time control channel. Trajectory execution is in scope — a client hands over a whole time-parameterised path and the robot's own motion kernel runs it (6.14.8) — and where a high-rate channel is genuinely needed, this specification brokers it (6.14.9). What it does not do is carry the samples: it defines no transport, closes no control loop, and runs at OPC UA rates. 6.3 draws that line as a normative limit rather than a caution.

- It is not a safety function, and it is not safety-rated. This is not a scoping preference; it is what the technology permits. OPC 10000-15 carries cyclic safety data from a provider to a consumer, and a client calling a Method has no way to supply safety-rated arguments — so no Method in this or any other companion specification can be a safety function. What is in scope is awareness: the model reports what the safety system is enforcing, and a Server shall refuse work that would exceed it. Clause 10 states both halves precisely.

Neither statement is about the importance of the omitted capability. The first is a division of labour with the layer beneath; the second is the boundary of what an OPC UA Method can be.

## Capabilities and versioning

Release 0.1.0 covers the intent vocabulary — joint, linear and circular moves, trajectories, Cartesian paths, force-controlled moves, grasping, picking and placing, tool change, output, program call, waiting, and six application processes — together with the execution lifecycle, queueing and blending, cancellation, missions with a committed base, a revisable horizon and a step graph, command authority, safety awareness, the robot's kinematic description, real-time channel brokerage, and capability declaration.

The NodeSet declares exactly one RequiredModel — the base OPC UA namespace — so a Server can adopt it without pulling in any companion model. A Server implements the facets it can honour and declares the rest false; only RI-Base is mandatory (11.2).

NOTE   PROVISIONAL WORKING DRAFT — not normative, not endorsed by the OPC Foundation. The document number, the Mantis project identifier and every NodeId in this document are placeholders; the OPC Foundation assigns the real values.

OPC Foundation

OPC is the interoperability standard for the secure and reliable exchange of data and information in the industrial automation space and in other industries. It is platform independent and ensures the seamless flow of information among devices from multiple vendors. The OPC Foundation is responsible for the development and maintenance of this standard.

OPC UA is a platform independent service-oriented architecture that integrates all the functionality of the individual OPC Classic specifications into one extensible framework. This multi-layered approach accomplishes the original design specification goals of:

- Platform independence: from an embedded microcontroller to cloud-based infrastructure

- Secure: encryption, authentication, authorisation, and auditing

- Extensible: ability to add new features including transports without affecting existing applications

- Comprehensive information modelling capabilities: for defining any model from simple to complex

VDMA and the Alliance for OpenUSD

# Normative references

The following referenced documents are indispensable for the application of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments and errata) applies.

NOTE   The OPC UA core specifications are regularly published as IEC 62541.

OPC 10000-1, OPC Unified Architecture - Part 1: Overview and Concepts

http://www.opcfoundation.org/documents/10000-1/

OPC 10000-2, OPC Unified Architecture - Part 2: Security Model

http://www.opcfoundation.org/documents/10000-2/

OPC 10000-3, OPC Unified Architecture - Part 3: Address Space Model

http://www.opcfoundation.org/documents/10000-3/

OPC 10000-4, OPC Unified Architecture - Part 4: Services

http://www.opcfoundation.org/documents/10000-4/

OPC 10000-5, OPC Unified Architecture - Part 5: Information Model

http://www.opcfoundation.org/documents/10000-5/

OPC 10000-6, OPC Unified Architecture - Part 6: Mappings

http://www.opcfoundation.org/documents/10000-6/

OPC 10000-7, OPC Unified Architecture - Part 7: Profiles

http://www.opcfoundation.org/documents/10000-7/

OPC 10000-10, OPC Unified Architecture - Part 10: Programs

http://www.opcfoundation.org/documents/10000-10/

OPC 10000-12, OPC Unified Architecture - Part 12: Discovery and Global Services

http://www.opcfoundation.org/documents/10000-12/

ISO 8373:2021, Robotics - Vocabulary

https://www.iso.org/standard/75539.html

ISO 9787:2013, Robots and robotic devices - Coordinate systems and motion nomenclatures

https://www.iso.org/standard/59090.html

ISO 10218-1, Robotics - Safety requirements - Part 1: Industrial robots

https://www.iso.org/standard/73933.html

ISO 10218-2, Robotics - Safety requirements - Part 2: Industrial robot applications and robot cells

https://www.iso.org/standard/73934.html

IEC 60204-1, Safety of machinery - Electrical equipment of machines - Part 1: General requirements

https://webstore.iec.ch/publication/26037

IEC 61800-5-2, Adjustable speed electrical power drive systems - Part 5-2: Safety requirements - Functional

https://webstore.iec.ch/publication/22850

IEC 61131-3, Programmable controllers - Part 3: Programming languages

https://webstore.iec.ch/publication/4552

# Terms, abbreviated terms and conventions

## Overview

## Robot Intent terms

It is assumed that basic concepts of OPC UA information modelling and of Robot Intent are understood in this document. This document uses these concepts to describe the Robot Intent Information Model. For the purposes of this document, the terms and definitions given in OPC 10000-1, OPC 10000-3, OPC 10000-4, OPC 10000-5 and OPC 10000-7, as well as the following, apply.

OPC UA terms and terms defined in this document are italicized in the document.

## Abbreviated terms

Intent

A single task-level request — one motion, one grasp, one pick. Modelled as a subtype of IntentDataType. It is what a client submits and what a mission step holds.

Intent operation

One submitted intent, tracked to completion. Modelled as IntentOperationType, a Part 10 program instance created per submission.

Mission

An ordered sequence of intents submitted and tracked as a unit. Modelled as MissionType.

Base (of a mission)

The prefix of released steps. Committed, possibly already executing, and immutable (6.15.2).

Horizon (of a mission)

The suffix of unreleased steps. Provisional, and revisable by UpdateMission (§7.2).

Pose

A rigid-body placement — three of position and three of orientation — expressed relative to a named frame. Carried by Pose3DDataType, whose orientation is a unit quaternion ordered (x, y, z, w) per 6.7.

Frame

A named right-handed Cartesian coordinate system a pose is expressed relative to. Frames form a tree through HasFrameParent. Modelled as CoordinateFrameType.

Tool centre point (TCP)

The origin of a tool frame; the point a motion intent drives to its target. OPC 40010-1 defines no such concept, so ToolType.TcpFrame supplies one.

Blending

Rounding the corner between two motions rather than stopping on the first one's target. Requested by BlendDataType, and by the blending values of BufferModeEnum.

Buffer mode

How a newly submitted intent relates to the one already executing — abort it, queue behind it, or blend into it. BufferModeEnum.

Blocking mode

Whether an intent tolerates motion and other intents alongside it. BlockingModeEnum.

Command authority

The exclusive right, held by at most one Session, to submit intents to one controller (clause 8). It is an arbitration mechanism between clients and not the single point of control that ISO 10218-2 requires.

Terminal state

An ExecutionStateEnum value from which execution does not resume: Succeeded, Failed, Cancelled, or Retriable.

Conformance unit

The smallest individually testable requirement of this specification. Grouped into facets rather than claimed one by one.

Facet

A named, individually claimable set of conformance units — a building block, not a complete claim. Clause 12.2 defines them.

Profile

A named set of facets describing one plausible robot Server, claimed by publishing its URI in Server/ServerCapabilities/ServerProfileArray. Clause 12.3 defines them.

## Conventions used in this document

Following are basic conventions that shall be followed for all formal definitions used.

### Conventions for Node descriptions

#### Node definitions

Node definitions are specified using tables (see Table 2).

Attributes are defined by providing the Attribute name and a value, or a description of the value.

References are defined by providing the ReferenceType name, the BrowseName of the TargetNode and its NodeClass.

If the TargetNode is a component of the Node being defined in the table the Attributes of the composed Node are defined in the same row of the table.

The DataType is only specified for Variables; “[<number>]” indicates a single-dimensional array, for multi-dimensional arrays the expression is repeated for each dimension (e.g. [2][3] for a two-dimensional array). For all arrays the ArrayDimensions is set as identified by <number> values. If no <number> is set, the corresponding dimension is set to 0, indicating an unknown size. If no number is provided at all the ArrayDimensions can be omitted. If no brackets are provided, it identifies a scalar DataType and the ValueRank is set to the corresponding value (see OPC 10000-3). In addition, ArrayDimensions is set to null or is omitted. If it can be Any or ScalarOrOneDimension, the value is put into “{<value>}”, so either “{Any}” or “{ScalarOrOneDimension}” and the ValueRank is set to the corresponding value (see OPC 10000-3) and the ArrayDimensions is set to null or is omitted. Examples are given in Table 1.

Table 1 – Examples of DataTypes

| Notation | DataType | ValueRank | ArrayDimensions | Description |
| --- | --- | --- | --- | --- |
| 0:Int32 | 0:Int32 | -1 | omitted or null | A scalar Int32. |
| 0:Int32{OneOrMoreDimensions} | 0:Int32 | 0 | omitted or null | The Int32 value is an array with one or more dimensions. |
| 0:Int32[] | 0:Int32 | 1 | omitted or {0} | Single-dimensional array of Int32 with an unknown size. |
| 0:Int32[][] | 0:Int32 | 2 | omitted or {0,0} | Two-dimensional array of Int32 with unknown sizes for both dimensions. |
| 0:Int32[3][] | 0:Int32 | 2 | {3,0} | Two-dimensional array of Int32 with a size of 3 for the first dimension and an unknown size for the second dimension. |
| 0:Int32[5][3] | 0:Int32 | 2 | {5,3} | Two-dimensional array of Int32 with a size of 5 for the first dimension and a size of 3 for the second dimension. |
| 0:Int32{Any} | 0:Int32 | -2 | omitted or null | An Int32 where it is unknown if it is scalar or array with any number of dimensions. |
| 0:Int32{ScalarOrOneDimension} | 0:Int32 | -3 | omitted or null | An Int32 where it is either a single-dimensional array or a scalar. |

The TypeDefinition is specified for Objects and Variables.

The TypeDefinition column specifies a symbolic name for a NodeId, i.e., the specified Node points with a HasTypeDefinition Reference to the corresponding Node.

The ModellingRule of the referenced component is provided by specifying the symbolic name of the rule in the ModellingRule column. In the AddressSpace, the Node shall use a HasModellingRule Reference to point to the corresponding ModellingRule Object.

If the NodeId of a DataType is provided, the symbolic name of the Node representing the DataType shall be used.

Note that if a symbolic name of a different namespace is used, it is prefixed by the NamespaceIndex (see 3.4.2.2).

Nodes of all other NodeClasses cannot be defined in the same table; therefore, only the used ReferenceType, their NodeClass and their BrowseName are specified. A reference to another part of this document points to their definition. Table 2 illustrates the table. If no components are provided, the DataType, TypeDefinition and Other columns can be omitted and only a Comment column is introduced to point to the Node definition.

Each Type Node or well-known Instance Node defined shall have one or more ConformanceUnits defined in 0 that require the Node to be in the AddressSpace.

The relations between Nodes and ConformanceUnits are defined at the end of the tables defining Nodes, one row per ConformanceUnit. The ConformanceUnits are reflected in the Category element for the Node definition in the UANodeSet (see OPC 10000-6).

The list of ConformanceUnits in the UANodeSet allows Servers to optimize resource consumption by using a list of supported ConformanceUnits to select a subset of the Nodes in an Information Model.

When a Node is selected in this way, all dependencies implied by the References are also selected.

Dependencies exist if the Node is the source of HasTypeDefinition, HasInterface, HasAddIn or any HierarchicalReference. Dependencies also exist if the Node is the target of a HasSubtype Reference. For Variables and VariableTypes, the value of the DataType Attribute is a dependency. For DataType Nodes, any DataTypes referenced in the DataTypeDefinition Attribute are also dependencies.

For additional details see OPC 10000-5.

Table 2 – Type Definition Table

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Attribute name | Attribute value. If it is an optional Attribute that is not set “--” is used. |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| ReferenceType name | NodeClass of the target Node. | BrowseName of the target Node. | DataType of the referenced Node, only applicable for Variables. | TypeDefinition of the referenced Node, only applicable for Variables and Objects. | Additional characteristics of the TargetNode such as the ModellingRule or AccessLevel. |
| NOTE Notes referencing footnotes of the table content. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| Name of ConformanceUnit, one row per ConformanceUnit |  |  |  |  |  |

Components of Nodes can be complex that is containing components by themselves. The TypeDefinition, NodeClass and DataType can be derived from the type definitions, and the symbolic name can be created as defined in 3.4.3.1. Therefore, those containing components are not explicitly specified; they are implicitly specified by the type definitions.

The Other column defines additional characteristics of the Node. Examples of characteristics that can appear in this column are show in Table 3.

Table 3 – Examples of Other characteristics

| Name | Short Name | Description |
| --- | --- | --- |
| 0:Mandatory | M | The Node has the Mandatory ModellingRule. |
| 0:Optional | O | The Node has the Optional ModellingRule. |
| 0:MandatoryPlaceholder | MP | The Node has the MandatoryPlaceholder ModellingRule. |
| 0:OptionalPlaceholder | OP | The Node has the OptionalPlaceholder ModellingRule. |
| ReadOnly | RO | The Node AccessLevel has the CurrentRead bit set but not the CurrentWrite bit. |
| ReadWrite | RW | The Node AccessLevel has the CurrentRead and CurrentWrite bits set. |
| WriteOnly | WO | The Node AccessLevel has the CurrentWrite bit set but not the CurrentRead bit. |

If multiple characteristics are defined, they are separated by commas. The name or the short name can be used.

#### Additional References

To provide information about additional References, the format as shown in Table 4 is used.

Table 4 – <some>Type additional References

| SourceBrowsePath | Reference Type | Is Forward | TargetBrowsePath |
| --- | --- | --- | --- |
| SourceBrowsePath is always relative to the TypeDefinition. Multiple elements are defined as separate rows of a nested table. | ReferenceType name | True = forward Reference. | TargetBrowsePath points to another Node, which can be a well-known instance or a TypeDefinition. You can use BrowsePaths here as well, which is either relative to the TypeDefinition or absolute. If absolute, the first entry needs to refer to a type or well-known instance, uniquely identified within a namespace by the BrowseName. |

References can be to any other Node.

#### Additional sub-components

To provide information about sub-components, the format as shown in Table 5 is used.

Table 5 – <some>Type additional subcomponents

| BrowsePath | References | NodeClass | BrowseName | DataType | TypeDefinition | Others |
| --- | --- | --- | --- | --- | --- | --- |
| BrowsePath is always relative to the TypeDefinition. Multiple elements are defined as separate rows of a nested table | NOTE Same as for Table 2 |  |  |  |  |  |

#### Additional Attribute values

The type definition table provides columns to specify the values for required Node Attributes for InstanceDeclarations. To provide information about additional Attributes, the format as shown in Table 6 is used.

Table 6 – <some>Type Attribute values for child Nodes

| BrowsePath | <Attribute name> Attribute |
| --- | --- |
| BrowsePath is always relative to the TypeDefinition. Multiple elements are defined as separate rows of a nested table | The values of attributes are converted to text by adapting the reversible JSON encoding rules defined in OPC 10000-6. If the JSON encoding of a value is a JSON string or a JSON number then that value is entered in the value field. Double quotes are not included. If the DataType includes a NamespaceIndex (QualifiedNames, NodeIds or ExpandedNodeIds) then the notation used for BrowseNames is used. If the value is an Enumeration the name of the enumeration value is entered. If the value is a Structure then a sequence of name and value pairs is entered. Each pair is followed by a newline. The name is followed by a colon. The names are the names of the fields in the DataTypeDefinition. If the value is an array of non-structures then a sequence of values is entered where each value is followed by a newline. If the value is an array of Structures or a Structure with fields that are arrays or with nested Structures then the complete JSON array or JSON object is entered. |

There can be multiple columns to define more than one Attribute.

### NodeIds and BrowseNames

#### NodeIds

The NodeIds of all Nodes described in this standard are only symbolic names. Annex A defines the actual NodeIds.

The symbolic name of each Node defined in this document is its BrowseName, or, when it is part of another Node, the BrowseName of the other Node, a “.”, and the BrowseName of itself. In this case “part of” means that the whole has a HasProperty or HasComponent Reference to its part. Since all Nodes not being part of another Node have a unique name in this document, the symbolic name is unique.

The NamespaceUri for all NodeIds defined in this document is defined in Annex A. The NamespaceIndex for this NamespaceUri is vendor-specific and depends on the position of the NamespaceUri in the server namespace table.

Note that this document not only defines concrete Nodes, but also requires that some Nodes shall be generated, for example one for each Session running on the Server. The NodeIds of those Nodes are Server-specific, including the namespace. But the NamespaceIndex of those Nodes cannot be the NamespaceIndex used for the Nodes defined in this document, because they are not defined by this document but generated by the Server.

#### BrowseNames

The text part of the BrowseNames for all Nodes defined in this document is specified in the tables defining the Nodes. The NamespaceUri for all BrowseNames defined in this document is defined in 16.2.

For InstanceDeclarations of NodeClass Object and Variable that are placeholders (OptionalPlaceholder and MandatoryPlaceholder ModellingRule), the BrowseName and the DisplayName are enclosed in angle brackets (<>) as recommended in OPC 10000-3.

If a BrowseName is not defined by this document, a namespace index prefix is added to the BrowseName (e.g., prefix '0' leading to ‘0:EngineeringUnits’ or prefix '2' leading to ‘2:DeviceRevision’). This is typically necessary if a Property of another specification is overwritten or used in the OPC UA types defined in this document. Table 40 provides a list of namespaces and their indexes as used in this document.

### Common Attributes

#### General

The Attributes of Nodes, their DataTypes and descriptions are defined in OPC 10000-3. Attributes not marked as optional are mandatory and shall be provided by a Server. The following tables define if the Attribute value is defined by this document or if it is server-specific.

For all Nodes specified in this document, the Attributes named in Table 7 shall be set as specified in the table.

Table 7 – Common Node Attributes

| Attribute | Value |
| --- | --- |
| DisplayName | The DisplayName is a LocalizedText. Each Server shall provide the DisplayName identical to the BrowseName of the Node for the LocaleId “en” unless specified differently in the specification. Whether the Server provides translated names for other LocaleIds is server-specific. |
| Description | Optionally a server-specific description is provided. |
| NodeClass | Shall reflect the NodeClass of the Node. |
| NodeId | The NodeId is described by BrowseNames as defined in 3.4.2.1. |
| WriteMask | Optionally the WriteMask Attribute can be provided. If the WriteMask Attribute is provided, it shall set all non-server-specific Attributes to not writable. For example, the Description Attribute can be set to writable since a Server can provide a server-specific description for the Node. The NodeId shall not be writable, because it is defined for each Node in this document. |
| UserWriteMask | Optionally the UserWriteMask Attribute can be provided. The same rules as for the WriteMask Attribute apply. |
| RolePermissions | Optionally server-specific role permissions can be provided. |
| UserRolePermissions | Optionally the role permissions of the current Session can be provided. The value is server-specific and depends on the RolePermissions Attribute (if provided) and the current Session. |
| AccessRestrictions | Optionally server-specific access restrictions can be provided. |

#### Objects

For all Objects specified in this document, the Attributes named in Table 8 shall be set as specified in the Table 8. The definitions for the Attributes can be found in OPC 10000-3.

Table 8 – Common Object Attributes

| Attribute | Value |
| --- | --- |
| EventNotifier | Whether the Node can be used to subscribe to Events or not is server-specific. |

#### Variables

For all Variables specified in this document, the Attributes named in Table 9 shall be set as specified in the table. The definitions for the Attributes can be found in OPC 10000-3.

Table 9 – Common Variable Attributes

| Attribute | Value |
| --- | --- |
| MinimumSamplingInterval | Optionally, a server-specific minimum sampling interval is provided. |
| AccessLevel | The access level for Variables used for type definitions is server-specific, for all other Variables defined in this document, the access level shall allow reading; other settings are server-specific. |
| UserAccessLevel | The value for the UserAccessLevel Attribute is server-specific. It is assumed that all Variables can be accessed by at least one user. |
| Value | For Variables used as InstanceDeclarations, the value is server-specific; otherwise, it shall represent the value described in the text. |
| ArrayDimensions | If the ValueRank does not identify an array of a specific dimension (i.e., ValueRank <= 0) the ArrayDimensions can either be set to null or the Attribute is missing. This behaviour is server-specific. If the ValueRank specifies an array of a specific dimension (i.e., ValueRank > 0) then the ArrayDimensions Attribute shall be specified in the table defining the Variable. |
| Historizing | The value for the Historizing Attribute is server-specific. |
| AccessLevelEx | If the AccessLevelEx Attribute is provided, it shall have the bits 8, 9, and 10 set to 0, meaning that read and write operations on an individual Variable are atomic, and arrays can be partly written. |

#### VariableTypes

For all VariableTypes specified in this document, the Attributes named in Table 10 shall be set as specified in the table. The definitions for the Attributes can be found in OPC 10000-3.

Table 10 – Common VariableType Attributes

| Attributes | Value |
| --- | --- |
| Value | Optionally a server-specific default value can be provided. |
| ArrayDimensions | If the ValueRank does not identify an array of a specific dimension (i.e., ValueRank <= 0) the ArrayDimensions can either be set to null or the Attribute is missing. This behaviour is server-specific. If the ValueRank specifies an array of a specific dimension (i.e., ValueRank > 0) then the ArrayDimensions Attribute shall be specified in the table defining the VariableType. |

#### Methods

For all Methods specified in this document, the Attributes named in Table 11 shall be set as specified in the table. The definitions for the Attributes can be found in OPC 10000-3.

Table 11 – Common Method Attributes

| Attributes | Value |
| --- | --- |
| Executable | All Methods defined in this document shall be executable (Executable Attribute set to “True”), unless it is defined differently in the Method definition. |
| UserExecutable | The value of the UserExecutable Attribute is server-specific. It is assumed that all Methods can be executed by at least one user. |

### Structures

OPC 10000-3 differentiates between different kinds of Structures. The following conventions explain, how these Structures shall be defined.

The first kind are Structures without optional fields where none of the fields allows subtype (except fields with abstract DataTypes). Its definition is in Table 12.

Table 12 – Structures without optional fields where none of the fields allow subtypes

| Name | Type | Description |
| --- | --- | --- |
| <someStructure> | structure | Subtype of <someParentStructure> defined in … |
| SP1 | 0:Byte[] | Setpoint 1 |
| SP2 | 0:Byte[] | Setpoint 2 |

The second kind are Structures with optional fields where none of the fields allows subtypes (except fields with abstract DataTypes). Its definition is in Table 13.

Structures with fields that are optional have an “Optional” column. Fields that are optional have True set, otherwise False.

Table 13 – Structures with optional fields

| Name | Type | Description | Optional |
| --- | --- | --- | --- |
| <someStructure> | structure | Subtype of <someParentStructure> defined in … |  |
| SP1 | 0:Byte[] | Setpoint 1 | False |
| Optional Field_1 | 0:String | Some Text | True |

The third kind are Structures without optional fields where one or more of the fields allow subtypes. Its definition is in Table 14.

Structures with fields that allow subtypes have an “Allow Subtypes” column. Fields that allow subtypes have True set, otherwise False. Fields with abstract DataTypes can always be subtyped.

Table 14 – Structures where one or more of the fields allow subtypes

| Name | Type | Description | Allow Subtypes |
| --- | --- | --- | --- |
| <someStructure> | structure | Subtype of <someParentStructure> defined in … |  |
| SP1 | 0:Byte[] | Setpoint 1 | False |
| Allow Subtypes | 0:ByteString | Some Bytestring | True |

Fields with abstract DataTypes shall have True in the “Allow Subtypes” column.

It is not allowed to add both columns to combine optional fields and fields that allow subtypes in one structure.

# General information to Robot Intent and OPC UA

## Introduction to Robot Intent

OPC 40010-1 describes what a robot is — its topology, its axes, its loads and its operating state — and defines no way to tell it to do anything. A client can read that a robot exists, has six axes and is in Automatic mode, and then has no standard verb with which to move it. Commanding therefore happens below the interface, in vendor motion languages that differ from manufacturer to manufacturer, so a cell that mixes two robots mixes two integrations of work that is identical in both.

The gap is not motion planning, and this document does not try to close that one. Trajectory generation, inverse kinematics, collision avoidance and servo control belong to the robot controller, which does them better than anything reachable over a network could. What is missing is the layer above: a way to say *move there*, *grasp that*, *pick from here and place at that*, to watch the request through a lifecycle that survives a dropped connection, and to be told plainly when the robot refuses.

This document defines that layer. Intents are a DataType hierarchy rather than one Method each, so a single submission and a mission step are the same shape and a new verb is a subtyping act. Each submitted intent is an OPC 10000-10 program instance, so its outcome survives the Session that requested it. Queueing and blending are adopted unchanged from PLCopen and VDA 5050 rather than reinvented. And clause 10 says plainly what this interface is not: it is not safety-rated, it reports what the safety system enforces and refuses what would exceed it, and neither of those makes anything safe.

## Introduction to OPC Unified Architecture

This is an OPC UA introduction that can be used as is, shortened or enhanced as appropriate.

### What is OPC UA?

OPC UA is an open and royalty free set of standards designed as a universal communication protocol. While there are numerous communication solutions available, OPC UA has key advantages:

- A state of art security model (see OPC 10000-2).

- A fault tolerant communication protocol.

- An information modelling framework that allows application developers to represent their data in a way that makes sense to them.

OPC UA has a broad scope which delivers for economies of scale for application developers. This means that a larger number of high-quality applications at a reasonable cost are available. When combined with semantic models such as Robot Intent, OPC UA makes it easier for end users to access data via generic commercial applications.

The OPC UA model is scalable from small devices to ERP systems. OPC UA Servers process information locally and then provide that data in a consistent format to any application requesting data - ERP, MES, PMS, Maintenance Systems, HMI, Smartphone, or a standard Browser, for examples. For a more complete overview see OPC 10000-1.

### Basics of OPC UA

As an open standard, OPC UA is based on standard internet technologies, like TCP/IP, HTTP, Web Sockets.

As an extensible standard, OPC UA provides a set of Services (see OPC 10000-4) and a basic information model framework. This framework provides an easy manner for creating and exposing vendor defined information in a standard way. More importantly all OPC UA Clients are expected to be able to discover and use vendor-defined information. This means OPC UA users can benefit from the economies of scale that come with generic visualisation and historian applications. This specification is an example of an OPC UA Information Model designed to meet the needs of developers and users.

OPC UA Clients can be any consumer of data from another device on the network to browser based thin clients and ERP systems. The full scope of OPC UA applications is shown in Figure 1.

Figure 1 – The scope of OPC UA within an enterprise

OPC UA provides a robust and reliable communication infrastructure having mechanisms for handling lost messages, failover, heartbeat, etc. With its binary encoded data, it offers a high-performing data exchange solution. Security is built into OPC UA as security requirements become more and more important especially since environments are connected to the office network or the internet and attackers are starting to focus on automation systems.

### Information modelling in OPC UA

#### Concepts

OPC UA provides a framework that can be used to represent complex information as Objects in an AddressSpace which can be accessed with standard services. These Objects consist of Nodes connected by References. Different classes of Nodes convey different semantics. For example, a Variable Node represents a value that can be read or written. The Variable Node has an associated DataType that can define the actual value, such as a string, float, structure etc. It can also describe the Variable value as a variant. A Method Node represents a function that can be called. Every Node has a number of Attributes including a unique identifier called a NodeId and non-localized name called as BrowseName. An Object representing a ‘Reservation’ is shown in Figure 2.

Figure 2 – A basic Object in an OPC UA Address Space

Object and Variable Nodes represent instances and they always reference a TypeDefinition (ObjectType or VariableType) Node which describes their semantics and structure. Figure 3 illustrates the relationship between an instance and its TypeDefinition.

The type Nodes are templates that define all of the children that can be present in an instance of the type. In the example in Figure 3 the PersonType ObjectType defines two children: First Name and Last Name. All instances of PersonType are expected to have the same children with the same BrowseNames. Within a type the BrowseNames uniquely identify the children. This means Client applications can be designed to search for children based on the BrowseNames from the type instead of NodeIds. This eliminates the need for manual reconfiguration of systems if a Client uses types that multiple Servers implement.

OPC UA also supports the concept of sub-typing. This allows a modeller to take an existing type and extend it. There are rules regarding sub-typing defined in OPC 10000-3, but in general they allow the extension of a given type or the restriction of a DataType. For example, the modeller can decide that the existing ObjectType in some cases needs an additional Variable. The modeller can create a subtype of the ObjectType and add the Variable. A Client that is expecting the parent type can treat the new type as if it was of the parent type. Regarding DataTypes, subtypes can only restrict. If a Variable is defined to have a numeric value, a subtype could restrict it to a float.

Figure 3 – The relationship between Type Definitions and Instances

References allow Nodes to be connected in ways that describe their relationships. All References have a ReferenceType that specifies the semantics of the relationship. References can be hierarchical or non-hierarchical. Hierarchical references are used to create the structure of Objects and Variables. Non-hierarchical are used to create arbitrary associations. Applications can define their own ReferenceType by creating subtypes of an existing ReferenceType. Subtypes inherit the semantics of the parent but can add additional restrictions. Figure 4 depicts several References, connecting different Objects.

Figure 4 – Examples of References between Objects

The figures above use a notation that was developed for the OPC UA specification. The notation is summarized in Figure 5. UML representations can also be used; however, the OPC UA notation is less ambiguous because there is a direct mapping from the elements in the figures to Nodes in the AddressSpace of an OPC UA Server.

Figure 5 – The OPC UA Information Model notation

A complete description of the different types of Nodes and References can be found in OPC 10000-3 and the base structure is described in OPC 10000-5.

The OPC UA specification defines a very wide range of functionality in its basic information model. It is not required that all Clients or Servers support all functionality in the OPC UA specifications. OPC UA includes the concept of Profiles, which segment the functionality into testable certifiable units. This allows the definition of functional subsets (that are expected to be implemented) within a companion specification. The Profiles do not restrict functionality, but generate requirements for a minimum set of functionalities (see OPC 10000-7).

#### Namespaces

OPC UA allows information from many different sources to be combined into a single coherent AddressSpace. Namespaces are used to make this possible by eliminating naming and id conflicts between information from different sources. Each namespace in OPC UA has a globally unique string called a NamespaceUri which identifies a naming authority and a locally unique integer called a NamespaceIndex, which is an index into the Server's table of NamespaceUris. The NamespaceIndex is unique only within the context of a Session between an OPC UA Client and an OPC UA Server- the NamespaceIndex can change between Sessions and still identify the same item even though the NamespaceUri's location in the table has changed. The Services defined for OPC UA use the NamespaceIndex to specify the Namespace for qualified values.

There are two types of structured values in OPC UA that are qualified with NamespaceIndexes: NodeIds and QualifiedNames. NodeIds are locally unique (and sometimes globally unique) identifiers for Nodes. The same globally unique NodeId can be used as the identifier in a node in many Servers – the node's instance data can vary but its semantic meaning is the same regardless of the Server it appears in. This means Clients can have built-in knowledge of of what the data means in these Nodes. OPC UA Information Models generally define globally unique NodeIds for the TypeDefinitions defined by the Information Model.

QualifiedNames are non-localized names qualified with a Namespace. They are used for the BrowseNames of Nodes and allow the same names to be used by different information models without conflict. TypeDefinitions are not allowed to have children with duplicate BrowseNames; however, instances do not have that restriction.

#### Companion Specifications

An OPC UA companion specification for an industry specific vertical market describes an Information Model by defining ObjectTypes, VariableTypes, DataTypes and ReferenceTypes that represent the concepts used in the vertical market, and potentially also well-defined Objects as entry points into the AddressSpace.

# Use cases

- Task-level cell control. A cell controller sequences a robot, a fixture and a conveyor without embedding vendor motion code, because the robot's capability is declared and its verbs are portable.

- Planner and agent integration. A planner — symbolic, learned, or a language model — emits intents against a declared vocabulary, and receives structured failures it can re-plan against rather than a vendor error string.

- Mixed-fleet work cells. Two robots from different manufacturers execute the same mission definition, because the mission is expressed in intents and each Server translates into its own controller's language.

- Long-running supervised operation. A supervisory system submits a mission, watches it progress, revises the part that has not yet been committed, and cancels it cleanly when the upstream process changes.

- Auditable commanding. Every intent records which Session commanded it, when, with what arguments and to what outcome. Part 10 carries that in ProgramDiagnostic, which it leaves Optional; §6.1 promotes it to Mandatory here, because a capability a specification advertises cannot rest on a member a conformant Server may omit.

# Robot Intent information model overview

## The layered contract

This specification occupies one band and touches neither the band above nor the two below.

Figure 1 – The layered contract

A Server implementing this specification shall translate intents into whatever its controller actually executes. It shall not require a client to know which controller that is: the point of the model is that the same intent, submitted to two Servers driving robots from two manufacturers, produces the same physical outcome within the tolerance each robot is capable of.

The safety layer is beside and beneath this interface and is never mediated by it. That relationship is normative and is stated in clause 10.

## Intent, not trajectory

An intent says what is to be achieved and constrains how far the Server may go in achieving it. It does not say how. The Server owns path planning, inverse kinematics, singularity avoidance, collision checking and the choice of configuration.

This is what makes the model portable. Every vendor already solves those problems, differently and well; a specification that dictated the solution would be implementable by nobody. MotionConstraintsDataType is therefore a set of bounds, not a plan, and a Server shall clamp a request to what the robot is configured to permit rather than refuse it — except where clause 10 requires refusal.

The same reasoning applies to blending. BlendDataType.Radius is a request in metres because that is the only unit two vendors share; controllers that expose a unitless blend scale map it as best they can, and a Server that cannot honour the exact radius shall still succeed. A client that needs the exact path uses Exact termination, which every vendor implements identically.

## What this interface carries, and what it brokers

OPC UA method invocation is not deterministic and, in the deployments this model is written for, completes in tens of milliseconds. Vendor real-time channels run two to four orders of magnitude faster on dedicated transports. That difference is not something a specification can argue away, so this model divides the work rather than pretending the gap is not there.

Carried here — one submission, executed by the robot. A trajectory, a Cartesian path or a force-controlled move is handed over whole and run by the robot's own motion kernel (6.14.8). The round trip happens once, at submission, so the transport's latency bounds how quickly work can be started and never how accurately it is executed. This is the same shape as FollowJointTrajectory in ROS and the buffered path function blocks of PLCopen, and it is why trajectory execution belongs here while trajectory streaming does not.

Brokered — described, leased, and left alone. Where a client genuinely needs a high-rate channel — visual servoing, force tracking, conveyor following — the Server describes one and leases it (6.14.9). The samples travel on that channel and never through this interface.

A Server shall not present this interface as a real-time control channel, and a client shall not use it as one. In particular:

- servo-level or joint-cyclic control shall not be attempted through repeated submission;

- a closed force or impedance loop requiring a bounded control period shall use a brokered channel, not ForceIntentDataType, which commands a move until contact and not a continuous loop;

- conveyor tracking, seam tracking and any other motion slaved to an external signal at rate shall use a brokered channel or the robot's own facility.

IntentOperationType.CurrentPose exists so a client can watch a motion. It is a status report delivered at whatever rate the client's Subscription asks for, and using it to close a control loop is outside this specification.

## Architecture

Figure 2 – Architecture of an intent controller in the AddressSpace

A client browses Server/RobotIntent/Controllers to find every robot it may command, reads that controller's Capabilities to learn what it accepts, resolves the frames and locations its intents will refer to, and then submits.

## Organisation of the model

The AddressSpace figures in this document use the OPC UA graphical notation of OPC 10000-3. A Node of an instance NodeClass — Object, Variable or View — is a plain rectangle, a Method is a rounded rectangle, and a type — ObjectType, VariableType, ReferenceType or DataType — is a rectangle standing on a shadow. An abstract type is set in italics, and a Node whose BrowseName is a placeholder is written in angle brackets. A HasTypeDefinition reference carries a solid arrowhead; a HasComponent reference is the plain unlabelled arrow; every other ReferenceType is drawn with its BrowseName on the arrow. A figure shows the part of the model its clause describes, never the whole of it.

Figure 3 – Graphical notation used by the AddressSpace figures

## Type hierarchy

The entry point is a root Object holding the controllers a Server exposes:

<!-- model-figure: root=ns=1;i=1001 require=mandatory external=BaseObjectType -->

Figure 4 – The type hierarchy

A controller carries the containers a client browses, the intent queue, and the control-ownership and submission Methods. The figure shows those; Annex A carries the full member list:

<!-- model-figure: root=ns=1;i=1002 external=BaseObjectType -->

Figure 5 – Instance structure of an intent controller

Table 1 – Type hierarchy

| Type | NodeId | Subtype of |
| --- | --- | --- |
| RobotIntentRootType | ns=1;i=1001 | BaseObjectType |
| IntentControllerType | ns=1;i=1002 | BaseObjectType |
| IntentOperationType | ns=1;i=1003 | ProgramStateMachineType (i=2391) |
| MissionType | ns=1;i=1004 | ProgramStateMachineType (i=2391) |
| IntentCapabilitiesType | ns=1;i=1005 | BaseObjectType |
| CoordinateFrameType | ns=1;i=1006 | BaseObjectType |
| ToolType | ns=1;i=1007 | BaseObjectType |
| LocationType | ns=1;i=1008 | BaseObjectType |
| AxisType | ns=1;i=1009 | BaseObjectType |
| OutputSignalType | ns=1;i=1010 | BaseObjectType |
| ProgramType | ns=1;i=1011 | BaseObjectType |
| SafetyStateType | ns=1;i=1012 | BaseObjectType |
| RealTimeChannelType | ns=1;i=1013 | BaseObjectType |
| RobotDescriptionType | ns=1;i=1014 | BaseObjectType |

Annex A is the authoritative node reference and carries every member with its DataType, ValueRank and ModellingRule.

## Poses, frames and units

Pose3DDataType (ns=1;i=3050) carries FrameId, a Position of three Double in metres, and an Orientation of four Double forming a unit quaternion ordered (x, y, z, w).

Four rules make this unambiguous, and a Server shall satisfy all of them.

- Every frame in this model is right-handed.

- Position is in metres; joint targets are in radians for a Revolute axis and metres for a Prismatic one; force is in newtons; time is in milliseconds where carried as Duration. These units are fixed by this specification and are not negotiable per-instance. Pose3DDataType appears as a Method argument, where no EUInformation property can reach it, so a per-instance unit would be undeliverable.

- Orientation shall be normalised. A Server receiving a quaternion whose norm differs from 1 by more than 1e-6 shall reject the intent with ParameterInvalid.

- FrameId names a CoordinateFrameType instance under the controller's Frames folder. An empty FrameId means the Server's default work frame.

Quaternions are used because OPC UA defines no quaternion DataType anywhere, and because the A, B, C fields of the core ThreeDOrientation carry no convention of their own — the only normative assignment of meaning to them is external to the base specification. A quaternion has no such ambiguity, no gimbal degeneracy, and is what robot controllers and scene descriptions already hold internally. Annex C gives the normative bidirectional conversion to ThreeDFrame, so a Server that also speaks OPC 40010-1 or a spatial-location model can move between the two without inventing a convention.

## The intent hierarchy

Intents are a DataType hierarchy, not one Method per verb.

Figure 6 – The intent DataType hierarchy

Three consequences follow, and each is the reason for the choice:

- A single intent and a mission step are the same shape. MissionStepDataType.Intent is an IntentDataType, so nothing has to be expressed twice.

- Extension is subtyping. A vendor or a later part adds an intent by deriving from IntentDataType. It is then carried, queued, cancelled and reported by the existing machinery without a new Method.

- Discovery is a read, not a probe. IntentCapabilitiesType.SupportedIntents names each accepted DataType. A client learns what a robot accepts by reading one Variable, rather than by browsing for BrowseNames and inferring support from their presence.

IntentDataType carries what every intent needs: IntentId, Label, BufferMode and BlockingMode. MotionIntentDataType adds ToolFrame, Constraints and Blend — the members that are meaningless for SetOutput and essential for a move.

Fields whose DataType is one of the two abstract structures are emitted with AllowSubTypes="true", so a client decoding a mission does not have to infer polymorphism from the abstractness of a DataType.

## Motion intents

JointMoveIntentDataType (ns=1;i=3055) interpolates in joint space. The tool centre point's path is not controlled and shall not be relied on. It carries either explicit JointTargets or a TargetPose, and HasJointTargets decides which — a Boolean discriminator rather than a sentinel, so that neither field has to encode "unset".

Giving a pose rather than joint values is the "move there, you choose how" case: the Server solves the kinematics and selects a configuration. A Server shall reject a JointMoveIntentDataType whose HasJointTargets is true and whose JointTargets length differs from IntentCapabilitiesType.AxisCount, with ParameterInvalid.

LinearMoveIntentDataType (ns=1;i=3056) drives the tool centre point along a straight line. CircularMoveIntentDataType (ns=1;i=3057) drives it along the arc through ViaPoint to Target; only the position of ViaPoint defines the arc, and a Server shall ignore its orientation.

The three correspond to the instructions every vendor already has, which is why they are three and not one:

Table 2 – Motion intents

| This specification | UR | ABB | KUKA | FANUC | Yaskawa |
| --- | --- | --- | --- | --- | --- |
| JointMoveIntentDataType | movej | MoveJ / MoveAbsJ | PTP | J | MOVJ |
| LinearMoveIntentDataType | movel | MoveL | LIN | L | MOVL |
| CircularMoveIntentDataType | movec | MoveC | CIRC | C | MOVC |

### Paths, trajectories and contact

Three further motion intents cover the work that a single target pose cannot express.

CartesianPathIntentDataType (ns=1;i=3074) carries a list of PathWaypointDataType (ns=1;i=3073), each a pose with the blend that applies at it. It carries no timing: the Server paces it from Constraints. This is the portable form of a taught path, and per-waypoint blending is exactly what distinguishes it from a sequence of separate linear moves — the robot need not stop between waypoints.

TrajectoryIntentDataType (ns=1;i=3072) carries TrajectoryPointDataType (ns=1;i=3070) points, each with TimeFromStart and per-axis positions, optionally velocities and accelerations. Timing is what makes it a trajectory rather than a path. It also carries a PathTolerance and a GoalTolerance, both MotionToleranceDataType (ns=1;i=3071), and a GoalTimeTolerance, which is a Duration — lateness is one number, not a pose deviation. Between them, "did it work?" has an answer the client set rather than one the Server chose.

A Server shall reject a trajectory whose points are not in ascending TimeFromStart order, or whose Positions length differs from AxisCount, with ParameterInvalid. Where MaxTrajectoryPoints is non-zero, it shall reject a longer trajectory with ParameterInvalid.

The whole trajectory is submitted in one call and executed by the robot's own motion kernel. That is the point: the transport is on the critical path once, at submission, and never during execution (6.3).

ForceIntentDataType (ns=1;i=3075) travels along Direction until ContactForce is reached or MaxDistance is exhausted. Exhausting the distance without contact shall be reported as Failed with ObjectNotFound — the intent was to touch something, and not touching it is not success. HoldForce keeps the robot pressing after contact instead of stopping.

A Server shall report ForceControlSupported false unless the robot genuinely regulates force. Accepting a force intent and ignoring the force would tell a client its part was pressed when it was only approached.

### Process intents

ProcessIntentDataType (ns=1;i=3076) is the abstract base of the intents that run an application process along a path. Beyond each process's own parameters it carries two things every process needs: ProcessProgram, referencing the procedure the equipment already holds, and Attributes for what this specification has not standardised.

Table 3 – Process intents

| Intent | NodeId | Covers |
| --- | --- | --- |
| ArcWeldIntentDataType | ns=1;i=3077 | voltage, wire feed and travel speed, gas pre- and post-flow, arc start delay, crater fill, weave, seam tracking |
| SpotWeldIntentDataType | ns=1;i=3078 | weld schedule, gun force, approach and retract distance, stack thickness, tip dress |
| DispenseIntentDataType | ns=1;i=3079 | flow rate, trigger-on and trigger-off distance, bead width, material temperature, purge |
| FastenIntentDataType | ns=1;i=3080 | joint reference, program number, torque, angle, snug torque |
| PalletiseIntentDataType | ns=1;i=3081 | pattern, layer, row, column, item orientation |
| SurfaceFinishIntentDataType | ns=1;i=3082 | contact force, feed rate, tool speed, step-over |

Each parameter set is the subset the vendor languages agree on — ABB seamdata/welddata/weavedata, FANUC weld schedules and KUKA ArcTech for arc welding, and their equivalents elsewhere. Where an installation works to a welding procedure specification, WeldProcedureRef names it and this specification does not restate its content.

Two of these deserve their reasoning stated.

Fastening is deliberately thin. OPC 40450 and OPC 40451 already define industrial joining and tightening in full, including step-wise results and traces. Where a controller exposes such a model, FastenIntentDataType.Joint references the joint in that model and the result belongs there. Where no such model is exposed under the controller, Joint is null, the remaining fastening parameters stand alone, and a Server shall refuse a non-null Joint with CapabilityNotSupported. Restating torque strategies here would create a second definition of a fact that specification already owns — the same reason PickIntentDataType.Source references a Location node instead of naming a station in a string.

Palletising references a pattern, not a computed pose. Pattern is a LocationType, so the geometry has one definition a client can read and subscribe to, rather than being recomputed from indices independently on both sides and disagreeing.

WeaveShapeEnum (ns=1;i=3018) gives the oscillation across an arc weld seam: None, Sine, Zigzag, Trapezoid.

## Manipulation intents

GraspIntentDataType and ReleaseIntentDataType actuate an end effector. Force and Width are requests: an end effector that cannot regulate force shall ignore Force and still succeed, because refusing would make the intent unusable on the majority of grippers that are open or closed and nothing else.

PickIntentDataType and PlaceIntentDataType reference a LocationType node through Source and Destination. They do not name a station in a string. A location therefore has exactly one definition — its pose, its occupancy, what it holds and how much of it — which a client can read, subscribe to and reason about. A free-text station identifier would be a second definition of the same fact, able to disagree with the first.

ToolChangeIntentDataType references the ToolType to fit; a null Tool releases the fitted tool and fits nothing.

## Auxiliary intents

SetOutputIntentDataType writes an OutputSignalType node, so the signal's range, unit and meaning are described once in the address space instead of being implied by a line name. Value is BaseDataType and shall match the signal's own DataType.

CallProgramIntentDataType runs a ProgramType held on the controller. It is the escape hatch for capability this model does not describe, and the bridge to the programs an OPC 40010-1 task control already exposes (Annex B).

WaitIntentDataType waits for a duration, for a signal, or for both. A mission needs it to express a rendezvous with something the robot does not control; without it a client has to hold the queue open from outside, which defeats the point of submitting a mission at all.

Signal is bounded so that 6.19.3 has something to check: it shall resolve either to an OutputSignalType instance under the controller being commanded, or to a Variable of DataType Boolean under it, and a Server shall refuse anything else with ParameterInvalid. A NodeId-valued member that no rule constrains cannot be validated, and an unvalidated NodeId is the surface 6.19.3 exists to close.

## Reference objects

CoordinateFrameType instances form a tree through HasFrameParent, so a pose given in one frame can be re-expressed in another by composing the transforms along the path between them. Role follows ISO 9787, which standardises which frames exist; the transform between them is carried explicitly because no standard says how to calibrate it.

ToolType.TcpFrame shall reference a CoordinateFrameType whose Role is Tool. At most one ToolType instance under a controller shall have Fitted true at any time.

AxisType.Index fixes the position of that axis in JointMoveIntentDataType.JointTargets, and Kind fixes the unit of that entry. The indices of the axes under one controller shall be the contiguous range 0 to AxisCount − 1.

### What the controller itself reports

Four reports of IntentControllerType say what the robot is doing right now, and each is read-only and normative.

ActiveMission is required only where MissionsSupported is true, because a Server that does not implement

missions has no mission instance to reference.

- OperationalMode is the robot's own report of the mode in force. 6.18.2 gates submission on it and forbids commanding it.

- Ready is true exactly when the Server would admit a well-formed intent from the Session that holds command authority. A Server shall report it false whenever 6.14.2 or 6.18.4 would refuse for a reason that does not depend on the intent — outside Automatic or AutomaticExternal, under an emergency or protective stop, or with SafetyControllerOk false. It exists so a client can see that submitting is pointless without submitting to find out; it is a hint about the Server, and a Server shall not treat a client's having read it as licence to skip any check of 6.14.2.

- ActiveIntent references the IntentOperationType instance whose ExecutionState is Executing or Cancelling, and is null when none is. On a Server that supports missions, where the executing intent belongs to a mission, ActiveMission references that mission's MissionType instance; otherwise ActiveMission is null.

- ControlOwner reports the Session holding command authority, or null. Clause 8 governs it.

None of these is a substitute for the per-operation state of 6.14.3. Two members may not report the state of one intent: the operation's own state machine decides, and these summarise it.

### RobotDescriptionType

RobotDescriptionType (ns=1;i=1014) carries enough of the robot's construction for a client to plan against it without a second specification: a KinematicChain of KinematicJointDataType (ns=1;i=3084) from the base outwards, a MountingPose, a ReachRadius, a PayloadLimit, and ceilings on tool centre point speed and acceleration.

Each KinematicJointDataType names the AxisType it corresponds to, its Kind, the OriginTransform of its frame within its predecessor's at zero position, and the unit AxisVector it rotates about or translates along.

This is additive, not duplicative. OPC 40010-1 describes a robot's topology and its axes in detail and defines no kinematic chain an inverse-kinematics solver could use, and no tool centre point at all. Where OPC 40010-1 is also implemented, its axis description is the fuller one and Annex B fixes which side decides.

### SafetyStateType

SafetyStateType (ns=1;i=1012) reports what the robot's safety system is doing: ActiveFunction, EmergencyStopActive, ProtectiveStopActive, SafeSpeedLimitActive, SafeSpeedLimit, SafetyControllerOk and a human-readable LastStopReason.

SafeMotionFunctionEnum (ns=1;i=3013) names the function being enforced, using the vocabulary of IEC 61800-5-2: None, Sto, Ss1, Ss2, Sos, Sls, Slp, Sdi, Sbc.

Every member is read-only and a report. The safety system enforces these independently of this interface and remains effective when the Server is unreachable. Clause 10 states what a Server must do with them and what a client may not conclude from them.

### RealTimeChannelType

RealTimeChannelType (ns=1;i=1013) describes a high-rate channel the Server can offer, so a client can find and open one: Transport, EndpointUrl, Initiator, NominalRate, PayloadDescriptor, RequiredMode, Available, and the current LeaseHolder and LeaseExpiry.

RealTimeTransportEnum (ns=1;i=3014) names the transport: Rtde, Egm, Fri, Rsi, MotoRos2, OpcUaFx, Other. Of these only OpcUaFx — OPC UA FX, OPC 10000-80 to -84 — is an OPC Foundation specification; the rest are vendor channels this model describes without defining.

ChannelInitiatorEnum (ns=1;i=3015) says which end opens the connection: Server or Client. It is stated rather than left to the reader because getting it wrong is the usual reason a first connection attempt fails.

6.14.9 defines the lease.

## Enumerations

The values below are normative. Where a value is cited as interoperable with another specification, renumbering it breaks that claim.

ExecutionStateEnum (ns=1;i=3001) — tabulated with its Part 10 pairing in §6.3: Accepted 0, Queued 1, Executing 2, Suspended 3, Cancelling 4, Succeeded 5, Failed 6, Cancelled 7, Retriable 8.

BufferModeEnum (ns=1;i=3002) — the values of PLCopen MC_BufferMode, adopted unchanged.

Table 4 – Enumerations

| Value |  | Meaning |
| --- | --- | --- |
| Aborting | 0 | Abort what is executing and start immediately. The default, and always accepted. |
| Buffered | 1 | Queue; start when the predecessor succeeds. |
| BlendingLow | 2 | Blend at the lower of the two boundary speeds. |
| BlendingPrevious | 3 | Blend at the predecessor's boundary speed. |
| BlendingNext | 4 | Blend at the successor's boundary speed. |
| BlendingHigh | 5 | Blend at the higher of the two boundary speeds. |

BlockingModeEnum (ns=1;i=3003) — the VDA 5050 blockingType matrix.

Table 5 – Enumerations (2)

| Value |  | Motion may continue | Other intents may run |
| --- | --- | --- | --- |
| None | 0 | yes | yes |
| Soft | 1 | no | yes |
| Single | 2 | yes | no |
| Hard | 3 | no | no |

TerminationModeEnum (ns=1;i=3004) — Exact 0 (come to rest on the target), Blend 1 (round the corner into the next motion).

ReleaseModeEnum (ns=1;i=3005) — Drop 0 (open where it is), Place 1 (set down under control at the target), Handover 2 (retain until a receiving party has taken it).

ApproachModeEnum (ns=1;i=3006) — Default 0 (the Server chooses), ToolZ 1 (along the tool's own Z axis), Top 2 (from above, in the target's frame), Side 3 (laterally, in the target's frame).

FrameRoleEnum (ns=1;i=3007) — the coordinate systems ISO 9787 standardises: World 0, Base 1, MechanicalInterface 2 (the flange), Tool 3, Object 4 (a workpiece frame), Other 5.

OperationalModeEnum (ns=1;i=3008) — the ISO 10218-1 modes, numbered as OPC 40010-1 numbers them: Other 0, ManualReducedSpeed 1, ManualHighSpeed 2, Automatic 3, AutomaticExternal 4. Clause 10 permits submission only in the last two.

StopModeEnum (ns=1;i=3010) — the values of PossibleStopModes in OPC 40010-1: OnPath 1, EndOfCycle 2, ProcessStop 3, QuickStop 4, EndOfInstruction 5. 6.18.3 states what these do not mean.

AxisKindEnum (ns=1;i=3011) — Revolute 0 (its joint target is in radians), Prismatic 1 (in metres).

MissionUpdateResultEnum (ns=1;i=3012) — Accepted 0, Outdated 1 (the update identifier was not greater), BaseConflict 2 (it would have altered a released step), UnknownMission 3, Rejected 4 (refused for a reason the Server states in Message).

IntentFailureEnum (ns=1;i=3009) — the reason an intent did not succeed. The set is small and diagnosable on purpose: a client decides whether to retry, re-plan or escalate from this value alone.

Table 6 – Enumerations (3)

| Value |  | Meaning |
| --- | --- | --- |
| None | 0 | No failure; reported on success. |
| Unreachable | 1 | The target lies outside the reachable workspace. |
| Kinematics | 2 | No kinematic solution, or a singularity on the path. |
| Collision | 3 | A collision was predicted or detected. |
| JointLimit | 4 | A joint limit would be or was exceeded. |
| SpeedLimit | 5 | The requested speed is not permitted in the active mode. |
| ToolMissing | 6 | The required tool is not fitted or not identified. |
| ObjectNotFound | 7 | The object to act on was not present. |
| GraspFailed | 8 | The object was not acquired, or was lost in transit. |
| Timeout | 9 | Did not complete within its permitted time. |
| NotPermittedInMode | 10 | Refused; the operational mode does not permit it (6.18.2). |
| ControlNotOwned | 11 | Refused; the caller does not hold command authority (clause 8). |
| CapabilityNotSupported | 12 | Not implemented, or not in this combination (clause 9). |
| ParameterInvalid | 13 | A parameter was missing, malformed or out of range. |
| QueueFull | 14 | The queue is at MaxQueueDepth. |
| Superseded | 15 | An Aborting submission or a mission update replaced it. |
| HardwareFault | 16 | A fault in the robot, the end effector or the controller. |
| SafetyStop | 17 | A safety function acted. The safety system decided this, not this interface. |
| Other | 18 | A reason none of the above describes; see Message. |
| SafetyLimitExceeded | 19 | Refused; the request would exceed a limit the safety system is enforcing (6.18.3). |
| NoTransition | 20 | A mission branch point had no true outgoing transition, or the selected transition target did not resolve (6.15.4). |

ErrorPolicyEnum (ns=1;i=3016) — what a mission does when a step does not succeed (6.15.4): Abort 0 (the default), Retry 1, Skip 2, Fallback 3, Compensate 4.

DivergenceKindEnum (ns=1;i=3017) — how the transitions leaving one step relate, following the divergence of an IEC 61131-3 sequential function chart: Alternative 0 (exactly one is taken — an OR divergence), Parallel 1 (all are taken and the branches run concurrently — an AND divergence).

## Intent lifecycle

### Why an intent is a program instance

A Call cannot outlive the Session that made it, and OPC 10000-4 §5.12.2 states that when a Session ends the method result is discarded "independent of the task actually performed at the Server". A robot commanded by a synchronous method therefore keeps moving after the answer has been thrown away — and OPC 10000-10 §4.1 gives the OPC Foundation's own resolution: a Method performs a calculation, a Program runs a batch process or a machine tool part program.

SubmitIntent accordingly returns as soon as the intent is admitted, not when the robot has finished. What it returns is a NodeId: an IntentOperationType instance created for that submission, which the client subscribes to for progress and reads for the result.

Building on ProgramStateMachineType rather than defining a fresh state machine buys four things this specification then does not have to invent: transition events, a terminal result object that survives the operation, invocation diagnostics recording which Session commanded what, and a lifetime model for the instance itself.

Two of those are Optional in Part 10, and this specification promotes both. IntentOperationType declares FinalResultData and ProgramDiagnostic Mandatory.

Inheriting them would not have been enough. 6.14.7 requires the result to be reachable under FinalResultData, and 5 advertises auditable commanding, which is ProgramDiagnostic and nothing else. Both would have rested on members a fully conformant Server could omit — so a Server could pass every conformance test while providing neither, and the two claims would be false against a legal implementation. Promoting them is what makes the claims testable rather than aspirational.

A promotion changes the ModellingRule and nothing else. Both members are therefore declared exactly as OPC 10000-10 declares them — FinalResultData an Object of BaseObjectType reached by HasComponent, and ProgramDiagnostic a Variable of ProgramDiagnostic2Type of DataType ProgramDiagnostic2DataType, also reached by HasComponent. Altering the reference type or the TypeDefinition would declare a second member beside the inherited one rather than promote it, and a client written against Part 10 would then find two.

### Submission

On SubmitIntent a Server shall, in this order:

- Refuse with ControlNotOwned if the calling Session does not hold command authority (clause 8).

- Refuse with NotPermittedInMode if OperationalMode is not Automatic or AutomaticExternal (clause 10).

- Refuse with CapabilityNotSupported if the intent's DataType is not among SupportedIntents, or if its BufferMode or BlockingMode is not among those the capability entry permits.

- Refuse with ParameterInvalid if a parameter is missing, malformed or out of range.

- Refuse with QueueFull if admitting it would exceed MaxQueueDepth.

- Otherwise create an IntentOperationType instance, assign an IntentId if the request left it empty, and return both.

A refusal at any of these steps shall not create an operation instance and shall not move the robot.

A refusal is an output, not a StatusCode. 6.18.5 makes refusal an ordinary outcome, so a Server shall return Good from SubmitIntent and report the refusal in its output arguments: Accepted false, Failure set to the IntentFailureEnum value named above, Message carrying detail for a human, IntentId empty and Operation null. On admission it returns Accepted true, Failure None, and the assigned IntentId and Operation. SubmitMission and Retry report a refusal the same way.

This is what makes the ordered rules above observable. A Server that signalled a refusal with a Bad StatusCode would tell a client only that something went wrong, and the whole point of a small, diagnosable failure set (6.13) is that a client decides whether to retry, re-plan or escalate from that value alone. A Server shall not substitute a Bad StatusCode for one of these refusals. Bad_ codes remain what they always were — the transport, the Session and the Service layer failing — and a client shall distinguish the two.

An IntentId returned by the Server shall be unique among the intents that Server currently holds. A client-supplied IntentId that collides with an outstanding one shall be refused with ParameterInvalid.

### States

The Part 10 state machine carries the coarse lifecycle. ExecutionState refines it, because Queued, Cancelling and the distinction between the three terminal outcomes cannot be told apart from CurrentState alone.

The state machine is authoritative for the coarse state and is what generates events. ExecutionState shall at all times be consistent with it according to the following table, which is exhaustive: a pairing not listed here is not legal.

Table 7 – States

| ExecutionState | Part 10 state | Meaning |
| --- | --- | --- |
| Accepted | Ready | Admitted and validated; not yet queued or executing. |
| Queued | Ready | Waiting behind another intent. QueuePosition is non-zero. |
| Executing | Running | Commanding the robot now. |
| Suspended | Suspended | Paused; position retained. |
| Cancelling | Running | A cancel was accepted; motion is being brought to a controlled end. |
| Succeeded | Halted | Terminal. Completed as requested. |
| Failed | Halted | Terminal. Result.Failure carries the reason. |
| Cancelled | Halted | Terminal. Ended early because a cancel was accepted. |
| Retriable | Halted | Terminal for now; Retry may re-attempt it. |

Figure 7 – The intent execution state model

The distinction between Accepted and Queued is the one PLCopen draws between an axis command that is busy and one that is actively commanding. A client that cannot see it cannot tell "the robot has not started your work yet" from "the robot is working on it".

### Queueing and blending

BufferMode on the submitted intent decides how it relates to what is already executing.

Aborting is the default and shall be accepted by every Server. It terminates the executing intent as Cancelled — with Result.Failure set to Superseded — and begins the new one.

A supersede carries no client-chosen StopMode, because the submission that caused it names none. The Server chooses, and should choose the most urgent stop the cell tolerates, since the successor is about to command motion of its own and the two must not overlap. A Server should document which mode a superseded intent is stopped with, so the behaviour is predictable rather than discovered.

Buffered queues the intent; it begins when its predecessor reaches Succeeded.

The four blending modes queue the intent and additionally ask that the robot not decelerate to a stop at the boundary. Where blending occurs, the predecessor reaches Succeeded when blending begins, not when its target is exactly attained, and its Result.AchievedPose shall record where the tool centre point was at that moment. This is the behaviour PLCopen defines, and reporting it any other way would tell a client the robot stopped somewhere it never was.

A Server that accepts a blending mode but executes it as Buffered shall report BlendingSupported false. A client can then tell a robot that blends from one that merely tolerates being asked to.

MaxQueueDepth bounds the queue. A Server with MaxQueueDepth zero accepts only Aborting submissions.

BlockingMode is orthogonal to BufferMode and constrains concurrency rather than ordering: whether motion may continue during the intent, and whether other intents may run alongside it. A Server shall not begin an intent whose BlockingMode is Single or Hard while any other intent is executing.

### Cancellation, and what the Cancel Service is not

NOTE   The OPC UA Cancel Service defined in OPC 10000-4 §5.7.5 cancels an outstanding service request. It does not stop the robot. Invoking it against a submission returns Bad_RequestCancelledByClient for that request and leaves the motion running.

Stopping a robot is CancelIntent, CancelMission or CancelAll — Methods on the information model, with real-world effect. This distinction is normative because a specification that leaves it implicit produces implementations that believe they have a stop button and do not.

A Server may refuse a cancel, and reports that in the Accepted output. Some motions cannot be abandoned part-way without leaving the cell in a worse state than completing them — a tool change mid-exchange, a placement mid-release. A capability entry whose CancelSupported is false declares this in advance for a whole intent type; Accepted false reports it for one occasion.

Where a cancel is accepted, the operation enters Cancelling and then Cancelled. Cancelling is not terminal: a client that treats the acceptance of a cancel as the end of motion will act too early.

StopMode says how urgently to stop. It carries the values of PossibleStopModes in OPC 40010-1 so that a Server implementing both reports one vocabulary. It selects no IEC 60204-1 stop category; clause 10 explains why it cannot.

A Server shall either honour the requested StopMode or treat every value as its single stop behaviour; it shall not vary silently between the two. Where it cannot differentiate, it should say so — the capability declaration is the natural place — because a client that asks for OnPath and silently receives a QuickStop has been told something untrue about how the cell came to rest. A Server that does differentiate should make the difference observable in Result.AchievedPose, which records where the tool centre point actually stopped.

### Pause, resume and retry

Pause suspends execution retaining position, and Resume continues it. Both are optional, and a capability entry declares per intent type whether they are honoured.

Suspending execution means the robot stops. A Server shall not report Suspended for an operation whose motion is still in progress: Part 10 defines that state as position retained, and a client or operator that reads it will act on the robot having come to rest. Since this specification defines no channel through which a Server can instruct an actuator to pause mid-motion, whether a Server can honour Pause for a running intent depends entirely on the underlying controller.

A Server that cannot suspend a running intent shall declare PauseSupported false for that intent type. It may still stop its queue on Pause - refusing to start further work is useful on its own - but the executing operation remains Executing until it finishes, and Ready shall reflect that no new work will be admitted. This is the same rule clause 9 applies to BlendingSupported: declare false rather than accept work that will not be performed.

Retriable is a terminal state a Server uses where it judges an intent worth another attempt — a grasp that closed on nothing, a location that was momentarily blocked. Retry creates a new IntentOperationType instance for the new attempt; the original remains, terminal, with its own result. A Server that does not offer Retry never enters Retriable and reports Failed instead.

Retry refuses like a submission does, and reports it the same way (6.14.2): Accepted false with a Failure and a Message. A named intent that is not in Retriable is refused with ParameterInvalid, and one whose capability entry declares RetrySupported false with CapabilityNotSupported.

### Results

When an operation reaches a terminal state its Result shall be complete and shall not change thereafter. The same IntentResultDataType value shall also be reachable under the FinalResultData object, which this specification promotes to Mandatory on IntentOperationType for exactly this reason — Part 10 leaves it Optional, and a shall that rests on a member a conformant Server may omit is not a requirement, so that a client written against Part 10 finds the result where Part 10 says it will be.

Result.AchievedPose records where the driven tool centre point came to rest, or was when blending began. It is what lets a client audit a placement and distinguish a blended corner from an exact stop.

Result.Failure is a small, diagnosable set precisely so that a client can decide from it alone whether to retry, re-plan or escalate. Message is for a human and shall not be parsed.

A Server shall retain a terminated operation for long enough that a client which was disconnected at the moment of completion can still read its result on reconnection. It may then delete it; AutoDelete and RecycleCount, inherited from Part 10, describe what it does.

### Trajectory execution

A trajectory is submitted like any other intent and tracked by the same lifecycle. Two things are particular to it.

It is handed over whole. A Server shall accept the entire trajectory at submission and execute it without further exchange. It shall not require a client to feed points during execution, because a transport that cannot guarantee the next point arrives in time cannot be part of the control loop (6.3).

Tolerance decides success. While executing, a Server shall report Failed with Kinematics if deviation exceeds PathTolerance. At the end it shall report Failed with Kinematics if the final deviation exceeds GoalTolerance, and Failed with Timeout if completion is later than the final point's TimeFromStart by more than GoalTimeTolerance. A tolerance of zero or less means the Server applies its own, and a Server that applies its own should publish it in Result.Outputs so the client can learn what was actually enforced.

Progress is meaningful for a trajectory in a way it is not for a single move: a Server should report the fraction of TimeFromStart elapsed.

### Brokering a real-time channel

Where a client needs a rate this interface cannot carry, the Server describes a channel and leases it. The samples travel on that channel; this specification defines no transport and inspects no payload.

OpenRealTimeChannel takes a lease. A Server shall refuse it — returning Granted false, with Message saying which of these it was — when the channel is not Available, when another Session holds the lease, when OperationalMode is not the channel's RequiredMode, or when the caller does not hold command authority. On success it returns the EndpointUrl, the PayloadDescriptor and a LeaseExpiry.

A Server shall bound RequestedLease to what it is willing to grant and report the bounded value in LeaseExpiry; a RequestedLease of zero or less asks for the Server's own default.

A lease shall lapse at LeaseExpiry unless renewed by a further OpenRealTimeChannel from the holding Session, and shall be released when that Session closes. This is the same reasoning as command authority in clause 8: a client that dies must not hold a resource for good.

CloseRealTimeChannel releases the lease explicitly.

Two rules keep the division of labour honest:

- While a channel lease is held, a Server shall refuse motion intents with CapabilityNotSupported unless it can genuinely arbitrate between the two sources. Two things commanding one robot with no arbitration is the failure this rule exists to prevent.

- A Server shall not represent a brokered channel as being under this interface's control. Its behaviour, its guarantees and its failure modes are the transport's.

Of the transports named, only OpcUaFx — OPC UA FX, OPC 10000-80 to -84 — is an OPC Foundation specification. It is the open path, and a Server that offers it gives a client something it can implement from published documents rather than from a vendor SDK.

## Missions

### Purpose

A mission is an ordered sequence of intents submitted and tracked as a unit. It exists so that a supervisor can commit work in advance — the robot keeps moving through a sequence without a round trip per step — while retaining the ability to change what has not yet been committed.

MissionDataType (ns=1;i=3068) carries the mission: a client-assigned MissionId, a monotonically increasing MissionUpdateId, a Label, and the ordered Steps. MissionType (ns=1;i=1004) is the instance that tracks one, and like IntentOperationType it is a Part 10 program instance for the reasons §6.1 gives.

Missions are optional. IntentCapabilitiesType.MissionsSupported declares whether a Server implements them.

### Base and horizon

Every step carries Released. The released steps form a prefix called the base; the rest form the horizon.

Figure 8 – A mission base and horizon

The base is committed and immutable. A Server shall assume every released step is executing or already executed, and shall refuse any update that would alter, remove or reorder one.

The horizon is provisional. UpdateMission replaces it wholly and may release some or all of its steps in doing so, extending the base.

ReleasedStepCount on the mission instance states how many steps are in the base, so a client does not have to scan the array to find the boundary.

A Server shall enforce all of the following, and shall apply an update atomically — an update either takes effect entirely or not at all:

- MissionUpdateId shall be strictly greater than the mission's current value. An update that is not is refused with Outdated. This is what makes two updates that crossed in flight safe: the later one wins and the earlier one is rejected rather than applied out of order.

- An update that would alter a released step is refused with BaseConflict.

- SequenceId shall ascend across the steps of a mission, and StepId shall be unique within it.

### Execution

A mission executes its steps in ascending SequenceId. Each step, when it begins, gets an IntentOperationType instance, and MissionStepDataType.Operation references it.

MissionStepDataType.Status is a hint. Where Operation is not null, that operation's state machine decides, and Status shall reflect it. Two members can report the state of one step; this sentence says which one is right.

A step that terminates Failed shall cause the mission to terminate Failed without beginning any later step, unless the step declares an error policy that says otherwise (6.15.4).

CancelMission cancels the mission and every intent belonging to it, subject to the same right of refusal as CancelIntent.

### The step graph

A mission may carry Transitions, an array of MissionTransitionDataType (ns=1;i=3083). Where it is empty, the mission is the ordered sequence 6.15.3 describes and nothing else applies — which is what makes the graph an addition rather than a replacement.

The model is the step-and-transition form of an IEC 61131-3 sequential function chart, chosen over a behaviour tree because it is the notation the audience implementing this already knows, it has an IEC serialization, and it needs no tick loop.

Each transition carries FromStepId, ToStepId, a Condition and a DivergenceKind.

Condition is an OPC UA ContentFilter — the base specification's own filter grammar, reused rather than invented. A specification that defined its own expression language would oblige every implementer to write a parser for it, and would then have to say what happens when two parsers disagree. An empty filter is always true.

DivergenceKind says how the transitions leaving one step relate:

- Alternative — exactly one is taken, the first whose Condition holds, in Transitions order. A Server shall evaluate them in that order so that two clients reading the same mission predict the same branch.

- Parallel — all are taken, and the branches execute concurrently. A Server shall report MissionBranchingSupported false if it cannot run branches concurrently, rather than silently serialising them.

Where a step has outgoing transitions and none of their conditions holds, the mission shall terminate Failed

with NoTransition. A Server shall report the same outcome if the transition selected for execution no longer

resolves to a step of the mission. It shall not report Succeeded, because Succeeded means the mission ran as

requested and would falsely state that unexecuted work was complete.

A Server shall refuse a mission whose transitions name a FromStepId or ToStepId that is not a step of that mission, and one that mixes Alternative and Parallel on transitions leaving the same step.

Error policies. MissionStepDataType.ErrorPolicy says what happens when the step does not succeed:

Table 8 – The step graph

| Policy | Behaviour |
| --- | --- |
| Abort | End the mission. The default, and what a mission without policies does. |
| Retry | Re-attempt the step. The Server bounds the attempts and reports Failed when they are exhausted. |
| Skip | Record the failure and continue with the next step. |
| Fallback | Continue at FallbackStepId. |
| Compensate | Run FallbackStepId to undo what has been done, then end the mission. |

A Server shall refuse a mission where ErrorPolicy is Fallback or Compensate and FallbackStepId does not name a step of that mission. Compensate differs from Fallback in what happens afterwards, and only in that: both run the fallback step, but Compensate then ends the mission rather than continuing.

Where a step's policy is honoured, the mission does not terminate Failed even though the step did — which is the whole reason the policies exist, and why 6.15.3's rule is stated as a default rather than an invariant.

## Command authority

At most one Session at a time holds command authority over a controller, and only that Session may submit intents or missions. ControlOwner reports which.

RequestControl grants authority when no other Session holds it, or when the holder's Session has closed. A Server shall release authority automatically when the holding Session closes, so that a crashed client does not lock a robot permanently. ReleaseControl gives it up explicitly; outstanding intents are unaffected, and a client that wants them stopped calls CancelAll first.

Reading, browsing and subscribing require no authority. Observation is always permitted.

NOTE   Command authority arbitrates between OPC UA clients. It is not the single point of control required by ISO 10218-2, which concerns the mutual exclusion of remote command and local manual control and is enforced by safety-rated means outside this interface. A Server shall not present command authority as satisfying that requirement.

## Capabilities and discovery

IntentCapabilitiesType is what makes an intent surface self-describing. A conformant Server shall populate it to reflect what it will actually accept — it is a contract, not documentation.

SupportedIntents carries one IntentCapabilityDataType per accepted intent type, naming the DataType and declaring whether cancel, pause and retry are honoured for it, which buffer and blocking modes it accepts, and which named Attributes this Server recognises. It is intentionally at intent-type granularity; it does not define per-member capabilities. The member-level scope of FastenIntentDataType.Joint is therefore structural: a client tells whether non-null Joint values are meaningful by browsing for the OPC 40450 / OPC 40451 joining or tightening model under the controller.

Four rules keep the declaration honest, and each is checkable against a running Server:

- A Server shall refuse an intent whose DataType is not listed, with CapabilityNotSupported.

- Every entry's SupportedBufferModes shall include Aborting.

- BlendingSupported shall be false unless the blending buffer modes actually blend.

- PauseSupported shall be false unless Pause actually suspends a running intent, per 6.14.6. Stopping only the queue is not suspending execution, and Suspended shall not be reported while the robot is still moving.

A fifth rule applies the same honesty to the Method surface, because a declaration a client cannot act on is worse than no declaration. Where a Server declares a capability, the Methods that make it usable shall be present on the controller and callable:

Table 9 – Capabilities and discovery

| Declaration | Methods or members that shall be present |
| --- | --- |
| MissionsSupported true | SubmitMission, CancelMission |
| MissionsSupported true | ActiveMission |
| MissionHorizonSupported true | UpdateMission |
| RealTimeChannelsSupported true | OpenRealTimeChannel, CloseRealTimeChannel |
| a capability entry with PauseSupported true | Pause, Resume |
| a capability entry with RetrySupported true | Retry |

These Methods are Optional on IntentControllerType, which is what makes the rule necessary: a Server can otherwise advertise missions while omitting SubmitMission entirely, and a client discovers the contradiction only by calling something that is not there. Like the others, this is observable against a running Server — browse the controller and compare what it offers with what it claims.

AxisCount states how many entries JointMoveIntentDataType.JointTargets must carry, and shall equal the number of AxisType instances under the controller.

Five further declarations cover the capability added beyond single moves. TrajectorySupported and ForceControlSupported say whether trajectories and force-controlled moves are accepted; RealTimeChannelsSupported whether channels are brokered; MissionBranchingSupported whether Transitions are evaluated at all; and MaxTrajectoryPoints bounds a trajectory, zero meaning the Server states no limit.

Each follows the same rule as BlendingSupported: a Server declares false rather than accepting work it will not actually perform. A Server that reports MissionBranchingSupported false executes the steps in order and ignores any transitions supplied, and a client reading that declaration knows not to express a branch it needs.

SupportedFacets is the same contract at the level of whole facets, and it is bound by every rule above. A facet is not a summary of the declaration a client has already read: some of what Table 12.2 requires — that blending modes are honoured, that the refusal rules of 6.14.2 are followed, that a mission base is immutable — cannot be established by reading the address space at all. Listing such a facet is therefore an attestation, and a Server that lists RI-Blending while treating the buffer modes as Buffered has made a false statement of exactly the kind rule 3 forbids, whatever BlendingSupported says. Clause 12.2 sets out which requirements are structural and which are attested.

## Safety

### This interface is not safety-rated

This specification defines a non-safety-rated application interface. The Methods defined here are processed by the robot controller as application-level requests. They do not constitute, and shall not be used as, safety functions as defined in IEC 61508, nor safety communication as defined in IEC 61784-3 or IEC 62541-15.

The safety functions of the robot system — emergency stop, protective stop, speed and separation monitoring, force limiting, and enabling device control — are implemented in safety-rated hardware and firmware independent of this interface, and shall remain effective regardless of its state, including when the Server is unreachable, the Session has closed, or a client is submitting intents as fast as the Server will accept them.

A Server shall not claim, and a client shall not assume, any safety integrity level or performance level for any part of this model.

This is a property of the technology, not a choice this working group made. OPC 10000-15 carries cyclic safety data from a SafetyProvider to a SafetyConsumer; the consumer's request carries an identifier, a monitoring number and one octet of explicitly non-safety flags, so a caller has no channel through which to supply safety-rated arguments. Every safety fieldbus — PROFIsafe, CIP Safety, FSoE, openSAFETY — expresses a safety command as a continuously asserted cyclic signal, because the integrity argument rests on the fail-safe state that follows when assertion stops. A Method call has no defined behaviour when it stops being called, and therefore cannot be a safety function however it is labelled.

A specification that ignored this would not be merely wrong; it would encourage an integrator to rely on something that fails silently. 6.18.4 says what can honestly be done instead.

### Operational mode gates submission

A Server shall refuse SubmitIntent and SubmitMission with NotPermittedInMode unless OperationalMode is Automatic or AutomaticExternal.

OperationalMode is read-only. This specification defines no way to command a mode change, because mode selection is a safety function performed by safety-rated means — a key switch, an interlock — and an interface that could change it from the network would defeat the arrangement it is reporting.

Where a person may be within the safeguarded space, the applicable safety standard, not this interface, decides what motion is permitted. A Server shall not rely on a client to observe such limits.

### A stop request is not a stop category

StopMode expresses urgency. It shall not be interpreted as selecting, implying or guaranteeing any IEC 60204-1 stop category. Category 0, 1 and 2 stops are determined solely by the robot controller's safety system in response to the active operational mode and the risk assessment for the installation.

A client that requires a category-rated stop shall obtain it from the safety system. It cannot be obtained here.

### Safety awareness, and its limits

What this specification can do is see what the safety system is enforcing and refuse work that would contradict it. SafetyStateType (6.12.3) reports it, and the following are normative.

A Server shall refuse SubmitIntent and SubmitMission:

- with SafetyLimitExceeded, when SafeSpeedLimitActive is true and the intent's Constraints.CartesianSpeed exceeds SafeSpeedLimit;

- with NotPermittedInMode, when EmergencyStopActive or ProtectiveStopActive is true;

- with NotPermittedInMode, when SafetyControllerOk is false.

A Server shall publish ActiveFunction, EmergencyStopActive, ProtectiveStopActive, SafeSpeedLimitActive, SafeSpeedLimit and SafetyControllerOk as the safety system reports them, and shall not publish a value the safety system has not asserted.

Each of these is observable against a running Server: assert a protective stop and a conformant Server refuses; lower SafeSpeedLimit below a submitted speed and it refuses.

What none of this makes true. These refusals are an application-layer courtesy performed by non-safety-rated software. They reduce the number of requests the safety system has to reject; they are not a protective measure and nothing may be assumed from their presence. In particular:

- a client shall not treat a Server's acceptance of an intent as evidence that the motion is safe;

- a client shall not treat SafeSpeedLimit as a limit this interface enforces — the safety system enforces it, and would enforce it identically if this model did not exist;

- a Server shall not offer any Method that commands a safe motion function, changes an operational mode, or clears a stop. Those are safety functions, and clause 10.1 says why no Method here can be one.

The asymmetry is deliberate: this model may observe the safety system and refuse on what it sees, and may never instruct it.

### Refusal is a normal outcome

Every rule above produces a refusal rather than a degraded execution, and each is observable against a running Server: a conformant Server can be seen to refuse a submission outside Automatic mode, and to refuse one from a Session that does not hold authority. A client shall treat refusal as an expected outcome and not as an error condition to be retried blindly.

## Security

### Commanding is a privileged operation

Every Method in this model moves a machine that can injure people and destroy property. A Server shall require an authenticated Session and should restrict the Methods of IntentControllerType by Role, distinctly from read access to the same address space. Observing a robot and commanding one are different privileges and shall not be conflated.

A Server should apply UserExecutable to the Methods it exposes so that a client discovers what it is permitted to invoke before invoking it.

### Command authority is not authorisation

Command authority (clause 8) prevents two authorised clients from interleaving motion. It grants nothing. A Server shall apply its access control independently: a Session that holds authority but lacks the necessary Role shall still be refused.

### NodeIds in intents are untrusted input

PickIntentDataType.Source, SetOutputIntentDataType.Output, CallProgramIntentDataType.Program and the other NodeId-valued members carry references chosen by the client. A Server shall validate that each resolves to a node of the expected type under the controller being commanded, and shall refuse with ParameterInvalid otherwise. A NodeId that resolves to a node belonging to a different controller, or to no node at all, shall not be acted on.

The expected type is fixed for every such member, so that "the expected type" is a checkable statement rather than an instruction to guess:

Table 10 – NodeIds in intents are untrusted input

| Member | Resolves to |
| --- | --- |
| PickIntentDataType.Source, PlaceIntentDataType.Destination, PalletiseIntentDataType.Pattern | a LocationType instance under the controller |
| MotionIntentDataType.ToolFrame, ForceIntentDataType.FrameId | a CoordinateFrameType instance under the controller; ToolFrame additionally of Role Tool |
| ToolChangeIntentDataType.Tool | a ToolType instance under the controller, or null to release the fitted tool |
| SetOutputIntentDataType.Output | an OutputSignalType instance under the controller; Value shall match that signal's own DataType |
| CallProgramIntentDataType.Program, ProcessIntentDataType.ProcessProgram | a ProgramType instance under the controller |
| WaitIntentDataType.Signal | an OutputSignalType instance under the controller, or a Variable of DataType Boolean under it |
| FastenIntentDataType.Joint | a joint in an OPC 40450 / OPC 40451 model under the controller where one is implemented; otherwise the intent's own parameters stand alone and the member is null |

CallProgramIntentDataType deserves particular care: it runs code the Server holds. A Server shall restrict it to programs it has published as ProgramType instances, and shall not accept a program identifier that names anything else.

For FastenIntentDataType.Joint, absence of an OPC 40450 / OPC 40451 joining or tightening model under the controller is itself the discoverable structural statement that non-null Joint values are not supported. A Server in that case shall refuse a non-null Joint with CapabilityNotSupported. Where such a model is exposed, a Joint that does not resolve to a joint in that model is malformed input and shall be refused with ParameterInvalid.

### Cybersecurity is in scope of the safety case

ISO 10218-1 addresses cybersecurity where a vulnerability could compromise robot safety. A networked command interface is exactly such a surface. The measures above are therefore not merely good practice; where this interface is deployed on a robot subject to that standard, they form part of the case that the installation is safe.

## Deliverables and reproducibility

Table 11 – Deliverables and reproducibility

| Artifact | Path |
| --- | --- |
| This specification | metaverse-specs/robot-intent/OPC-UA-Robot-Intent.md |
| Research report | metaverse-specs/robot-intent/OPC-UA-Robot-Intent-Research.md |
| Information model | metaverse-specs/robot-intent/Opc.Ua.RobotIntent.NodeSet2.xml |
| NodeId assignments | metaverse-specs/robot-intent/Opc.Ua.RobotIntent.NodeIds.csv |
| Generator | metaverse-specs/extras/robot-intent/tools/build_model.py |
| Validator | metaverse-specs/extras/robot-intent/tools/validate_local.py |
| Annex A (generated) | metaverse-specs/extras/robot-intent/tools/model-reference.md |

The NodeSet, the CSV and Annex A are generated from a single in-code source of truth and are deterministic: regenerating reproduces them byte for byte. The generator is edited; the generated files are not.

python metaverse-specs\extras\robot-intent\tools\build_model.py

python metaverse-specs\extras\robot-intent\tools\validate_local.py

# OPC UA ObjectTypes

## RobotIntentRootType

Server-level entry point. A client that has just connected browses here to find every robot it can command, without knowing the Server's layout.

Table 12 – RobotIntentRootType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:RobotIntentRootType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:BaseObjectType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasComponent | Object | Controllers |  | 0:FolderType | M |
| 0:HasProperty | Variable | SpecificationVersion | 0:String | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent |  |  |  |  |  |

## IntentControllerType

The intent surface for one robot: what it can be asked to do, the frames and objects those requests refer to, and the intents and missions currently outstanding. Everything a client needs in order to command the robot hangs from here.

Table 13 – IntentControllerType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:IntentControllerType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:BaseObjectType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | OperationalMode | OperationalModeEnum | 0:PropertyType | M |
| 0:HasProperty | Variable | Ready | 0:Boolean | 0:PropertyType | M |
| 0:HasProperty | Variable | ControlOwner | 0:NodeId | 0:PropertyType | M |
| 0:HasProperty | Variable | MaxQueueDepth | 0:UInt32 | 0:PropertyType | M |
| 0:HasComponent | Variable | ActiveIntent | 0:NodeId | 0:BaseDataVariableType | M |
| 0:HasComponent | Variable | ActiveMission | 0:NodeId | 0:BaseDataVariableType | O |
| 0:HasComponent | Object | Capabilities |  | IntentCapabilitiesType | M |
| 0:HasComponent | Object | Frames |  | 0:FolderType | M |
| 0:HasComponent | Object | Tools |  | 0:FolderType | M |
| 0:HasComponent | Object | Locations |  | 0:FolderType | M |
| 0:HasComponent | Object | Axes |  | 0:FolderType | M |
| 0:HasComponent | Object | Outputs |  | 0:FolderType | O |
| 0:HasComponent | Object | Programs |  | 0:FolderType | O |
| 0:HasComponent | Object | Intents |  | 0:FolderType | M |
| 0:HasComponent | Object | Missions |  | 0:FolderType | O |
| 0:HasComponent | Method | RequestControl |  |  | M |
| 0:HasComponent | Method | ReleaseControl |  |  | M |
| 0:HasComponent | Method | SubmitIntent |  |  | M |
| 0:HasComponent | Method | CancelIntent |  |  | M |
| 0:HasComponent | Method | CancelAll |  |  | M |
| 0:HasComponent | Method | Pause |  |  | O |
| 0:HasComponent | Method | Resume |  |  | O |
| 0:HasComponent | Method | Retry |  |  | O |
| 0:HasComponent | Method | SubmitMission |  |  | O |
| 0:HasComponent | Method | UpdateMission |  |  | O |
| 0:HasComponent | Method | CancelMission |  |  | O |
| 0:HasComponent | Object | SafetyState |  | SafetyStateType | M |
| 0:HasComponent | Object | Description |  | RobotDescriptionType | O |
| 0:HasComponent | Object | RealTimeChannels |  | 0:FolderType | O |
| 0:HasComponent | Method | OpenRealTimeChannel |  |  | O |
| 0:HasComponent | Method | CloseRealTimeChannel |  |  | O |
| Conformance Units |  |  |  |  |  |
| RobotIntent |  |  |  |  |  |

### RequestControl

Take command authority. A Server grants it only when no other Session holds it, or when the holder's Session has closed. Holding authority is a precondition for submitting, and exists so that two clients cannot interleave motion; it is NOT the single point of control that ISO 10218-2 requires, which is enforced by safety-rated means outside this interface.

Signature

RequestControl (

[out] 0:Boolean      Granted,

[out] 0:NodeId       CurrentOwner);

Table 14 – RequestControl Method Arguments

| Argument | Description |
| --- | --- |
| Granted | True when the caller now holds authority. |
| CurrentOwner | SessionId of the holder after the call. |

Table 15 – RequestControl Method AddressSpace definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | RequestControl |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | ModellingRule |
| 0:HasProperty | Variable | 0:OutputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |

### ReleaseControl

Give up command authority. Outstanding intents are unaffected; use CancelAll to stop them.

Signature

ReleaseControl ();

### SubmitIntent

Submit one intent. Returns as soon as the intent is admitted - NOT when the robot has finished, which may be minutes later. The returned Operation is a node the client subscribes to for progress and reads for the result. This is the whole reason the model is built on the Part 10 program lifecycle: an OPC UA Call cannot stay open for the duration of a motion.

Signature

SubmitIntent (

[in]  IntentDataType Intent,

[out] 0:Boolean      Accepted,

[out] 0:String       IntentId,

[out] 0:NodeId       Operation,

[out] IntentFailureEnum Failure,

[out] 0:LocalizedText Message);

Table 16 – SubmitIntent Method Arguments

| Argument | Description |
| --- | --- |
| Intent | The intent to execute. |
| Accepted | True when the intent was admitted. False is a refusal, which clause 10.5 makes an ordinary outcome rather than an error. |
| IntentId | Identifier of the intent, assigned by the Server when the request left it empty. Empty on a refusal. |
| Operation | The IntentOperation that tracks it. Null on a refusal, because a refusal creates no operation instance. |
| Failure | Why the intent was refused, or None when it was admitted. |
| Message | Human-readable detail on a refusal. For a human; never parsed. |

Table 17 – SubmitIntent Method AddressSpace definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | SubmitIntent |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | ModellingRule |
| 0:HasProperty | Variable | 0:InputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |
| 0:HasProperty | Variable | 0:OutputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |

### CancelIntent

Ask the Server to end an intent early. The Server MAY refuse, and says so in Accepted, because some motions cannot be abandoned safely part-way. This is not the OPC UA Cancel Service, which discards a pending response and leaves the robot moving; see clause 6.5.

Signature

CancelIntent (

[in]  0:String       IntentId,

[in]  StopModeEnum   StopMode,

[out] 0:Boolean      Accepted);

Table 18 – CancelIntent Method Arguments

| Argument | Description |
| --- | --- |
| IntentId | The intent to cancel. |
| StopMode | How urgently to stop. |
| Accepted | True when the Server will act on it. |

Table 19 – CancelIntent Method AddressSpace definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | CancelIntent |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | ModellingRule |
| 0:HasProperty | Variable | 0:InputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |
| 0:HasProperty | Variable | 0:OutputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |

### CancelAll

Ask the Server to end every outstanding intent and mission.

Signature

CancelAll (

[in]  StopModeEnum   StopMode,

[out] 0:UInt32       Cancelled);

Table 20 – CancelAll Method Arguments

| Argument | Description |
| --- | --- |
| StopMode | How urgently to stop. |
| Cancelled | How many were acted on. |

Table 21 – CancelAll Method AddressSpace definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | CancelAll |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | ModellingRule |
| 0:HasProperty | Variable | 0:InputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |
| 0:HasProperty | Variable | 0:OutputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |

### Pause

Suspend execution, retaining position so it can be resumed.

Signature

Pause (

[out] 0:Boolean      Accepted);

Table 22 – Pause Method Arguments

| Argument | Description |
| --- | --- |
| Accepted | True when execution is suspending. |

Table 23 – Pause Method AddressSpace definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | Pause |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | ModellingRule |
| 0:HasProperty | Variable | 0:OutputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |

### Resume

Continue execution suspended by Pause.

Signature

Resume (

[out] 0:Boolean      Accepted);

Table 24 – Resume Method Arguments

| Argument | Description |
| --- | --- |
| Accepted | True when execution is resuming. |

Table 25 – Resume Method AddressSpace definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | Resume |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | ModellingRule |
| 0:HasProperty | Variable | 0:OutputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |

### Retry

Re-attempt an intent that terminated Retriable.

Signature

Retry (

[in]  0:String       IntentId,

[out] 0:Boolean      Accepted,

[out] 0:NodeId       Operation,

[out] IntentFailureEnum Failure,

[out] 0:LocalizedText Message);

Table 26 – Retry Method Arguments

| Argument | Description |
| --- | --- |
| IntentId | The intent to re-attempt. |
| Accepted | True when a new attempt was admitted. |
| Operation | The IntentOperation that tracks the new attempt. Null on a refusal. |
| Failure | Why the re-attempt was refused, or None when it was admitted. |
| Message | Human-readable detail on a refusal. |

Table 27 – Retry Method AddressSpace definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | Retry |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | ModellingRule |
| 0:HasProperty | Variable | 0:InputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |
| 0:HasProperty | Variable | 0:OutputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |

### SubmitMission

Submit an ordered sequence of intents as one unit. Steps marked Released form the base and are committed; the rest form the horizon and may still be revised by UpdateMission.

Signature

SubmitMission (

[in]  MissionDataType Mission,

[out] 0:Boolean      Accepted,

[out] 0:String       MissionId,

[out] 0:NodeId       Operation,

[out] IntentFailureEnum Failure,

[out] 0:LocalizedText Message);

Table 28 – SubmitMission Method Arguments

| Argument | Description |
| --- | --- |
| Mission | The mission to execute. |
| Accepted | True when the mission was admitted. |
| MissionId | Identifier of the mission, assigned by the Server when the request left it empty. Empty on a refusal. |
| Operation | The Mission that tracks it. Null on a refusal. |
| Failure | Why the mission was refused, or None when it was admitted. |
| Message | Human-readable detail on a refusal. |

Table 29 – SubmitMission Method AddressSpace definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | SubmitMission |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | ModellingRule |
| 0:HasProperty | Variable | 0:InputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |
| 0:HasProperty | Variable | 0:OutputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |

### UpdateMission

Replace the horizon of a mission already submitted. The base is untouchable: it has been committed and may already have executed, so an update that would alter a released step is refused rather than partly applied.

Signature

UpdateMission (

[in]  0:String       MissionId,

[in]  0:UInt32       MissionUpdateId,

[in]  MissionStepDataType[] Steps,

[out] MissionUpdateResultEnum Result,

[out] 0:LocalizedText Message);

Table 30 – UpdateMission Method Arguments

| Argument | Description |
| --- | --- |
| MissionId | The mission to update. |
| MissionUpdateId | Revision of the update. Must be greater than the mission's current value. |
| Steps | The steps that replace the horizon. |
| Result | Outcome of the update. |
| Message | Human-readable detail on a refusal. |

Table 31 – UpdateMission Method AddressSpace definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | UpdateMission |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | ModellingRule |
| 0:HasProperty | Variable | 0:InputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |
| 0:HasProperty | Variable | 0:OutputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |

### CancelMission

Ask the Server to end a mission and every intent belonging to it.

Signature

CancelMission (

[in]  0:String       MissionId,

[in]  StopModeEnum   StopMode,

[out] 0:Boolean      Accepted);

Table 32 – CancelMission Method Arguments

| Argument | Description |
| --- | --- |
| MissionId | The mission to cancel. |
| StopMode | How urgently to stop. |
| Accepted | True when the Server will act on it. |

Table 33 – CancelMission Method AddressSpace definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | CancelMission |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | ModellingRule |
| 0:HasProperty | Variable | 0:InputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |
| 0:HasProperty | Variable | 0:OutputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |

### OpenRealTimeChannel

Take a lease on a brokered real-time channel. The Server prepares the transport and returns what the client needs in order to connect; it does not carry the samples. A lease that is not renewed lapses, which is what stops a dead client from holding the channel.

Signature

OpenRealTimeChannel (

[in]  0:String       ChannelId,

[in]  0:Duration     RequestedLease,

[out] 0:Boolean      Granted,

[out] 0:String       EndpointUrl,

[out] 0:String       PayloadDescriptor,

[out] 0:UtcTime      LeaseExpiry,

[out] 0:LocalizedText Message);

Table 34 – OpenRealTimeChannel Method Arguments

| Argument | Description |
| --- | --- |
| ChannelId | The channel to open. |
| RequestedLease | How long the lease is wanted for, in milliseconds. |
| Granted | True when the lease was taken. |
| EndpointUrl | Where to connect. |
| PayloadDescriptor | The transport's own configuration. |
| LeaseExpiry | When the lease lapses. |
| Message | Human-readable detail on a refusal. For a human; never parsed. |

Table 35 – OpenRealTimeChannel Method AddressSpace definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | OpenRealTimeChannel |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | ModellingRule |
| 0:HasProperty | Variable | 0:InputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |
| 0:HasProperty | Variable | 0:OutputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |

### CloseRealTimeChannel

Give up a lease on a brokered channel.

Signature

CloseRealTimeChannel (

[in]  0:String       ChannelId,

[out] 0:Boolean      Released);

Table 36 – CloseRealTimeChannel Method Arguments

| Argument | Description |
| --- | --- |
| ChannelId | The channel to release. |
| Released | True when the lease was held and is now released. |

Table 37 – CloseRealTimeChannel Method AddressSpace definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | CloseRealTimeChannel |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | ModellingRule |
| 0:HasProperty | Variable | 0:InputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |
| 0:HasProperty | Variable | 0:OutputArguments | 0:Argument[] | 0:PropertyType | 0:Mandatory |

## IntentOperationType

One submitted intent, tracked to completion. It is a Part 10 program instance, so its lifecycle is the one OPC UA already defines for work that outlives a service call: transitions raise ProgramTransitionEvents, the terminal result survives in FinalResultData, and ProgramDiagnostic2DataType records which Session commanded it without this specification having to model provenance itself.

Table 38 – IntentOperationType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:IntentOperationType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:ProgramStateMachineType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | IntentId | 0:String | 0:PropertyType | M |
| 0:HasComponent | Variable | Intent | IntentDataType | 0:BaseDataVariableType | M |
| 0:HasProperty | Variable | ExecutionState | ExecutionStateEnum | 0:PropertyType | M |
| 0:HasProperty | Variable | Progress | 0:Double | 0:PropertyType | O |
| 0:HasComponent | Variable | CurrentPose | Pose3DDataType | 0:BaseDataVariableType | O |
| 0:HasComponent | Variable | Result | IntentResultDataType | 0:BaseDataVariableType | M |
| 0:HasProperty | Variable | MissionId | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | QueuePosition | 0:UInt32 | 0:PropertyType | O |
| 0:HasComponent | Object | FinalResultData |  | 0:BaseObjectType | M |
| 0:HasComponent | Variable | ProgramDiagnostic | 0:ProgramDiagnostic2DataType | 0:ProgramDiagnostic2Type | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent |  |  |  |  |  |

## MissionType

One submitted mission, tracked to completion. It is a Part 10 program instance for the same reasons an IntentOperation is, and it owns the IntentOperations of its steps.

Table 39 – MissionType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:MissionType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:ProgramStateMachineType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | MissionId | 0:String | 0:PropertyType | M |
| 0:HasProperty | Variable | MissionUpdateId | 0:UInt32 | 0:PropertyType | M |
| 0:HasComponent | Variable | Mission | MissionDataType | 0:BaseDataVariableType | M |
| 0:HasProperty | Variable | ExecutionState | ExecutionStateEnum | 0:PropertyType | M |
| 0:HasProperty | Variable | CurrentStepId | 0:String | 0:PropertyType | M |
| 0:HasProperty | Variable | ReleasedStepCount | 0:UInt32 | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent |  |  |  |  |  |

## IntentCapabilitiesType

What one robot will accept. A client reads this once, before it submits anything, and knows what the robot can do and under what constraints.

Table 40 – IntentCapabilitiesType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:IntentCapabilitiesType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:BaseObjectType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasComponent | Variable | SupportedIntents | IntentCapabilityDataType[] | 0:BaseDataVariableType | M |
| 0:HasProperty | Variable | MissionsSupported | 0:Boolean | 0:PropertyType | M |
| 0:HasProperty | Variable | MissionHorizonSupported | 0:Boolean | 0:PropertyType | M |
| 0:HasProperty | Variable | BlendingSupported | 0:Boolean | 0:PropertyType | M |
| 0:HasProperty | Variable | MaxBlendRadius | 0:Double | 0:PropertyType | O |
| 0:HasProperty | Variable | AxisCount | 0:UInt32 | 0:PropertyType | M |
| 0:HasProperty | Variable | TrajectorySupported | 0:Boolean | 0:PropertyType | M |
| 0:HasProperty | Variable | ForceControlSupported | 0:Boolean | 0:PropertyType | M |
| 0:HasProperty | Variable | RealTimeChannelsSupported | 0:Boolean | 0:PropertyType | M |
| 0:HasProperty | Variable | MissionBranchingSupported | 0:Boolean | 0:PropertyType | M |
| 0:HasProperty | Variable | MaxTrajectoryPoints | 0:UInt32 | 0:PropertyType | O |
| 0:HasProperty | Variable | SupportedFacets | 0:String[] | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent |  |  |  |  |  |

## CoordinateFrameType

A named right-handed Cartesian frame. Frames form a tree through HasFrameParent, so a client can compose a chain from a tool frame to a world frame. Roles follow ISO 9787, which standardises which frames exist; the transform between them is carried explicitly because no standard says how to calibrate it.

Table 41 – CoordinateFrameType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:CoordinateFrameType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:BaseObjectType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | FrameId | 0:String | 0:PropertyType | M |
| 0:HasProperty | Variable | Role | FrameRoleEnum | 0:PropertyType | M |
| 0:HasComponent | Variable | Transform | Pose3DDataType | 0:BaseDataVariableType | O |
| Conformance Units |  |  |  |  |  |
| RobotIntent |  |  |  |  |  |

## ToolType

An end effector. Its tool centre point is a CoordinateFrame of role Tool, which is what a motion intent drives to a target - OPC 40010-1 models robot topology in detail but has no tool centre point at all, so this specification supplies one.

Table 42 – ToolType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:ToolType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:BaseObjectType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | ToolId | 0:String | 0:PropertyType | M |
| 0:HasProperty | Variable | Name | 0:LocalizedText | 0:PropertyType | M |
| 0:HasProperty | Variable | Fitted | 0:Boolean | 0:PropertyType | M |
| 0:HasProperty | Variable | TcpFrame | 0:NodeId | 0:PropertyType | M |
| 0:HasProperty | Variable | Mass | 0:Double | 0:PropertyType | O |
| 0:HasProperty | Variable | MaxGraspForce | 0:Double | 0:PropertyType | O |
| 0:HasProperty | Variable | MaxOpening | 0:Double | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| RobotIntent |  |  |  |  |  |

## LocationType

A named place an intent can refer to. Pick and Place reference these nodes rather than naming a station in a string, so a location has one definition that a client can read, subscribe to and reason about.

Table 43 – LocationType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:LocationType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:BaseObjectType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | LocationId | 0:String | 0:PropertyType | M |
| 0:HasProperty | Variable | Name | 0:LocalizedText | 0:PropertyType | M |
| 0:HasComponent | Variable | Pose | Pose3DDataType | 0:BaseDataVariableType | M |
| 0:HasProperty | Variable | Occupied | 0:Boolean | 0:PropertyType | O |
| 0:HasProperty | Variable | ObjectClass | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | Capacity | 0:UInt32 | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| RobotIntent |  |  |  |  |  |

## AxisType

One axis of the robot. The order of these nodes under the controller's Axes folder fixes the order of JointMoveIntentDataType.JointTargets, and Kind fixes each entry's unit.

Table 44 – AxisType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:AxisType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:BaseObjectType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | AxisId | 0:String | 0:PropertyType | M |
| 0:HasProperty | Variable | Index | 0:UInt32 | 0:PropertyType | M |
| 0:HasProperty | Variable | Kind | AxisKindEnum | 0:PropertyType | M |
| 0:HasProperty | Variable | MinPosition | 0:Double | 0:PropertyType | M |
| 0:HasProperty | Variable | MaxPosition | 0:Double | 0:PropertyType | M |
| 0:HasProperty | Variable | MaxSpeed | 0:Double | 0:PropertyType | O |
| 0:HasComponent | Variable | Position | 0:Double | 0:BaseDataVariableType | O |
| Conformance Units |  |  |  |  |  |
| RobotIntent |  |  |  |  |  |

## OutputSignalType

A signal SetOutput can write. Modelling it as a node means the range, the unit and the meaning are described once, instead of every client having to know what a line name implies.

Table 45 – OutputSignalType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:OutputSignalType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:BaseObjectType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | SignalId | 0:String | 0:PropertyType | M |
| 0:HasProperty | Variable | Name | 0:LocalizedText | 0:PropertyType | M |
| 0:HasComponent | Variable | Value | 0:BaseDataType | 0:BaseDataVariableType | M |
| 0:HasProperty | Variable | Writable | 0:Boolean | 0:PropertyType | M |
| 0:HasProperty | Variable | EngineeringUnits | 0:EUInformation | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| RobotIntent |  |  |  |  |  |

## ProgramType

A program held on the controller that CallProgram can run. This is the bridge to capability this specification does not model, and to the programs an OPC 40010-1 task control already exposes.

Table 46 – ProgramType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:ProgramType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:BaseObjectType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | ProgramId | 0:String | 0:PropertyType | M |
| 0:HasProperty | Variable | Name | 0:LocalizedText | 0:PropertyType | M |
| 0:HasProperty | Variable | Description | 0:LocalizedText | 0:PropertyType | O |
| 0:HasComponent | Variable | Parameters | 0:KeyValuePair[] | 0:BaseDataVariableType | O |
| Conformance Units |  |  |  |  |  |
| RobotIntent |  |  |  |  |  |

## SafetyStateType

What the robot's safety system is doing, reported so a client can act sensibly around it. Every member is READ-ONLY and every one is a report: the safety system enforces these independently, remains effective when this interface is unreachable, and is not commanded from here. Clause 10 states the boundary and this type does not cross it.

Table 47 – SafetyStateType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:SafetyStateType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:BaseObjectType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | ActiveFunction | SafeMotionFunctionEnum | 0:PropertyType | M |
| 0:HasProperty | Variable | EmergencyStopActive | 0:Boolean | 0:PropertyType | M |
| 0:HasProperty | Variable | ProtectiveStopActive | 0:Boolean | 0:PropertyType | M |
| 0:HasProperty | Variable | SafeSpeedLimitActive | 0:Boolean | 0:PropertyType | M |
| 0:HasProperty | Variable | SafeSpeedLimit | 0:Double | 0:PropertyType | M |
| 0:HasProperty | Variable | SafetyControllerOk | 0:Boolean | 0:PropertyType | M |
| 0:HasProperty | Variable | LastStopReason | 0:LocalizedText | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| RobotIntent |  |  |  |  |  |

## RealTimeChannelType

A high-rate channel this Server can offer, described so a client can open it. The samples never traverse OPC UA: this brokers the endpoint and nothing more, in the same way the Vision model brokers a media endpoint rather than carrying pixels. Clause 4.3 explains why the alternative is not available.

Table 48 – RealTimeChannelType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:RealTimeChannelType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:BaseObjectType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | ChannelId | 0:String | 0:PropertyType | M |
| 0:HasProperty | Variable | Transport | RealTimeTransportEnum | 0:PropertyType | M |
| 0:HasProperty | Variable | EndpointUrl | 0:String | 0:PropertyType | M |
| 0:HasProperty | Variable | Initiator | ChannelInitiatorEnum | 0:PropertyType | M |
| 0:HasProperty | Variable | NominalRate | 0:Double | 0:PropertyType | M |
| 0:HasProperty | Variable | PayloadDescriptor | 0:String | 0:PropertyType | O |
| 0:HasProperty | Variable | RequiredMode | OperationalModeEnum | 0:PropertyType | M |
| 0:HasProperty | Variable | Available | 0:Boolean | 0:PropertyType | M |
| 0:HasProperty | Variable | LeaseHolder | 0:NodeId | 0:PropertyType | M |
| 0:HasProperty | Variable | LeaseExpiry | 0:UtcTime | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| RobotIntent |  |  |  |  |  |

## RobotDescriptionType

Enough of the robot's construction for a client to plan against it without a second specification: the kinematic chain, the space it can reach, and what it can carry. OPC 40010-1 describes topology and axes and defines no kinematic chain and no tool centre point, so this adds rather than restates - Annex B says which side decides where both are present.

Table 49 – RobotDescriptionType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:RobotDescriptionType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:BaseObjectType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | Manufacturer | 0:LocalizedText | 0:PropertyType | O |
| 0:HasProperty | Variable | Model | 0:String | 0:PropertyType | O |
| 0:HasComponent | Variable | KinematicChain | KinematicJointDataType[] | 0:BaseDataVariableType | M |
| 0:HasComponent | Variable | MountingPose | Pose3DDataType | 0:BaseDataVariableType | O |
| 0:HasProperty | Variable | ReachRadius | 0:Double | 0:PropertyType | M |
| 0:HasProperty | Variable | PayloadLimit | 0:Double | 0:PropertyType | M |
| 0:HasProperty | Variable | MaxCartesianSpeed | 0:Double | 0:PropertyType | M |
| 0:HasProperty | Variable | MaxCartesianAcceleration | 0:Double | 0:PropertyType | O |
| Conformance Units |  |  |  |  |  |
| RobotIntent |  |  |  |  |  |

# OPC UA DataTypes

## ExecutionStateEnum

Fine-grained execution state of an intent or a mission. This REFINES the Part 10 program state machine rather than restating it: Queued, Cancelling and the three terminal outcomes cannot be told apart from CurrentState alone. Clause 6.3 fixes which ExecutionState may accompany which Part 10 state, and a Server shall satisfy that table.

Table 50 – ExecutionStateEnum Items

| Name | Value | Description |
| --- | --- | --- |
| Accepted | 0 | Admitted and validated, not yet queued or executing. |
| Queued | 1 | Waiting behind another intent because BufferMode is Buffered or a blending mode. Corresponds to PLCopen Busy without Active. |
| Executing | 2 | Commanding the robot now. |
| Suspended | 3 | Paused by request; position is retained and execution can resume. |
| Cancelling | 4 | A cancel was accepted and the Server is bringing the motion to a controlled end. Not yet terminal. |
| Succeeded | 5 | Terminal. Completed as requested. |
| Failed | 6 | Terminal. Did not complete; Failure carries the reason. |
| Cancelled | 7 | Terminal. Ended early because a cancel was accepted. |
| Retriable | 8 | Terminal for now, but the Server can re-attempt it on Retry. A Server that does not offer Retry never enters this state and reports Failed instead. |

Table 51 – ExecutionStateEnum definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:ExecutionStateEnum |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | EnumStrings | 0:LocalizedText[9] | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## BufferModeEnum

How a newly submitted intent relates to the one already executing. The values and their meanings are those of PLCopen Motion Control MC_BufferMode, adopted unchanged because every motion runtime already implements them. In all blending modes the robot does not decelerate to a stop at the boundary, and the predecessor reaches Succeeded when blending begins rather than when its target is exactly attained.

Table 52 – BufferModeEnum Items

| Name | Value | Description |
| --- | --- | --- |
| Aborting | 0 | Abort what is executing and start immediately. The aborted intent terminates as Cancelled. This is the default. |
| Buffered | 1 | Queue; start when the predecessor succeeds. |
| BlendingLow | 2 | Blend at the lower of the two boundary speeds. |
| BlendingPrevious | 3 | Blend at the predecessor's boundary speed. |
| BlendingNext | 4 | Blend at the successor's boundary speed. |
| BlendingHigh | 5 | Blend at the higher of the two boundary speeds. |

Table 53 – BufferModeEnum definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:BufferModeEnum |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | EnumStrings | 0:LocalizedText[6] | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## BlockingModeEnum

Whether an intent tolerates motion and other intents running alongside it. The four values are the two-by-two matrix of VDA 5050 blockingType, adopted because it is the only widely deployed concurrency annotation for robot actions.

Table 54 – BlockingModeEnum Items

| Name | Value | Description |
| --- | --- | --- |
| None | 0 | Runs in the background; motion may continue and other intents may run concurrently. |
| Soft | 1 | Motion stops for the duration; other intents may still run. |
| Single | 2 | Motion may continue; no other intent may run concurrently. |
| Hard | 3 | Motion stops and no other intent may run concurrently. The intent has exclusive use of the robot. |

Table 55 – BlockingModeEnum definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:BlockingModeEnum |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | EnumStrings | 0:LocalizedText[4] | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## TerminationModeEnum

Whether a motion ends exactly on its target or is blended into the next one. This is the only distinction every vendor expresses identically; the blend magnitude in BlendDataType.Radius is a request, not a guarantee.

Table 56 – TerminationModeEnum Items

| Name | Value | Description |
| --- | --- | --- |
| Exact | 0 | Come to rest on the target before the next motion begins. ABB fine, FANUC FINE, Yaskawa PL=0, KUKA no approximation. |
| Blend | 1 | Round the corner into the next motion without stopping. |

Table 57 – TerminationModeEnum definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:TerminationModeEnum |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | EnumStrings | 0:LocalizedText[2] | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## ReleaseModeEnum

How a held object is given up.

Table 58 – ReleaseModeEnum Items

| Name | Value | Description |
| --- | --- | --- |
| Drop | 0 | Open the end effector where it is, without placing. |
| Place | 1 | Set the object down under control at the target. |
| Handover | 2 | Retain the object until the Server judges a receiving party has taken it. |

Table 59 – ReleaseModeEnum definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:ReleaseModeEnum |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | EnumStrings | 0:LocalizedText[3] | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## ApproachModeEnum

Direction from which an end effector approaches an object or a placement.

Table 60 – ApproachModeEnum Items

| Name | Value | Description |
| --- | --- | --- |
| Default | 0 | The Server chooses. |
| ToolZ | 1 | Along the tool's own Z axis. |
| Top | 2 | From above, in the frame the target is expressed in. |
| Side | 3 | Laterally, in the frame the target is expressed in. |

Table 61 – ApproachModeEnum definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:ApproachModeEnum |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | EnumStrings | 0:LocalizedText[4] | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## FrameRoleEnum

Role of a coordinate frame, following the coordinate systems ISO 9787 standardises. The roles say WHICH frames exist; no standard says how to calibrate between them, which is why CoordinateFrameType carries the transform explicitly.

Table 62 – FrameRoleEnum Items

| Name | Value | Description |
| --- | --- | --- |
| World | 0 | The cell-level reference frame. |
| Base | 1 | The robot base frame. |
| MechanicalInterface | 2 | The flange at the end of the last link, to which an end effector is fitted. |
| Tool | 3 | A tool frame, whose origin is a tool centre point. |
| Object | 4 | A workpiece or work-object frame. |
| Other | 5 | A frame whose role is none of the above. |

Table 63 – FrameRoleEnum definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:FrameRoleEnum |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | EnumStrings | 0:LocalizedText[6] | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## OperationalModeEnum

Operational mode of the robot system, as defined by ISO 10218-1 and reported identically by OPC 40010-1. It is READ-ONLY here: mode selection is a safety function performed by safety-rated means, and this specification defines no way to command it. Clause 10 restricts intent submission to Automatic and AutomaticExternal.

Table 64 – OperationalModeEnum Items

| Name | Value | Description |
| --- | --- | --- |
| Other | 0 | Booting, uncalibrated, or a safety system fault. |
| ManualReducedSpeed | 1 | Teaching mode with a speed ceiling and a held enabling device. |
| ManualHighSpeed | 2 | Program verification with an enabling device. |
| Automatic | 3 | Automatic operation with the safeguarded space secured. |
| AutomaticExternal | 4 | Automatic operation commanded by an external system. This is the mode this specification is written for. |

Table 65 – OperationalModeEnum definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:OperationalModeEnum |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | EnumStrings | 0:LocalizedText[5] | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## IntentFailureEnum

Why an intent did not succeed. The set is deliberately small and diagnosable: a client decides whether to retry, re-plan or escalate from this value alone, and reads Message only to show a human.

Table 66 – IntentFailureEnum Items

| Name | Value | Description |
| --- | --- | --- |
| None | 0 | No failure. Reported on a successful outcome. |
| Unreachable | 1 | The target lies outside the reachable workspace. |
| Kinematics | 2 | No kinematic solution, or a singularity on the path. |
| Collision | 3 | A collision was predicted or detected. |
| JointLimit | 4 | A joint limit would be or was exceeded. |
| SpeedLimit | 5 | The requested speed or acceleration is not permitted in the active mode. |
| ToolMissing | 6 | The required tool is not fitted or not identified. |
| ObjectNotFound | 7 | The object to act on was not present. |
| GraspFailed | 8 | The object was not acquired, or was lost in transit. |
| Timeout | 9 | The intent did not complete within its permitted time. |
| NotPermittedInMode | 10 | Refused because the operational mode does not permit it. See clause 10. |
| ControlNotOwned | 11 | Refused because the caller does not hold command authority. See clause 8. |
| CapabilityNotSupported | 12 | The Server does not implement this intent type, or this combination of options. |
| ParameterInvalid | 13 | A parameter was missing, malformed or out of range. |
| QueueFull | 14 | The queue is at MaxQueueDepth. |
| Superseded | 15 | An Aborting submission or a mission update replaced it before it could run. |
| HardwareFault | 16 | A fault in the robot, the end effector or the controller. |
| SafetyStop | 17 | A safety function acted. The safety system, not this interface, decided this. |
| Other | 18 | A reason none of the above describes; see Message. |
| SafetyLimitExceeded | 19 | Refused because the request would exceed a limit the safety system is enforcing. See clause 10.3. |
| NoTransition | 20 | A mission branch point had no true outgoing transition, or the selected transition target did not resolve. See clause 7.4. |

Table 67 – IntentFailureEnum definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:IntentFailureEnum |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | EnumStrings | 0:LocalizedText[21] | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## StopModeEnum

How urgently a cancellation should bring motion to an end. The values are those of PossibleStopModes in OPC 40010-1, so a Server implementing both reports one vocabulary. This is an APPLICATION-LEVEL request: it does not select, imply or guarantee any IEC 60204-1 stop category, which only the robot's safety system determines. See clause 10.

Table 68 – StopModeEnum Items

| Name | Value | Description |
| --- | --- | --- |
| OnPath | 1 | Decelerate along the programmed path. |
| EndOfCycle | 2 | Stop when the current cycle completes. |
| ProcessStop | 3 | Stop at a point the process defines as safe. |
| QuickStop | 4 | Decelerate as quickly as the drives allow. |
| EndOfInstruction | 5 | Stop when the current instruction completes. |

Table 69 – StopModeEnum definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:StopModeEnum |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | EnumStrings | 0:LocalizedText[5] | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## AxisKindEnum

Whether an axis rotates or translates. This fixes the unit of the corresponding entry of JointMoveIntentDataType.JointTargets.

Table 70 – AxisKindEnum Items

| Name | Value | Description |
| --- | --- | --- |
| Revolute | 0 | Rotates. Its joint target is in radians. |
| Prismatic | 1 | Translates. Its joint target is in metres. |

Table 71 – AxisKindEnum definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:AxisKindEnum |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | EnumStrings | 0:LocalizedText[2] | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## MissionUpdateResultEnum

Outcome of a mission update, reported so a client can tell a stale update from a rejected one without parsing a StatusCode.

Table 72 – MissionUpdateResultEnum Items

| Name | Value | Description |
| --- | --- | --- |
| Accepted | 0 | The horizon was replaced as requested. |
| Outdated | 1 | MissionUpdateId was not greater than the current one. |
| BaseConflict | 2 | The update would have altered a released step. |
| UnknownMission | 3 | No mission with that MissionId is held. |
| Rejected | 4 | Refused for a reason the Server states in a message. |

Table 73 – MissionUpdateResultEnum definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:MissionUpdateResultEnum |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | EnumStrings | 0:LocalizedText[5] | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## Pose3DDataType

A rigid-body pose. Position is metres; Orientation is a UNIT QUATERNION ordered (x, y, z, w). Quaternions are used because OPC UA defines no quaternion type and the Euler triple in ThreeDOrientation is ambiguous without an external convention; Annex C gives the normative conversion to and from ThreeDFrame. All frames are right-handed. FrameId names the CoordinateFrame the pose is expressed in; an empty FrameId means the Server's default work frame.

Table 74 – Pose3DDataType Items

| Name | Value | Description |
| --- | --- | --- |
| FrameId |  | FrameId of the CoordinateFrame this pose is expressed in; empty for the default work frame. |
| Position |  | Translation (x, y, z) in metres. |
| Orientation |  | Unit quaternion (x, y, z, w). |

Table 75 – Pose3DDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:Pose3DDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## MotionConstraintsDataType

Limits a motion is to respect. Every field is a REQUEST bounded by what the robot is configured to permit: a Server clamps rather than refuses, except where clause 10 requires refusal. A value of zero or less means the field is unspecified and the Server chooses.

Table 76 – MotionConstraintsDataType Items

| Name | Value | Description |
| --- | --- | --- |
| SpeedFraction |  | Fraction of the configured maximum speed, in the range 0 to 1. This is the portable speed control: every vendor supports it. |
| CartesianSpeed |  | Tool centre point speed in metres per second. Ignored by joint moves. |
| CartesianAcceleration |  | Tool centre point acceleration in metres per second squared. |
| Jerk |  | Rate of change of acceleration in metres per second cubed. |

Table 77 – MotionConstraintsDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:MotionConstraintsDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## BlendDataType

How a motion ends. Radius is interpreted only when Termination is Blend, and is a request: controllers that expose a unitless blend scale rather than a distance map it as best they can, and a Server that cannot honour the exact radius still succeeds.

Table 78 – BlendDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Termination |  | Exact stop, or blend into the next motion. |
| Radius |  | Blend radius in metres, measured from the target. Zero or less means the Server chooses. |

Table 79 – BlendDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:BlendDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## IntentDataType

Abstract base of every intent. An intent is a single task-level request; it is what a client submits and what a mission step holds, so the two are the same shape. Extension is by SUBTYPING this structure, which keeps new intents discoverable through IntentCapabilitiesType rather than by probing for BrowseNames.

Table 80 – IntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| IntentId |  | Client-assigned identifier, unique among the intents the client has outstanding. Empty asks the Server to assign one, which it returns. |
| Label |  | Human-readable description. Never interpreted. |
| BufferMode |  | How this intent relates to one already executing. |
| BlockingMode |  | Whether motion and other intents may proceed alongside it. |

Table 81 – IntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:IntentDataType |  |  |  |  |
| IsAbstract | True |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## MotionIntentDataType

Abstract base of the intents that move the robot. ToolFrame names the frame whose origin is driven to the target - without it a pose target is meaningless, and OPC 40010-1 defines no tool centre point at all.

Table 82 – MotionIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| ToolFrame |  | The CoordinateFrame, of role Tool, whose origin is driven to the target. Null means the tool currently fitted. |
| Constraints |  | Speed and acceleration limits for this motion. |
| Blend |  | How this motion ends. |

Table 83 – MotionIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:MotionIntentDataType |  |  |  |  |
| IsAbstract | True |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the IntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## JointMoveIntentDataType

Move by interpolating in joint space. This is the fastest way between two configurations and the path the tool centre point takes is not controlled. It is the portable equivalent of PTP, MoveJ, J and MOVJ. Giving a pose rather than joint values asks the Server to solve the kinematics itself, which is the 'move to this pose, you choose how' case.

Table 84 – JointMoveIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| HasJointTargets |  | True when JointTargets is meaningful; False when TargetPose is. |
| JointTargets |  | One value per axis, in the order the axes are declared under the controller: radians for a Revolute axis, metres for a Prismatic one. |
| TargetPose |  | Pose to reach, used when HasJointTargets is False. |

Table 85 – JointMoveIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:JointMoveIntentDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the MotionIntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## LinearMoveIntentDataType

Move the tool centre point along a straight line to the target. The portable equivalent of LIN, MoveL, L and MOVL.

Table 86 – LinearMoveIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Target |  | Pose to reach. |

Table 87 – LinearMoveIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:LinearMoveIntentDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the MotionIntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## CircularMoveIntentDataType

Move the tool centre point along the circular arc that passes through ViaPoint and ends at Target. The portable equivalent of CIRC, MoveC, C and MOVC. Only the position of ViaPoint defines the arc; its orientation is ignored.

Table 88 – CircularMoveIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| ViaPoint |  | A pose on the arc between the start and the target. Only its position is used. |
| Target |  | Pose to reach. |

Table 89 – CircularMoveIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:CircularMoveIntentDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the MotionIntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## GraspIntentDataType

Close the end effector on an object. Force and Width are requests; an end effector that cannot regulate force ignores Force and still succeeds.

Table 90 – GraspIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Tool |  | The Tool to actuate. Null means the tool currently fitted. |
| Force |  | Grasp force in newtons. Zero or less means the Server chooses. |
| Width |  | Opening at which to close, in metres. Zero or less means the Server chooses. |
| Approach |  | Direction of approach. |

Table 91 – GraspIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:GraspIntentDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the IntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## ReleaseIntentDataType

Give up a held object.

Table 92 – ReleaseIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Tool |  | The Tool to actuate. Null means the tool currently fitted. |
| Mode |  | How the object is given up. |
| HasTarget |  | True when Target is meaningful. |
| Target |  | Where to place the object, when Mode is Place. |

Table 93 – ReleaseIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:ReleaseIntentDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the IntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## PickIntentDataType

Take an object from a location. Source is a REFERENCE TO A LOCATION NODE, not a name: the location's pose and its properties are then read from the address space, so the station identity has exactly one definition.

Table 94 – PickIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Source |  | The Location to pick from. |
| Tool |  | The Tool to use. Null means the tool currently fitted. |
| ObjectClass |  | What to pick, when the location can hold more than one kind. Empty means whatever is there. |
| Force |  | Grasp force in newtons. Zero or less means the Server chooses. |
| Approach |  | Direction of approach. |
| Attributes |  | Further named parameters the Server declares in its capabilities. |

Table 95 – PickIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:PickIntentDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the IntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## PlaceIntentDataType

Put a held object at a location. Destination is a reference to a Location node, for the same reason as PickIntentDataType.Source.

Table 96 – PlaceIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Destination |  | The Location to place at. |
| Tool |  | The Tool to use. Null means the tool currently fitted. |
| Approach |  | Direction of approach. |
| Attributes |  | Further named parameters the Server declares in its capabilities. |

Table 97 – PlaceIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:PlaceIntentDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the IntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## ToolChangeIntentDataType

Exchange the fitted end effector.

Table 98 – ToolChangeIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Tool |  | The Tool to fit. Null means release the fitted tool and fit nothing. |
| DockStation |  | The Location of the tool changer. Null lets the Server choose. |

Table 99 – ToolChangeIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:ToolChangeIntentDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the IntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## SetOutputIntentDataType

Set a discrete or analogue output. Output references an OutputSignal node, so the signal's meaning, range and unit are described once in the address space instead of being implied by a string.

Table 100 – SetOutputIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Output |  | The OutputSignal to write. |
| Value |  | Value to write, of the signal's own DataType. |

Table 101 – SetOutputIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:SetOutputIntentDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the IntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## CallProgramIntentDataType

Run a program that already exists on the controller. This is the escape hatch for capability this specification does not model, and the bridge to the OPC 40010-1 task control surface.

Table 102 – CallProgramIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Program |  | The Program to run. |
| Arguments |  | Named arguments for the program. |

Table 103 – CallProgramIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:CallProgramIntentDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the IntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## WaitIntentDataType

Do nothing for a while, or until released. A mission needs this to express a rendezvous with something the robot does not control; without it a client has to hold the queue open from outside.

Table 104 – WaitIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Duration |  | How long to wait, in milliseconds. Zero or less waits until Signal is released. |
| Signal |  | An OutputSignal under this controller, or another Variable of DataType Boolean, whose becoming true ends the wait. Null waits only for Duration. Clause 11.3 requires the Server to check that it resolves to one of those two, under the controller being commanded, and to refuse it otherwise. |

Table 105 – WaitIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:WaitIntentDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the IntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## IntentResultDataType

The outcome of one intent, preserved after it terminates. AchievedPose records where the tool centre point actually ended, which is what lets a client tell a blended corner from an exact stop and audit a placement.

Table 106 – IntentResultDataType Items

| Name | Value | Description |
| --- | --- | --- |
| IntentId |  | The intent this describes. |
| State |  | Terminal state reached. |
| Failure |  | Reason, or None on success. |
| Message |  | Human-readable detail. Never parsed. |
| HasAchievedPose |  | True when AchievedPose is meaningful. |
| AchievedPose |  | Where the tool centre point came to rest, or was when blending began. |
| StartTime |  | When execution began. |
| EndTime |  | When the terminal state was reached. |
| Outputs |  | Named results the intent produced, for example the identity of a picked object. |

Table 107 – IntentResultDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:IntentResultDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## MissionStepDataType

One step of a mission. Released is what splits a mission into its immutable base and its revisable horizon: a released step has been committed and may already be executing, so an update may not touch it. Status is a HINT - where Operation is not null, that IntentOperation's state machine decides.

Table 108 – MissionStepDataType Items

| Name | Value | Description |
| --- | --- | --- |
| StepId |  | Identifier unique within the mission. |
| SequenceId |  | Execution order within the mission, ascending. |
| Released |  | True for a base step, which is committed and immutable. False for a horizon step, which an update may replace or remove. |
| Intent |  | The intent this step executes. |
| Status |  | Reported status. Where Operation is not null its state machine is authoritative and this reflects it. |
| Operation |  | The IntentOperation executing this step, or null while the step has not begun. |
| ErrorPolicy |  | What the mission does when this step does not succeed. |
| FallbackStepId |  | The step to continue at, when ErrorPolicy is Fallback or Compensate. |

Table 109 – MissionStepDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:MissionStepDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## MissionDataType

An ordered sequence of intents submitted and tracked as a unit. MissionUpdateId increases with every update, so a Server can reject an update that crossed with another in flight instead of applying it out of order.

Table 110 – MissionDataType Items

| Name | Value | Description |
| --- | --- | --- |
| MissionId |  | Client-assigned identifier. Empty asks the Server to assign one, which it returns. |
| MissionUpdateId |  | Revision of this mission, increasing. The first submission is 0. |
| Label |  | Human-readable description. Never interpreted. |
| Steps |  | The steps, in ascending SequenceId order. |
| Transitions |  | The step graph. An empty array means the steps run in order, which is the flat sequence a mission without branching has always been. |

Table 111 – MissionDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:MissionDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## IntentCapabilityDataType

What the Server will accept for one intent type. This is the machine-readable declaration that makes an intent surface discoverable: a client reads it once and knows what it may submit, instead of submitting to find out. It is the analogue of the VDA 5050 factsheet.

Table 112 – IntentCapabilityDataType Items

| Name | Value | Description |
| --- | --- | --- |
| IntentType |  | The DataType of the intent, a subtype of IntentDataType. |
| Description |  | What this Server does with it. |
| CancelSupported |  | True when Cancel is honoured for it. A Server may still refuse a particular cancel; see clause 6.5. |
| PauseSupported |  | True when Pause and Resume are honoured for a running intent. False when the Server can only stop its queue: suspending execution means the robot stops, and Suspended is never reported while it is still moving. |
| RetrySupported |  | True when it can terminate Retriable and be re-attempted. |
| SupportedBufferModes |  | Buffer modes accepted for it. Aborting is always accepted and always listed. |
| SupportedBlockingModes |  | Blocking modes accepted for it. |
| Attributes |  | Named parameters this Server recognises in the intent's Attributes field. |

Table 113 – IntentCapabilityDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:IntentCapabilityDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## SafeMotionFunctionEnum

The safe motion function a safety system is enforcing, as defined by IEC 61800-5-2. This is a REPORT. The safety system enforces these independently of this interface, and a client reading them has not thereby obtained any safety function - see clause 10.

Table 114 – SafeMotionFunctionEnum Items

| Name | Value | Description |
| --- | --- | --- |
| None | 0 | No safe motion function is active. |
| Sto | 1 | Safe Torque Off: torque is removed. |
| Ss1 | 2 | Safe Stop 1: a controlled ramp to standstill, then Safe Torque Off. |
| Ss2 | 3 | Safe Stop 2: a controlled ramp to standstill, which is then held under power. |
| Sos | 4 | Safe Operating Stop: standstill is monitored while the drive remains energised. |
| Sls | 5 | Safely Limited Speed: speed is monitored against a limit. |
| Slp | 6 | Safely Limited Position: position is monitored against a limit. |
| Sdi | 7 | Safe Direction: motion is permitted in one direction only. |
| Sbc | 8 | Safe Brake Control: a brake is commanded safely. |

Table 115 – SafeMotionFunctionEnum definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:SafeMotionFunctionEnum |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | EnumStrings | 0:LocalizedText[9] | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## RealTimeTransportEnum

The transport of a brokered real-time channel. This specification defines none of these: it describes them so a client can find and open one, and the samples never traverse this interface.

Table 116 – RealTimeTransportEnum Items

| Name | Value | Description |
| --- | --- | --- |
| Rtde | 0 | Universal Robots Real-Time Data Exchange. |
| Egm | 1 | ABB Externally Guided Motion. |
| Fri | 2 | KUKA Fast Research Interface. |
| Rsi | 3 | KUKA Robot Sensor Interface. |
| MotoRos2 | 4 | Yaskawa MotoROS2. |
| OpcUaFx | 5 | OPC UA FX (OPC 10000-80 to -84). The open path, and the only one in this list that is an OPC Foundation specification. |
| Other | 6 | A transport identified by the channel's own descriptor. |

Table 117 – RealTimeTransportEnum definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:RealTimeTransportEnum |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | EnumStrings | 0:LocalizedText[7] | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## ChannelInitiatorEnum

Which end opens the transport connection of a brokered channel. Getting this wrong is the usual reason a first connection attempt fails, so it is stated rather than left to the reader.

Table 118 – ChannelInitiatorEnum Items

| Name | Value | Description |
| --- | --- | --- |
| Server | 0 | The Server connects to an endpoint the client is listening on. |
| Client | 1 | The client connects to the endpoint the Server publishes. |

Table 119 – ChannelInitiatorEnum definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:ChannelInitiatorEnum |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | EnumStrings | 0:LocalizedText[2] | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## ErrorPolicyEnum

What a mission does when one of its steps does not succeed. Without this a mission can only abort, which forces every recovery out into the client.

Table 120 – ErrorPolicyEnum Items

| Name | Value | Description |
| --- | --- | --- |
| Abort | 0 | End the mission. This is the default and the behaviour of a mission that declares no policy. |
| Retry | 1 | Re-attempt the step. A Server bounds the attempts and reports Failed when they are exhausted. |
| Skip | 2 | Record the failure and continue with the next step. |
| Fallback | 3 | Continue at FallbackStepId instead. |
| Compensate | 4 | Run the fallback step to undo the work already done, then end the mission. |

Table 121 – ErrorPolicyEnum definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:ErrorPolicyEnum |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | EnumStrings | 0:LocalizedText[5] | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## DivergenceKindEnum

How the transitions leaving one step relate to each other, following the divergence of an IEC 61131-3 sequential function chart.

Table 122 – DivergenceKindEnum Items

| Name | Value | Description |
| --- | --- | --- |
| Alternative | 0 | Exactly one transition is taken - the first whose condition holds. An OR divergence. |
| Parallel | 1 | Every transition is taken and the branches run concurrently. An AND divergence. |

Table 123 – DivergenceKindEnum definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:DivergenceKindEnum |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | EnumStrings | 0:LocalizedText[2] | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## WeaveShapeEnum

The oscillation applied across an arc weld seam.

Table 124 – WeaveShapeEnum Items

| Name | Value | Description |
| --- | --- | --- |
| None | 0 | No weave. |
| Sine | 1 | Sinusoidal oscillation. |
| Zigzag | 2 | Triangular oscillation. |
| Trapezoid | 3 | Trapezoidal oscillation, with a dwell at each edge. |

Table 125 – WeaveShapeEnum definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:WeaveShapeEnum |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Enumeration defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasProperty | Variable | EnumStrings | 0:LocalizedText[4] | 0:PropertyType | M |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## TrajectoryPointDataType

One point of a time-parameterised path. Positions are per axis in the order the axes are declared, in radians or metres by AxisKind. TimeFromStart is measured from the start of the trajectory, which is what makes the path a trajectory rather than a list of waypoints. Velocities and Accelerations are optional and may be empty.

Table 126 – TrajectoryPointDataType Items

| Name | Value | Description |
| --- | --- | --- |
| TimeFromStart |  | Milliseconds from the start of the trajectory. |
| Positions |  | One value per axis. |
| Velocities |  | One value per axis, or empty. |
| Accelerations |  | One value per axis, or empty. |

Table 127 – TrajectoryPointDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:TrajectoryPointDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## MotionToleranceDataType

How far execution may deviate before it is a failure. A tolerance of zero or less means the Server applies its own.

Table 128 – MotionToleranceDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Position |  | Positional tolerance in metres. |
| Orientation |  | Orientation tolerance in radians. |
| Time |  | Timing tolerance in milliseconds. |

Table 129 – MotionToleranceDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:MotionToleranceDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## TrajectoryIntentDataType

Execute a time-parameterised path. The Server's own motion kernel runs it; this interface hands the whole trajectory over in one submission and does not stream it. That is what makes trajectory execution expressible here when real-time control is not - see clause 4.3.

Table 130 – TrajectoryIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Points |  | The trajectory, in ascending TimeFromStart order. |
| PathTolerance |  | Permitted deviation while following the path. |
| GoalTolerance |  | Permitted deviation at the final point. |
| GoalTimeTolerance |  | How much later than the final point's TimeFromStart completion may be, in milliseconds. |

Table 131 – TrajectoryIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:TrajectoryIntentDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the MotionIntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## PathWaypointDataType

One waypoint of a Cartesian path, with the blend that applies at it. Per-waypoint blending is what distinguishes a path from a sequence of separate linear moves: the robot need not stop between them.

Table 132 – PathWaypointDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Pose |  | Where the tool centre point passes. |
| Blend |  | How this waypoint is left. |

Table 133 – PathWaypointDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:PathWaypointDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## CartesianPathIntentDataType

Follow a list of Cartesian waypoints. This is the portable form of a taught path, and unlike a trajectory it carries no timing: the Server paces it from the motion constraints.

Table 134 – CartesianPathIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Waypoints |  | The path, in order. |

Table 135 – CartesianPathIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:CartesianPathIntentDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the MotionIntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## ForceIntentDataType

Move until contact. The portable subset of the force-controlled moves every vendor offers: travel along a direction until a contact force is reached or a distance is exhausted, whichever comes first. Reaching the distance without contact is a failure, because the intent was to touch something.

Table 136 – ForceIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Direction |  | Unit direction of travel (x, y, z) in the frame of the target. |
| FrameId |  | The CoordinateFrame Direction is expressed in; empty for the default work frame. |
| ContactForce |  | The force in newtons at which contact is declared. |
| MaxDistance |  | How far to travel before giving up, in metres. |
| HoldForce |  | True to keep pressing at ContactForce after contact rather than stopping. |

Table 137 – ForceIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:ForceIntentDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the MotionIntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## ProcessIntentDataType

Abstract base of the intents that run an application process along a path. Every process needs the same two things beyond its own parameters: a reference to the process program or procedure the equipment holds, and room for the parameters this specification has not standardised.

Table 138 – ProcessIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| ProcessProgram |  | The Program or equipment-side procedure to run, or null when the parameters here are sufficient. |
| Attributes |  | Further named parameters the Server declares in its capabilities. |

Table 139 – ProcessIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:ProcessIntentDataType |  |  |  |  |
| IsAbstract | True |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the MotionIntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## ArcWeldIntentDataType

Lay an arc weld along the path. The parameters are the subset ABB seamdata/welddata/weavedata, FANUC weld schedules and KUKA ArcTech all carry. WeldProcedureRef points at a welding procedure specification (ISO 15609) where the installation works to one; this specification does not restate its content.

Table 140 – ArcWeldIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Voltage |  | Arc voltage in volts. |
| WireFeedSpeed |  | Wire feed speed in metres per minute. |
| TravelSpeed |  | Tool centre point speed in metres per second. |
| GasPreflowTime |  | Shielding gas pre-flow, in milliseconds. |
| GasPostflowTime |  | Shielding gas post-flow, in milliseconds. |
| ArcStartDelay |  | Delay before travel begins, in milliseconds. |
| CraterFillTime |  | Crater fill at the end of the seam, in milliseconds. |
| WeaveShape |  | Oscillation across the seam. |
| WeaveAmplitude |  | Peak-to-peak weave width in metres. |
| WeaveFrequency |  | Weave frequency in hertz. |
| SeamTrackingEnabled |  | True to enable seam tracking where the equipment provides it. |
| WeldProcedureRef |  | Identifier of the welding procedure specification this weld works to. |

Table 141 – ArcWeldIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:ArcWeldIntentDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the ProcessIntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## SpotWeldIntentDataType

Make a resistance spot weld at the target. WeldSchedule selects the weld controller's own program: current and time are the weld controller's business, and are carried here only where the installation drives them from the robot.

Table 142 – SpotWeldIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| WeldSchedule |  | The weld controller program to run. |
| GunForce |  | Electrode force in newtons. |
| ApproachDistance |  | Gun open distance before the weld, in metres. |
| RetractDistance |  | Gun open distance after the weld, in metres. |
| MaterialThickness |  | Total stack thickness in metres, where the schedule is chosen adaptively. |
| TipDressRequested |  | True to dress the tips after this weld. |

Table 143 – SpotWeldIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:SpotWeldIntentDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the ProcessIntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## DispenseIntentDataType

Lay a bead of adhesive, sealant or paint along the path. The trigger distances exist because material does not start and stop instantly: a Server begins dispensing before the path and stops before its end.

Table 144 – DispenseIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| FlowRate |  | Nominal flow in millilitres per minute. |
| TriggerOnDistance |  | Distance before the path start at which dispensing begins, in metres. |
| TriggerOffDistance |  | Distance before the path end at which dispensing stops, in metres. |
| BeadWidth |  | Target bead width in metres. |
| MaterialTemperature |  | Material temperature in degrees Celsius, for hot-melt work. |
| PurgeCycles |  | Nozzle purge cycles before starting. |

Table 145 – DispenseIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:DispenseIntentDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the ProcessIntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## FastenIntentDataType

Drive a fastener at the target. This intent is deliberately THIN: OPC 40450 and OPC 40451 already define joining and tightening in full, so where such a model is exposed, Joint references the joint in that model and the result belongs there; otherwise Joint is null and the remaining fastening parameters stand alone. Restating those parameters here would create a second definition of the same fact.

Table 146 – FastenIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Joint |  | The joint being fastened, in an OPC UA joining or tightening model where one is implemented; null otherwise. |
| ProgramNumber |  | The tightening program the tool is to run. |
| TargetTorque |  | Target torque in newton metres, where the robot supplies it rather than the tool. |
| TargetAngle |  | Target angle in radians, for angle-controlled strategies. |
| SnugTorque |  | Torque at which angle counting begins, in newton metres. |

Table 147 – FastenIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:FastenIntentDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the ProcessIntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## PalletiseIntentDataType

Place an item into a pattern. The pattern itself is a Location, so its geometry has one definition that a client can read rather than being recomputed from indices on both sides.

Table 148 – PalletiseIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| Pattern |  | The Location describing the pallet or pattern. |
| Layer |  | Layer index, counting from zero. |
| Row |  | Row index within the layer, counting from zero. |
| Column |  | Column index within the row, counting from zero. |
| ItemOrientation |  | Rotation of the item about the pattern normal, in radians. |

Table 149 – PalletiseIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:PalletiseIntentDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the ProcessIntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## SurfaceFinishIntentDataType

Follow the path pressing into the surface - grinding, polishing, deburring or sanding. ContactForce is what distinguishes it from a plain path: the robot yields normal to the surface to hold that force.

Table 150 – SurfaceFinishIntentDataType Items

| Name | Value | Description |
| --- | --- | --- |
| ContactForce |  | Force normal to the surface, in newtons. |
| FeedRate |  | Travel speed along the surface, in metres per second. |
| ToolSpeed |  | Rotational speed of the tool, in revolutions per minute. |
| StepOver |  | Lateral offset between adjacent passes, in metres. |

Table 151 – SurfaceFinishIntentDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:SurfaceFinishIntentDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the ProcessIntentDataType defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## MissionTransitionDataType

One edge of a mission's step graph, following the step-and-transition form of an IEC 61131-3 sequential function chart. Condition is an OPC UA ContentFilter - the base specification's own filter grammar, reused so that this specification does not invent an expression language for implementers to write a parser for.

Table 152 – MissionTransitionDataType Items

| Name | Value | Description |
| --- | --- | --- |
| FromStepId |  | The step this transition leaves. |
| ToStepId |  | The step this transition enters. |
| Condition |  | The condition under which it is taken. An empty filter is always true. |
| DivergenceKind |  | How this transition relates to the others leaving the same step. |

Table 153 – MissionTransitionDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:MissionTransitionDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

## KinematicJointDataType

One joint of the kinematic chain: where it sits relative to its predecessor and which way it acts. OPC 40010-1 describes a robot's topology and its axes but defines no kinematic chain, so this is additive rather than a second account of the same thing.

Table 154 – KinematicJointDataType Items

| Name | Value | Description |
| --- | --- | --- |
| AxisId |  | The AxisType this joint corresponds to. |
| Kind |  | Whether it rotates or translates. |
| OriginTransform |  | Pose of this joint's frame within its predecessor's, at zero position. |
| AxisVector |  | Unit vector (x, y, z) the joint rotates about or translates along, in its own frame. |

Table 155 – KinematicJointDataType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:KinematicJointDataType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:Structure defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| 0:HasEncoding | Object | Default Binary |  | 0:DataTypeEncodingType |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent DataTypes |  |  |  |  |  |

# OPC UA ReferenceTypes

## HasIntentController

Binds an intent surface to the thing it commands. This is how the model attaches to a robot described by another specification - an OPC 40010-1 MotionDeviceSystem, say - without depending on it. Annex B defines that binding.

Table 156 – HasIntentController definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:HasIntentController |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| InverseName | IntentControllerOf |  |  |  |  |
| Symmetric | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:NonHierarchicalReferences defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent ReferenceTypes |  |  |  |  |  |

## HasFrameParent

From a CoordinateFrame to the frame its Transform is expressed in. Frames form a tree, so a pose given in one frame can be re-expressed in another by composing the transforms along the path between them.

Table 157 – HasFrameParent definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:HasFrameParent |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| InverseName | FrameParentOf |  |  |  |  |
| Symmetric | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:NonHierarchicalReferences defined in OPC 10000-5, i.e. inheriting the InstanceDeclarations of that Node. |  |  |  |  |  |
| Conformance Units |  |  |  |  |  |
| RobotIntent ReferenceTypes |  |  |  |  |  |

# Instances

## Default Binary

Default Binary encoding of the structure.

Table 158 – Default Binary definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 0:Default Binary |  |  |  |  |
| TypeDefinition | 0:DataTypeEncodingType |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Conformance Units |  |  |  |  |  |
| OpenUSD Default Binary |  |  |  |  |  |

## RobotIntent

Entry point for robot intent on this Server. A client browses Server/RobotIntent/Controllers to find every robot it can command.

Table 159 – RobotIntent definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | 1:RobotIntent |  |  |  |  |
| TypeDefinition | RobotIntentRootType |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| Conformance Units |  |  |  |  |  |
| OpenUSD RobotIntent |  |  |  |  |  |

# Profiles and conformance units

## Declaring conformance

A Server declares conformance by exposing RobotIntentRootType under the Server object with SpecificationVersion set to the release it implements, by populating IntentCapabilitiesType truthfully, and by listing in SupportedFacets the facets of Table 12.2 that each controller satisfies.

SupportedFacets carries the facet names of Table 12.2 verbatim. It exists because conformance is otherwise not machine-readable: a client would have to re-derive the whole table from the address space, and since several rows are behavioural, two clients deriving independently could reach different conclusions about the same Server. This mirrors ServerCapabilitiesType.ServerProfileArray in OPC 10000-5, where a Server states its profiles rather than leaving them to be inferred.

A Server shall not list a facet whose structural requirements are unmet. The behavioural requirements are the Server's own attestation and are subject to the honesty rules of clause 9 — a Server that lists RI-Blending while treating the blending buffer modes as Buffered is making a false statement in exactly the sense clause 9 forbids, and is no more conformant than one that reports BlendingSupported true under the same conditions.

The NodeSet assigns every Node to one of three conformance units: RobotIntent for the ObjectTypes and their members, RobotIntent DataTypes for the intent hierarchy and the enumerations, and RobotIntent ReferenceTypes for the references. The facets below are expressed over those Nodes, so a Server claiming a facet implements the units the facet's members belong to.

## Facets

Requirements are of two kinds. Structural requirements are settled by reading the address space and the capability declaration: a client can check them, and so can a compliance tool, without commanding the robot. Behavioural requirements — written below as accepting, honouring, maintaining or observing a rule — cannot be settled by reading, only by exercising the Server, and are the Server's attestation under clause 9.

Table 160 – Facets

| Facet | Requires |
| --- | --- |
| RI-Base (mandatory) | RobotIntentRootType; at least one IntentControllerType with Capabilities, Frames, Tools, Locations, Axes and Intents; SubmitIntent, CancelIntent, CancelAll, RequestControl, ReleaseControl; IntentOperationType instances with the state model of 6.14.3; the refusal rules of 6.14.2 and clause 10. |
| RI-Motion-Joint | JointMoveIntentDataType, with AxisType instances covering 0 to AxisCount − 1. |
| RI-Motion-Linear | LinearMoveIntentDataType. |
| RI-Motion-Circular | CircularMoveIntentDataType. |
| RI-Trajectory | TrajectoryIntentDataType, TrajectorySupported true, and the tolerance rules of 6.14.8. |
| RI-Path | CartesianPathIntentDataType and TrajectorySupported true. |
| RI-Force | ForceIntentDataType and ForceControlSupported true — the robot genuinely regulates force. |
| RI-RealTimeChannel | RealTimeChannelsSupported true; the RealTimeChannels folder with at least one RealTimeChannelType; OpenRealTimeChannel and CloseRealTimeChannel with the lease rules of 6.14.9. |
| RI-Safety | SafetyState populated from the safety system, and the refusals of 6.18.4. |
| RI-Description | Description with a KinematicChain covering every axis, ReachRadius, PayloadLimit and MaxCartesianSpeed. |
| RI-Process-ArcWeld | ArcWeldIntentDataType. |
| RI-Process-SpotWeld | SpotWeldIntentDataType. |
| RI-Process-Dispense | DispenseIntentDataType. |
| RI-Process-Fasten | FastenIntentDataType. Support for non-null Joint is structural: where an OPC UA joining or tightening model is implemented, Joint resolves into it; where no such model is exposed under the controller, Joint is null and a non-null Joint is refused with CapabilityNotSupported. |
| RI-Process-Palletise | PalletiseIntentDataType and at least one LocationType describing a pattern. |
| RI-Process-SurfaceFinish | SurfaceFinishIntentDataType and RI-Force. |
| RI-Grasp | GraspIntentDataType and ReleaseIntentDataType, and at least one ToolType with a TcpFrame. |
| RI-PickPlace | PickIntentDataType, PlaceIntentDataType, and at least one LocationType. |
| RI-ToolChange | ToolChangeIntentDataType, and more than one ToolType. |
| RI-Output | SetOutputIntentDataType and the Outputs folder. |
| RI-Program | CallProgramIntentDataType and the Programs folder. |
| RI-Wait | WaitIntentDataType. |
| RI-Queue | MaxQueueDepth greater than zero; Buffered accepted; QueuePosition maintained. |
| RI-Blending | BlendingSupported true; the four blending buffer modes accepted and honoured; Result.AchievedPose reported at the blend point per 6.14.4. |
| RI-Pause | Pause and Resume. |
| RI-Retry | Retry, and Retriable reachable. |
| RI-Mission | MissionsSupported true; SubmitMission, CancelMission; MissionType instances. |
| RI-Mission-Horizon | RI-Mission, plus MissionHorizonSupported true, UpdateMission, and the base immutability rules of §7.2. |
| RI-Mission-Branching | RI-Mission, plus MissionBranchingSupported true, Transitions evaluated per 6.15.4, and the error policies honoured. |
| RI-Interop-40010 | Annex B. |
| RI-Interop-Vision | Annex E. |

A facet other than RI-Base is claimed only where every intent type it names appears in SupportedIntents.

RI-Base additionally requires SupportedFacets, since a conformance claim that cannot be read is not a claim.

Conformance is therefore declared at two levels, and they answer different questions. SupportedFacets is a member of IntentCapabilitiesType, so it is stated per controller: a Server hosting two robots of different capability has two answers, and a client asking whether this controller blends must read this controller's list. ServerProfileArray is a member of the Server object, so it is stated once for the Server, which is the right granularity for a profile (11.3) — a named shape an integrator specifies and a supplier builds to.

A Server shall publish the URI of every profile it claims in Server/ServerCapabilities/ServerProfileArray, and

shall also publish the facet URIs for every facet required by those profiles and every facet listed by any

controller in SupportedFacets. The two discovery paths shall agree: a profile URI shall be backed by at least one

controller whose SupportedFacets includes every facet in that profile, and a facet URI shall be backed by at least

one controller that lists the corresponding facet. A facet URI on the Server that no controller lists in

SupportedFacets is a claim nothing in the address space backs, and a client that read only one of them would be told

something untrue by the other.

## Profiles

A facet is a building block. A profile is a complete claim: a named set of facets describing one plausible robot Server, which is what an integrator specifies and what two manufacturers implementing the same shape agree they have built. 5's use cases are written about profiles even though they do not use the word — a mixed-fleet cell works only because two robots claim the same one.

Four are defined. Each includes the Robot Motion Server set, and a Server may claim more than one: a robot that both follows paths and executes missions claims two.

Claiming a profile is claiming every facet in it, on the terms 11.2 sets out — structural requirements a client can check by reading, behavioural ones the Server attests to under clause 9. A profile is a shorter way to say the same thing, not a weaker one.

Table 161 – Profiles

| Profile | Facets | The Server it describes |
| --- | --- | --- |
| Robot Motion Server | RI-Base, RI-Motion-Joint, RI-Motion-Linear, RI-Description, RI-Safety | The baseline. A robot that can be commanded to a joint configuration or a Cartesian pose, that describes its own kinematics and limits, and that reports what its safety system is enforcing. |
| Robot Handling Server | Motion, plus RI-Motion-Circular, RI-Grasp, RI-PickPlace, RI-ToolChange, RI-Output, RI-Queue | Material handling. Picking, placing, changing tools and driving the discrete outputs a gripper needs, with a queue so a cell controller can stay ahead of the robot. |
| Robot Path Server | Motion, plus RI-Trajectory, RI-Path, RI-Blending | Continuous-path work. A whole path is handed over once and the robot's own motion kernel runs it, blending between segments rather than stopping at each. |
| Robot Mission Server | Motion, plus RI-Mission, RI-Program, RI-Wait, RI-Pause, RI-Retry | Long-running supervised operation. A mission is submitted, watched, paused, retried and cancelled, which is 5's fourth use case stated as a claim. |

RI-Safety is in the baseline rather than optional to it. Clause 10 is explicit that this specification is not safety-rated and that no Method here is a safety function. What it does require is a duty: the Server reports what the safety system enforces and refuses work that would exceed it. Every profiled Server owes that duty, because an integrator specifying a profile is entitled to assume a robot will decline an intent its safety configuration forbids rather than attempt it. A robot that cannot read its safety system claims facets individually and not a profile.

The process facets — RI-Process-ArcWeld and its siblings — are deliberately in no profile. A welding robot is a Robot Path Server that additionally claims RI-Process-ArcWeld, and bundling the process into a profile would have produced one profile per process and no way to say that the underlying motion is the same. The same reasoning keeps RI-Force, RI-RealTimeChannel and the two interop facets outside all four.

## Profile and facet URIs

A profile name is for a human. ServerProfileArray holds URIs, and unless this specification states them two Servers implementing the same profile publish different strings and no client can match either.

Profiles are published under http://opcfoundation.org/UA-Profile/RobotIntent/Server/:

Table 162 – Profile and facet URIs

| Profile | URI suffix |
| --- | --- |
| Robot Motion Server | Motion |
| Robot Handling Server | Handling |
| Robot Path Server | Path |
| Robot Mission Server | Mission |

Facets are published under http://opcfoundation.org/UA-Profile/RobotIntent/Facet/, with the suffix being the facet name after the RI- prefix: RI-Base is Base, RI-Motion-Joint is Motion-Joint, RI-Process-ArcWeld is Process-ArcWeld, and so on for every row of 11.2. A Server claiming a profile publishes the profile URI and the facet URIs that make the profile true; a Server claiming additional per-controller facets publishes those facet URIs as well. These URIs exist so a generic OPC UA tool that reads ServerProfileArray and knows nothing about robots can still recognise both the profile and the facets behind it; the authority on which facets a given controller satisfies is that controller's SupportedFacets, because only it is stated per controller.

These URIs are provisional, on the same terms as the namespace URI and the NodeIds: this is a working-group draft, and the OPC Foundation assigns the final values.

# Namespaces

## Namespace metadata

The namespace metadata provide standardized information about the elements of this namespace, which an aggregating Server relies on. All Nodes defined by this document are static: they are identical in every Server, including the Value Attribute.

The information is provided as an Object of type NamespaceMetadataType, a component of the Namespaces Object of the Server Object. The NamespaceMetadataType ObjectType and its Properties are defined in OPC 10000-5. The same version information is carried by the ModelTableEntry of the UANodeSet XML file.

Table 163 – NamespaceMetadata Object for this document

| Property | DataType | Value |
| --- | --- | --- |
| NamespaceUri | String | http://opcfoundation.org/UA/RobotIntent/ |
| NamespaceVersion | String | 0.1.0 |
| NamespacePublicationDate | DateTime | 2026-08-02 |
| IsNamespaceSubset | Boolean | False |
| StaticNodeIdTypes | IdType[] | 0 (Numeric) |
| StaticNumericNodeIdRange | NumericRange[] | 1001:7001 |
| StaticStringNodeIdPattern | String | -- |

The IsNamespaceSubset Property is False because the UANodeSet XML file contains the complete namespace. A Server exposing only a subset sets it to True.

## Handling of OPC UA namespaces

Namespaces are used by OPC UA to create unique identifiers across different naming authorities. The NodeId and BrowseName Attributes are identifiers. A Node in the AddressSpace is unambiguously identified by its NodeId; a BrowseName is not unique and is used to build a browse path or to name a standard Property.

Table 164 – Namespaces used in an OpenUSD Server

| NamespaceURI | Description |
| --- | --- |
| http://opcfoundation.org/UA/ | Namespace for NodeIds and BrowseNames defined in the OPC UA specification (OPC 10000-3, OPC 10000-5). |
| Local Server URI | Namespace for Nodes defined in the local Server. This namespace shall have index 1. |
| http://opcfoundation.org/UA/xRegistry/ | Namespace of the RequiredModel that carries the abstract registry base types this document extends. |
| http://opcfoundation.org/UA/RobotIntent/ | Namespace for NodeIds and BrowseNames defined in this document. |
| Vendor specific types | A Server may provide vendor-specific types derived from the types defined in this document, in one or more additional namespaces. |

The following table lists the namespaces and their indexes used for BrowseNames in this document. The default namespace of this document is not listed, because every BrowseName without a prefix uses it.

Table 165 – Namespaces used in this document

| NamespaceURI | Namespace Index | Example |
| --- | --- | --- |
| http://opcfoundation.org/UA/ | 0 | 0:EngineeringUnits |
| http://opcfoundation.org/UA/xRegistry/ | 1 | 1:ResourceType |

(normative) 

Robot Intent namespace and mappings

NodeSet and supplementary files for Robot Intent Information Model

An Information Model is formally defined in an XML file called a NodeSet This file conforms to the standard syntax defined in the Annex “Information Model XML Schema” OPC 10000-6. It can be read and processed by a computer program.

An Information Model is identified by a URI – the so-called NamespaceUri.

A NamespaceUri follows one of these conventions:

http://opcfoundation.org/UA/RI/

tag:opcfoundation.org,yyyy-MM:UA:RI

Where RI is described in 15 and yyyy-MM is the date when the NamespaceUri was first published. NamespaceUris are not network accessible URLs and the text should not suggest they are. The tag URI syntax allows authors to choose a URI that cannot be used as URL by mistake. Note that the date in the tag syntax is not the same as the PublicationDate for the NodeSet. It is set once when the URI is created and never changed.

The Online Reference provides a summary page for every NamespaceUri of released Information Models which has the form:

https://reference.opcfoundation.org/nodesets?u=http://opcfoundation.org/UA/RobotIntent/

The Robot Intent Information Model is identified by the following URI:

http://opcfoundation.org/UA/RI/

Documentation for the NamespaceUri can be found here.

“here” is a hyperlink to the summary page of the Online Reference.

In the hyperlink, the http://opcfoundation.org/UA/RobotIntent/ has to be replaced with the concrete URI for this specification.

The NodeSet associated with this version of specification can be found here:

https://reference.opcfoundation.org/nodesets/?u=http://opcfoundation.org/UA/RobotIntent/&v=0.1.0&i=1

The NodeSet associated with the latest version of the specification can be found here:

https://reference.opcfoundation.org/nodesets/?u=http://opcfoundation.org/UA/RobotIntent/&i=1

http://opcfoundation.org/UA/RobotIntent/ is the NamespaceUri for the Information Model.

The 0.1.0 is the string in the NamespaceVersion from the Namespace Metadata (see 16.1). This value is also the value of the Version attribute in the NodeSet.

Supplementary files for the Robot Intent Information Model can be found here:

https://reference.opcfoundation.org/nodesets/?u=http://opcfoundation.org/UA/RobotIntent/&v=0.1.0&i=2

The files associated with the latest version of the specification can be found here:

https://reference.opcfoundation.org/nodesets/?u=http://opcfoundation.org/UA/RobotIntent/&i=2

Supplementary files should be provided when appropriate (i.e., IRDI NodeSets or examples)

File Names

NodeIds: 	Opc.Ua.RI.NodeIds.csv

NodeSet: 	Opc.Ua.RI.NodeSet2.xml;

Any other files should have a prefix that provides context when the file is downloaded in a browser.

When DataTypes are defined in the NodeSet:

Opc.ua.RI.Types.xsd –

The XML schema for the DataTypes defined by the specification;

Opc.ua.RI.Types.bsd –

The OPC Binary schema for the DataTypes defined by the specification;

In case there is the need to reference individual files the following syntax should be added to the template:

https://reference.opcfoundation.org/files/<file>?u=<namespaceUri>&v=<version>
https://reference.opcfoundation.org/files/<file>?u=<namespaceUri>

for example:

https://reference.opcfoundation.org/files/StatusCode.csv?u=http://opcfoundation.org/UA/&v=1.05.02
https://reference.opcfoundation.org/files/StatusCode.csv?u=http://opcfoundation.org/UA/
https://reference.opcfoundation.org/files/Opc.Ua.Types.xsd?u=http://opcfoundation.org/UA/&v=1.05.02
https://reference.opcfoundation.org/files/Opc.Ua.Types.xsd?u=http://opcfoundation.org/UA/

Capability identifier

ServerCapabilityIdentifiers are defined in OPC 10000-12. They can be used for features, like certain information models, which are likely to be useful during the discovery process. The identifiers shall be short because of length restrictions for fields used in the mDNS specification.

The identifier shall be up to 6 characters. It is recommended to use the RI introduced in the guideline, at the beginning of clause 15 if that meets the requirement of up to 6 characters.

Note, that such identifiers are not required. If not needed, this Annex section shall be deleted.

The capability identifier for this document shall be:

RI

Node reference

The following table lists every Node this document defines, with the provisional NodeId assigned by the generator. It is generated from the UANodeSet and is informative for navigation; the UANodeSet is authoritative.

Table 166 – Nodes defined by this document

| NodeId | BrowseName | NodeClass | Description |
| --- | --- | --- | --- |
| ns=1;i=3001 | ExecutionStateEnum | DataType | Fine-grained execution state of an intent or a mission. This REFINES the Part 10 program state machine rather than restating it: Queued, Cancelling and the three terminal outcomes cannot be told apart from CurrentState alone. Clause 6.3 fixes which ExecutionState may accompany which Part 10 state, and a Server shall satisfy that table. |
| ns=1;i=3901 | EnumStrings | Variable |  |
| ns=1;i=3002 | BufferModeEnum | DataType | How a newly submitted intent relates to the one already executing. The values and their meanings are those of PLCopen Motion Control MC_BufferMode, adopted unchanged because every motion runtime already implements them. In all blending modes the robot does not decelerate to a stop at the boundary, and the predecessor reaches Succeeded when blending begins rather than when its target is exactly attained. |
| ns=1;i=3902 | EnumStrings | Variable |  |
| ns=1;i=3003 | BlockingModeEnum | DataType | Whether an intent tolerates motion and other intents running alongside it. The four values are the two-by-two matrix of VDA 5050 blockingType, adopted because it is the only widely deployed concurrency annotation for robot actions. |
| ns=1;i=3903 | EnumStrings | Variable |  |
| ns=1;i=3004 | TerminationModeEnum | DataType | Whether a motion ends exactly on its target or is blended into the next one. This is the only distinction every vendor expresses identically; the blend magnitude in BlendDataType.Radius is a request, not a guarantee. |
| ns=1;i=3904 | EnumStrings | Variable |  |
| ns=1;i=3005 | ReleaseModeEnum | DataType | How a held object is given up. |
| ns=1;i=3905 | EnumStrings | Variable |  |
| ns=1;i=3006 | ApproachModeEnum | DataType | Direction from which an end effector approaches an object or a placement. |
| ns=1;i=3906 | EnumStrings | Variable |  |
| ns=1;i=3007 | FrameRoleEnum | DataType | Role of a coordinate frame, following the coordinate systems ISO 9787 standardises. The roles say WHICH frames exist; no standard says how to calibrate between them, which is why CoordinateFrameType carries the transform explicitly. |
| ns=1;i=3907 | EnumStrings | Variable |  |
| ns=1;i=3008 | OperationalModeEnum | DataType | Operational mode of the robot system, as defined by ISO 10218-1 and reported identically by OPC 40010-1. It is READ-ONLY here: mode selection is a safety function performed by safety-rated means, and this specification defines no way to command it. Clause 10 restricts intent submission to Automatic and AutomaticExternal. |
| ns=1;i=3908 | EnumStrings | Variable |  |
| ns=1;i=3009 | IntentFailureEnum | DataType | Why an intent did not succeed. The set is deliberately small and diagnosable: a client decides whether to retry, re-plan or escalate from this value alone, and reads Message only to show a human. |
| ns=1;i=3909 | EnumStrings | Variable |  |
| ns=1;i=3010 | StopModeEnum | DataType | How urgently a cancellation should bring motion to an end. The values are those of PossibleStopModes in OPC 40010-1, so a Server implementing both reports one vocabulary. This is an APPLICATION-LEVEL request: it does not select, imply or guarantee any IEC 60204-1 stop category, which only the robot's safety system determines. See clause 10. |
| ns=1;i=3910 | EnumStrings | Variable |  |
| ns=1;i=3011 | AxisKindEnum | DataType | Whether an axis rotates or translates. This fixes the unit of the corresponding entry of JointMoveIntentDataType.JointTargets. |
| ns=1;i=3911 | EnumStrings | Variable |  |
| ns=1;i=3012 | MissionUpdateResultEnum | DataType | Outcome of a mission update, reported so a client can tell a stale update from a rejected one without parsing a StatusCode. |
| ns=1;i=3912 | EnumStrings | Variable |  |
| ns=1;i=3050 | Pose3DDataType | DataType | A rigid-body pose. Position is metres; Orientation is a UNIT QUATERNION ordered (x, y, z, w). Quaternions are used because OPC UA defines no quaternion type and the Euler triple in ThreeDOrientation is ambiguous without an external convention; Annex C gives the normative conversion to and from ThreeDFrame. All frames are right-handed. FrameId names the CoordinateFrame the pose is expressed in; an empty FrameId means the Server's default work frame. |
| ns=1;i=5001 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3051 | MotionConstraintsDataType | DataType | Limits a motion is to respect. Every field is a REQUEST bounded by what the robot is configured to permit: a Server clamps rather than refuses, except where clause 10 requires refusal. A value of zero or less means the field is unspecified and the Server chooses. |
| ns=1;i=5002 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3052 | BlendDataType | DataType | How a motion ends. Radius is interpreted only when Termination is Blend, and is a request: controllers that expose a unitless blend scale rather than a distance map it as best they can, and a Server that cannot honour the exact radius still succeeds. |
| ns=1;i=5003 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3053 | IntentDataType | DataType | Abstract base of every intent. An intent is a single task-level request; it is what a client submits and what a mission step holds, so the two are the same shape. Extension is by SUBTYPING this structure, which keeps new intents discoverable through IntentCapabilitiesType rather than by probing for BrowseNames. |
| ns=1;i=3054 | MotionIntentDataType | DataType | Abstract base of the intents that move the robot. ToolFrame names the frame whose origin is driven to the target - without it a pose target is meaningless, and OPC 40010-1 defines no tool centre point at all. |
| ns=1;i=3055 | JointMoveIntentDataType | DataType | Move by interpolating in joint space. This is the fastest way between two configurations and the path the tool centre point takes is not controlled. It is the portable equivalent of PTP, MoveJ, J and MOVJ. Giving a pose rather than joint values asks the Server to solve the kinematics itself, which is the 'move to this pose, you choose how' case. |
| ns=1;i=5004 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3056 | LinearMoveIntentDataType | DataType | Move the tool centre point along a straight line to the target. The portable equivalent of LIN, MoveL, L and MOVL. |
| ns=1;i=5005 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3057 | CircularMoveIntentDataType | DataType | Move the tool centre point along the circular arc that passes through ViaPoint and ends at Target. The portable equivalent of CIRC, MoveC, C and MOVC. Only the position of ViaPoint defines the arc; its orientation is ignored. |
| ns=1;i=5006 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3058 | GraspIntentDataType | DataType | Close the end effector on an object. Force and Width are requests; an end effector that cannot regulate force ignores Force and still succeeds. |
| ns=1;i=5007 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3059 | ReleaseIntentDataType | DataType | Give up a held object. |
| ns=1;i=5008 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3060 | PickIntentDataType | DataType | Take an object from a location. Source is a REFERENCE TO A LOCATION NODE, not a name: the location's pose and its properties are then read from the address space, so the station identity has exactly one definition. |
| ns=1;i=5009 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3061 | PlaceIntentDataType | DataType | Put a held object at a location. Destination is a reference to a Location node, for the same reason as PickIntentDataType.Source. |
| ns=1;i=5010 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3062 | ToolChangeIntentDataType | DataType | Exchange the fitted end effector. |
| ns=1;i=5011 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3063 | SetOutputIntentDataType | DataType | Set a discrete or analogue output. Output references an OutputSignal node, so the signal's meaning, range and unit are described once in the address space instead of being implied by a string. |
| ns=1;i=5012 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3064 | CallProgramIntentDataType | DataType | Run a program that already exists on the controller. This is the escape hatch for capability this specification does not model, and the bridge to the OPC 40010-1 task control surface. |
| ns=1;i=5013 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3065 | WaitIntentDataType | DataType | Do nothing for a while, or until released. A mission needs this to express a rendezvous with something the robot does not control; without it a client has to hold the queue open from outside. |
| ns=1;i=5014 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3066 | IntentResultDataType | DataType | The outcome of one intent, preserved after it terminates. AchievedPose records where the tool centre point actually ended, which is what lets a client tell a blended corner from an exact stop and audit a placement. |
| ns=1;i=5015 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3067 | MissionStepDataType | DataType | One step of a mission. Released is what splits a mission into its immutable base and its revisable horizon: a released step has been committed and may already be executing, so an update may not touch it. Status is a HINT - where Operation is not null, that IntentOperation's state machine decides. |
| ns=1;i=5016 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3068 | MissionDataType | DataType | An ordered sequence of intents submitted and tracked as a unit. MissionUpdateId increases with every update, so a Server can reject an update that crossed with another in flight instead of applying it out of order. |
| ns=1;i=5017 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3069 | IntentCapabilityDataType | DataType | What the Server will accept for one intent type. This is the machine-readable declaration that makes an intent surface discoverable: a client reads it once and knows what it may submit, instead of submitting to find out. It is the analogue of the VDA 5050 factsheet. |
| ns=1;i=5018 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=4001 | HasIntentController | ReferenceType | Binds an intent surface to the thing it commands. This is how the model attaches to a robot described by another specification - an OPC 40010-1 MotionDeviceSystem, say - without depending on it. Annex B defines that binding. |
| ns=1;i=4002 | HasFrameParent | ReferenceType | From a CoordinateFrame to the frame its Transform is expressed in. Frames form a tree, so a pose given in one frame can be re-expressed in another by composing the transforms along the path between them. |
| ns=1;i=1001 | RobotIntentRootType | ObjectType | Server-level entry point. A client that has just connected browses here to find every robot it can command, without knowing the Server's layout. |
| ns=1;i=6001 | Controllers | Object | The intent surfaces this Server offers, one per commandable robot. |
| ns=1;i=6002 | SpecificationVersion | Variable | Release of this specification the Server implements, for example '0.1.0'. |
| ns=1;i=1002 | IntentControllerType | ObjectType | The intent surface for one robot: what it can be asked to do, the frames and objects those requests refer to, and the intents and missions currently outstanding. Everything a client needs in order to command the robot hangs from here. |
| ns=1;i=6003 | OperationalMode | Variable | Operational mode reported by the robot. Read-only: mode selection is a safety function and this specification defines no way to command it. Clause 10 restricts submission to Automatic and AutomaticExternal. |
| ns=1;i=6004 | Ready | Variable | True when the robot will accept intents now. A client checks this before submitting rather than inferring readiness from OperationalMode alone. |
| ns=1;i=6005 | ControlOwner | Variable | SessionId of the client that currently holds command authority, or null when no client does. Only that client may submit; see clause 8. |
| ns=1;i=6006 | MaxQueueDepth | Variable | How many intents may be queued behind the executing one. Zero means the Server accepts only Aborting submissions. |
| ns=1;i=6007 | ActiveIntent | Variable | The IntentOperation executing now, or null. |
| ns=1;i=6008 | ActiveMission | Variable | The Mission executing now, or null. |
| ns=1;i=6009 | Capabilities | Object | What this robot will accept. |
| ns=1;i=6010 | Frames | Object | The coordinate frames poses may be expressed in. |
| ns=1;i=6011 | Tools | Object | The end effectors this robot can use. |
| ns=1;i=6012 | Locations | Object | The named places intents refer to. |
| ns=1;i=6013 | Axes | Object | The axes, in the order JointMoveIntentDataType.JointTargets uses. |
| ns=1;i=6014 | Outputs | Object | The signals SetOutput can write. |
| ns=1;i=6015 | Programs | Object | The controller programs CallProgram can run. |
| ns=1;i=6016 | Intents | Object | Outstanding and recently completed intents, one Object each. |
| ns=1;i=6017 | Missions | Object | Outstanding and recently completed missions, one Object each. |
| ns=1;i=6018 | RequestControl | Method | Take command authority. A Server grants it only when no other Session holds it, or when the holder's Session has closed. Holding authority is a precondition for submitting, and exists so that two clients cannot interleave motion; it is NOT the single point of control that ISO 10218-2 requires, which is enforced by safety-rated means outside this interface. |
| ns=1;i=6019 | OutputArguments | Variable |  |
| ns=1;i=6020 | ReleaseControl | Method | Give up command authority. Outstanding intents are unaffected; use CancelAll to stop them. |
| ns=1;i=6021 | SubmitIntent | Method | Submit one intent. Returns as soon as the intent is admitted - NOT when the robot has finished, which may be minutes later. The returned Operation is a node the client subscribes to for progress and reads for the result. This is the whole reason the model is built on the Part 10 program lifecycle: an OPC UA Call cannot stay open for the duration of a motion. |
| ns=1;i=6022 | InputArguments | Variable |  |
| ns=1;i=6023 | OutputArguments | Variable |  |
| ns=1;i=6024 | CancelIntent | Method | Ask the Server to end an intent early. The Server MAY refuse, and says so in Accepted, because some motions cannot be abandoned safely part-way. This is not the OPC UA Cancel Service, which discards a pending response and leaves the robot moving; see clause 6.5. |
| ns=1;i=6025 | InputArguments | Variable |  |
| ns=1;i=6026 | OutputArguments | Variable |  |
| ns=1;i=6027 | CancelAll | Method | Ask the Server to end every outstanding intent and mission. |
| ns=1;i=6028 | InputArguments | Variable |  |
| ns=1;i=6029 | OutputArguments | Variable |  |
| ns=1;i=6030 | Pause | Method | Suspend execution, retaining position so it can be resumed. |
| ns=1;i=6031 | OutputArguments | Variable |  |
| ns=1;i=6032 | Resume | Method | Continue execution suspended by Pause. |
| ns=1;i=6033 | OutputArguments | Variable |  |
| ns=1;i=6034 | Retry | Method | Re-attempt an intent that terminated Retriable. |
| ns=1;i=6035 | InputArguments | Variable |  |
| ns=1;i=6036 | OutputArguments | Variable |  |
| ns=1;i=6037 | SubmitMission | Method | Submit an ordered sequence of intents as one unit. Steps marked Released form the base and are committed; the rest form the horizon and may still be revised by UpdateMission. |
| ns=1;i=6038 | InputArguments | Variable |  |
| ns=1;i=6039 | OutputArguments | Variable |  |
| ns=1;i=6040 | UpdateMission | Method | Replace the horizon of a mission already submitted. The base is untouchable: it has been committed and may already have executed, so an update that would alter a released step is refused rather than partly applied. |
| ns=1;i=6041 | InputArguments | Variable |  |
| ns=1;i=6042 | OutputArguments | Variable |  |
| ns=1;i=6043 | CancelMission | Method | Ask the Server to end a mission and every intent belonging to it. |
| ns=1;i=6044 | InputArguments | Variable |  |
| ns=1;i=6045 | OutputArguments | Variable |  |
| ns=1;i=1003 | IntentOperationType | ObjectType | One submitted intent, tracked to completion. It is a Part 10 program instance, so its lifecycle is the one OPC UA already defines for work that outlives a service call: transitions raise ProgramTransitionEvents, the terminal result survives in FinalResultData, and ProgramDiagnostic2DataType records which Session commanded it without this specification having to model provenance itself. |
| ns=1;i=6046 | IntentId | Variable | Identifier of the intent this instance executes. |
| ns=1;i=6047 | Intent | Variable | The intent as admitted, after the Server applied its defaults. A client reads this to learn what it actually asked for. |
| ns=1;i=6048 | ExecutionState | Variable | Fine-grained state. It refines CurrentState rather than restating it; clause 6.3 fixes which pairs are legal. |
| ns=1;i=6049 | Progress | Variable | Fraction of the intent completed, 0 to 1, where the Server can estimate it. Negative means it cannot. |
| ns=1;i=6050 | CurrentPose | Variable | Where the driven tool centre point is now. Subscribe for tracking; this is a status report at the sampling rate the client asks for, not a control signal, and clause 4.3 explains why it must not be used as one. |
| ns=1;i=6051 | Result | Variable | Outcome, meaningful once ExecutionState is terminal. The same value is placed under FinalResultData so a Part 10 client finds it where Part 10 says it will be. |
| ns=1;i=6052 | MissionId | Variable | The mission this intent belongs to, or empty when it was submitted alone. |
| ns=1;i=6053 | QueuePosition | Variable | Place in the queue while ExecutionState is Queued, 1 being next. Zero once it is no longer queued. |
| ns=1;i=1004 | MissionType | ObjectType | One submitted mission, tracked to completion. It is a Part 10 program instance for the same reasons an IntentOperation is, and it owns the IntentOperations of its steps. |
| ns=1;i=6054 | MissionId | Variable | Identifier of the mission this instance executes. |
| ns=1;i=6055 | MissionUpdateId | Variable | Revision currently in force. |
| ns=1;i=6056 | Mission | Variable | The mission as it now stands, base and horizon together. |
| ns=1;i=6057 | ExecutionState | Variable | Fine-grained state, refining CurrentState as clause 6.3 fixes. |
| ns=1;i=6058 | CurrentStepId | Variable | The step executing now, or empty. |
| ns=1;i=6059 | ReleasedStepCount | Variable | How many steps are in the base. The first this many steps of Mission.Steps are committed; the rest are the horizon. |
| ns=1;i=1005 | IntentCapabilitiesType | ObjectType | What one robot will accept. A client reads this once, before it submits anything, and knows what the robot can do and under what constraints. |
| ns=1;i=6060 | SupportedIntents | Variable | One entry per intent type this Server accepts. |
| ns=1;i=6061 | MissionsSupported | Variable | True when SubmitMission is implemented. |
| ns=1;i=6062 | MissionHorizonSupported | Variable | True when UpdateMission can revise the horizon of a running mission. |
| ns=1;i=6063 | BlendingSupported | Variable | True when the blending buffer modes actually blend. A Server that treats them as Buffered reports False, so a client is not misled about the path. |
| ns=1;i=6064 | MaxBlendRadius | Variable | Largest blend radius the robot will honour, in metres. Zero or less means it is not bounded here. |
| ns=1;i=6065 | AxisCount | Variable | How many axes JointMoveIntentDataType.JointTargets must carry. |
| ns=1;i=1006 | CoordinateFrameType | ObjectType | A named right-handed Cartesian frame. Frames form a tree through HasFrameParent, so a client can compose a chain from a tool frame to a world frame. Roles follow ISO 9787, which standardises which frames exist; the transform between them is carried explicitly because no standard says how to calibrate it. |
| ns=1;i=6066 | FrameId | Variable | Identifier referenced by Pose3DDataType.FrameId. Unique among the frames of one controller. |
| ns=1;i=6067 | Role | Variable | Role of this frame. |
| ns=1;i=6068 | Transform | Variable | Pose of this frame within its parent. Ignored for a root frame. |
| ns=1;i=1007 | ToolType | ObjectType | An end effector. Its tool centre point is a CoordinateFrame of role Tool, which is what a motion intent drives to a target - OPC 40010-1 models robot topology in detail but has no tool centre point at all, so this specification supplies one. |
| ns=1;i=6069 | ToolId | Variable | Identifier unique within the controller. |
| ns=1;i=6070 | Name | Variable | Human-readable name. |
| ns=1;i=6071 | Fitted | Variable | True when this tool is on the robot now. |
| ns=1;i=6072 | TcpFrame | Variable | The CoordinateFrame, of role Tool, that is this tool's centre point. |
| ns=1;i=6073 | Mass | Variable | Mass in kilograms. Zero or less when not stated. |
| ns=1;i=6074 | MaxGraspForce | Variable | Largest grasp force in newtons, or zero when the tool does not grasp. |
| ns=1;i=6075 | MaxOpening | Variable | Largest opening in metres, or zero when the tool does not grasp. |
| ns=1;i=1008 | LocationType | ObjectType | A named place an intent can refer to. Pick and Place reference these nodes rather than naming a station in a string, so a location has one definition that a client can read, subscribe to and reason about. |
| ns=1;i=6076 | LocationId | Variable | Identifier unique within the controller. |
| ns=1;i=6077 | Name | Variable | Human-readable name. |
| ns=1;i=6078 | Pose | Variable | Where the location is. |
| ns=1;i=6079 | Occupied | Variable | True when the Server believes something is there. |
| ns=1;i=6080 | ObjectClass | Variable | What the location holds, when it holds one kind of thing. Empty otherwise. |
| ns=1;i=6081 | Capacity | Variable | How many objects it can hold. Zero means unstated. |
| ns=1;i=1009 | AxisType | ObjectType | One axis of the robot. The order of these nodes under the controller's Axes folder fixes the order of JointMoveIntentDataType.JointTargets, and Kind fixes each entry's unit. |
| ns=1;i=6082 | AxisId | Variable | Identifier unique within the controller. |
| ns=1;i=6083 | Index | Variable | Position in JointTargets, counting from zero. |
| ns=1;i=6084 | Kind | Variable | Whether it rotates or translates, which fixes the unit of its joint target. |
| ns=1;i=6085 | MinPosition | Variable | Lower limit, in radians or metres by Kind. |
| ns=1;i=6086 | MaxPosition | Variable | Upper limit, in radians or metres by Kind. |
| ns=1;i=6087 | MaxSpeed | Variable | Largest speed, in radians or metres per second by Kind. |
| ns=1;i=6088 | Position | Variable | Where the axis is now, in radians or metres by Kind. |
| ns=1;i=1010 | OutputSignalType | ObjectType | A signal SetOutput can write. Modelling it as a node means the range, the unit and the meaning are described once, instead of every client having to know what a line name implies. |
| ns=1;i=6089 | SignalId | Variable | Identifier unique within the controller. |
| ns=1;i=6090 | Name | Variable | Human-readable name. |
| ns=1;i=6091 | Value | Variable | Current value. |
| ns=1;i=6092 | Writable | Variable | True when SetOutput may write it. |
| ns=1;i=6093 | EngineeringUnits | Variable | Unit of an analogue signal. |
| ns=1;i=1011 | ProgramType | ObjectType | A program held on the controller that CallProgram can run. This is the bridge to capability this specification does not model, and to the programs an OPC 40010-1 task control already exposes. |
| ns=1;i=6094 | ProgramId | Variable | Identifier unique within the controller. |
| ns=1;i=6095 | Name | Variable | Human-readable name. |
| ns=1;i=6096 | Description | Variable | What the program does. |
| ns=1;i=6097 | Parameters | Variable | Named parameters it accepts, with their default values. |
| ns=1;i=7001 | RobotIntent | Object | Entry point for robot intent on this Server. A client browses Server/RobotIntent/Controllers to find every robot it can command. |
| ns=1;i=3013 | SafeMotionFunctionEnum | DataType | The safe motion function a safety system is enforcing, as defined by IEC 61800-5-2. This is a REPORT. The safety system enforces these independently of this interface, and a client reading them has not thereby obtained any safety function - see clause 10. |
| ns=1;i=3913 | EnumStrings | Variable |  |
| ns=1;i=3014 | RealTimeTransportEnum | DataType | The transport of a brokered real-time channel. This specification defines none of these: it describes them so a client can find and open one, and the samples never traverse this interface. |
| ns=1;i=3914 | EnumStrings | Variable |  |
| ns=1;i=3015 | ChannelInitiatorEnum | DataType | Which end opens the transport connection of a brokered channel. Getting this wrong is the usual reason a first connection attempt fails, so it is stated rather than left to the reader. |
| ns=1;i=3915 | EnumStrings | Variable |  |
| ns=1;i=3016 | ErrorPolicyEnum | DataType | What a mission does when one of its steps does not succeed. Without this a mission can only abort, which forces every recovery out into the client. |
| ns=1;i=3916 | EnumStrings | Variable |  |
| ns=1;i=3017 | DivergenceKindEnum | DataType | How the transitions leaving one step relate to each other, following the divergence of an IEC 61131-3 sequential function chart. |
| ns=1;i=3917 | EnumStrings | Variable |  |
| ns=1;i=3018 | WeaveShapeEnum | DataType | The oscillation applied across an arc weld seam. |
| ns=1;i=3918 | EnumStrings | Variable |  |
| ns=1;i=3070 | TrajectoryPointDataType | DataType | One point of a time-parameterised path. Positions are per axis in the order the axes are declared, in radians or metres by AxisKind. TimeFromStart is measured from the start of the trajectory, which is what makes the path a trajectory rather than a list of waypoints. Velocities and Accelerations are optional and may be empty. |
| ns=1;i=5019 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3071 | MotionToleranceDataType | DataType | How far execution may deviate before it is a failure. A tolerance of zero or less means the Server applies its own. |
| ns=1;i=5020 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3072 | TrajectoryIntentDataType | DataType | Execute a time-parameterised path. The Server's own motion kernel runs it; this interface hands the whole trajectory over in one submission and does not stream it. That is what makes trajectory execution expressible here when real-time control is not - see clause 4.3. |
| ns=1;i=5021 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3073 | PathWaypointDataType | DataType | One waypoint of a Cartesian path, with the blend that applies at it. Per-waypoint blending is what distinguishes a path from a sequence of separate linear moves: the robot need not stop between them. |
| ns=1;i=5022 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3074 | CartesianPathIntentDataType | DataType | Follow a list of Cartesian waypoints. This is the portable form of a taught path, and unlike a trajectory it carries no timing: the Server paces it from the motion constraints. |
| ns=1;i=5023 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3075 | ForceIntentDataType | DataType | Move until contact. The portable subset of the force-controlled moves every vendor offers: travel along a direction until a contact force is reached or a distance is exhausted, whichever comes first. Reaching the distance without contact is a failure, because the intent was to touch something. |
| ns=1;i=5024 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3076 | ProcessIntentDataType | DataType | Abstract base of the intents that run an application process along a path. Every process needs the same two things beyond its own parameters: a reference to the process program or procedure the equipment holds, and room for the parameters this specification has not standardised. |
| ns=1;i=3077 | ArcWeldIntentDataType | DataType | Lay an arc weld along the path. The parameters are the subset ABB seamdata/welddata/weavedata, FANUC weld schedules and KUKA ArcTech all carry. WeldProcedureRef points at a welding procedure specification (ISO 15609) where the installation works to one; this specification does not restate its content. |
| ns=1;i=5025 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3078 | SpotWeldIntentDataType | DataType | Make a resistance spot weld at the target. WeldSchedule selects the weld controller's own program: current and time are the weld controller's business, and are carried here only where the installation drives them from the robot. |
| ns=1;i=5026 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3079 | DispenseIntentDataType | DataType | Lay a bead of adhesive, sealant or paint along the path. The trigger distances exist because material does not start and stop instantly: a Server begins dispensing before the path and stops before its end. |
| ns=1;i=5027 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3080 | FastenIntentDataType | DataType | Drive a fastener at the target. This intent is deliberately THIN: OPC 40450 and OPC 40451 already define joining and tightening in full, so where such a model is exposed, Joint references the joint in that model and the result belongs there; otherwise Joint is null and the remaining fastening parameters stand alone. Restating those parameters here would create a second definition of the same fact. |
| ns=1;i=5028 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3081 | PalletiseIntentDataType | DataType | Place an item into a pattern. The pattern itself is a Location, so its geometry has one definition that a client can read rather than being recomputed from indices on both sides. |
| ns=1;i=5029 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3082 | SurfaceFinishIntentDataType | DataType | Follow the path pressing into the surface - grinding, polishing, deburring or sanding. ContactForce is what distinguishes it from a plain path: the robot yields normal to the surface to hold that force. |
| ns=1;i=5030 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3083 | MissionTransitionDataType | DataType | One edge of a mission's step graph, following the step-and-transition form of an IEC 61131-3 sequential function chart. Condition is an OPC UA ContentFilter - the base specification's own filter grammar, reused so that this specification does not invent an expression language for implementers to write a parser for. |
| ns=1;i=5031 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=3084 | KinematicJointDataType | DataType | One joint of the kinematic chain: where it sits relative to its predecessor and which way it acts. OPC 40010-1 describes a robot's topology and its axes but defines no kinematic chain, so this is additive rather than a second account of the same thing. |
| ns=1;i=5032 | Default Binary | Object | Default Binary encoding of the structure. |
| ns=1;i=1012 | SafetyStateType | ObjectType | What the robot's safety system is doing, reported so a client can act sensibly around it. Every member is READ-ONLY and every one is a report: the safety system enforces these independently, remains effective when this interface is unreachable, and is not commanded from here. Clause 10 states the boundary and this type does not cross it. |
| ns=1;i=6098 | ActiveFunction | Variable | The safe motion function currently enforced, per IEC 61800-5-2. |
| ns=1;i=6099 | EmergencyStopActive | Variable | True while an emergency stop is asserted. |
| ns=1;i=6100 | ProtectiveStopActive | Variable | True while a protective stop is asserted. |
| ns=1;i=6101 | SafeSpeedLimitActive | Variable | True while a safely limited speed is being enforced. |
| ns=1;i=6102 | SafeSpeedLimit | Variable | The tool centre point speed limit being enforced, in metres per second. Meaningful only while SafeSpeedLimitActive. Clause 10.3 requires a Server to refuse an intent that asks to exceed it. |
| ns=1;i=6103 | SafetyControllerOk | Variable | False when the safety system reports its own fault. A Server accepts no intent while it is false. |
| ns=1;i=6104 | LastStopReason | Variable | Why the last stop occurred, for a human. Never parsed. |
| ns=1;i=1013 | RealTimeChannelType | ObjectType | A high-rate channel this Server can offer, described so a client can open it. The samples never traverse OPC UA: this brokers the endpoint and nothing more, in the same way the Vision model brokers a media endpoint rather than carrying pixels. Clause 4.3 explains why the alternative is not available. |
| ns=1;i=6105 | ChannelId | Variable | Identifier unique within the controller. |
| ns=1;i=6106 | Transport | Variable | The transport this channel speaks. |
| ns=1;i=6107 | EndpointUrl | Variable | Where the channel is reached. Its scheme and form are the transport's, not this specification's. |
| ns=1;i=6108 | Initiator | Variable | Which end opens the connection. |
| ns=1;i=6109 | NominalRate | Variable | The rate the channel runs at, in hertz. |
| ns=1;i=6110 | PayloadDescriptor | Variable | The recipe, signal list or configuration the transport requires, in the transport's own form. |
| ns=1;i=6111 | RequiredMode | Variable | The operational mode the robot must be in before the channel will carry motion. |
| ns=1;i=6112 | Available | Variable | True when the channel can be opened now. |
| ns=1;i=6113 | LeaseHolder | Variable | SessionId of the client currently holding the channel, or null. |
| ns=1;i=6114 | LeaseExpiry | Variable | When the current lease lapses. A lease that is not renewed frees the channel, so a client that dies does not hold it for good. |
| ns=1;i=1014 | RobotDescriptionType | ObjectType | Enough of the robot's construction for a client to plan against it without a second specification: the kinematic chain, the space it can reach, and what it can carry. OPC 40010-1 describes topology and axes and defines no kinematic chain and no tool centre point, so this adds rather than restates - Annex B says which side decides where both are present. |
| ns=1;i=6115 | Manufacturer | Variable | Who made the robot. |
| ns=1;i=6116 | Model | Variable | The robot's model designation. |
| ns=1;i=6117 | KinematicChain | Variable | The joints from the base outwards, in order. |
| ns=1;i=6118 | MountingPose | Variable | Pose of the robot base within the world frame. |
| ns=1;i=6119 | ReachRadius | Variable | Radius of the reachable workspace from the base, in metres. |
| ns=1;i=6120 | PayloadLimit | Variable | Largest payload at the mechanical interface, in kilograms. |
| ns=1;i=6121 | MaxCartesianSpeed | Variable | Largest tool centre point speed the robot will produce, in metres per second. |
| ns=1;i=6122 | MaxCartesianAcceleration | Variable | Largest tool centre point acceleration, in metres per second squared. |
| ns=1;i=6123 | SafetyState | Object | What the safety system is doing. A client reads this before it plans, and subscribes to it so that it learns of a stop rather than inferring one from a refusal. |
| ns=1;i=6124 | Description | Object | The robot's kinematics and limits. |
| ns=1;i=6125 | RealTimeChannels | Object | The high-rate channels this Server can broker. |
| ns=1;i=6126 | OpenRealTimeChannel | Method | Take a lease on a brokered real-time channel. The Server prepares the transport and returns what the client needs in order to connect; it does not carry the samples. A lease that is not renewed lapses, which is what stops a dead client from holding the channel. |
| ns=1;i=6127 | InputArguments | Variable |  |
| ns=1;i=6128 | OutputArguments | Variable |  |
| ns=1;i=6129 | CloseRealTimeChannel | Method | Give up a lease on a brokered channel. |
| ns=1;i=6130 | InputArguments | Variable |  |
| ns=1;i=6131 | OutputArguments | Variable |  |
| ns=1;i=6132 | TrajectorySupported | Variable | True when trajectory and Cartesian path intents are accepted. |
| ns=1;i=6133 | ForceControlSupported | Variable | True when force intents are accepted and the robot can actually regulate force. A Server that would ignore the force reports false rather than accepting an intent it cannot honour. |
| ns=1;i=6134 | RealTimeChannelsSupported | Variable | True when the Server brokers real-time channels. |
| ns=1;i=6135 | MissionBranchingSupported | Variable | True when mission transitions are evaluated. A Server that reports false executes the steps in order and ignores any transitions supplied. |
| ns=1;i=6136 | MaxTrajectoryPoints | Variable | Largest number of points accepted in one trajectory. Zero means the Server states no limit. |
| ns=1;i=6137 | FinalResultData | Object | Part 10 result container. Carries the same IntentResultDataType value as Result, so a Part 10 client finds the outcome where Part 10 says it will be. Mandatory here because clause 6.7 requires it and an Optional member cannot carry a SHALL. |
| ns=1;i=6138 | ProgramDiagnostic | Variable | Part 10 invocation diagnostics: which Session invoked the program, when, with what arguments and to what outcome. Mandatory here because the auditable-commanding property of clause 1.2 is exactly this member, and a capability the specification advertises cannot rest on one a Server may omit. Declared exactly as OPC 10000-10 declares it - a Variable of ProgramDiagnostic2Type reached by HasComponent, with the inherited namespace-0 BrowseName - because a promotion that altered the inherited member's TypeDefinition, reference type or BrowseName namespace would declare a second member rather than promote the inherited one. |
| ns=1;i=6139 | SupportedFacets | Variable | The facets of Table 12.2 this controller claims, by the names given there. A client reads this instead of re-deriving conformance from the address space. Clause 12.2 binds the claim: a facet whose structural requirements are unmet shall not appear, and the behavioural requirements are subject to the honesty rules of clause 9. |

(normative)

OPC 40010 interop profile

OPC 40010-1 describes the robot; this specification commands it. The two are joined by one reference and are otherwise independent — this model takes no dependency on the Robotics NodeSet, and a Server implementing only this specification is fully conformant.

A Server claiming RI-Interop-40010 shall:

- Expose a HasIntentController reference from the MotionDeviceSystemType instance describing the robot to the IntentControllerType instance that commands it. The inverse, IntentControllerOf, leads from the intent surface back to the machine.

- Report IntentControllerType.OperationalMode with the same value as the corresponding OPC 40010-1 operational mode of that motion device system. The two shall not disagree; where they would, the OPC 40010-1 value is the robot's own report and decides.

- Publish, as ProgramType instances under the controller's Programs folder, exactly those programs the OPC 40010-1 task control can load, so that CallProgramIntentDataType and the OPC 40010-1 task control name the same things.

- Express the poses it publishes in frames whose transforms are consistent with the mounting and geometry the OPC 40010-1 model describes.

A Server shall not duplicate OPC 40010-1's topology in this model. AxisType exists here only to fix the order, kind and limits that a joint target needs; where OPC 40010-1 is also implemented, its axis description is the fuller one and this model's AxisType instances shall agree with it. The same rule governs RobotDescriptionType: its KinematicChain is additive — OPC 40010-1 defines no kinematic chain — but where the two describe the same axis, OPC 40010-1 decides and this model reflects it.

Note that OPC 40010-1 defines no tool centre point. ToolType.TcpFrame supplies the concept, and there is nothing in OPC 40010-1 for it to contradict.

A Server claiming RI-Interop-40010 together with RI-Safety shall report SafetyStateType consistently with the OPC 40010-1 safety state of the same motion device system. Where they would disagree, the OPC 40010-1 value is the robot's own report and decides — and neither is safety-rated, as both specifications say of themselves.

(normative)

Pose conversion

A Server that exchanges poses with a model using the core ThreeDFrame — OPC 40010-1, or a spatial-location model — shall convert as follows.

Position. ThreeDCartesianCoordinates X, Y, Z are metres, as is Pose3DDataType.Position. The mapping is the identity.

Orientation. The A, B, C fields of ThreeDOrientation are, per ISO 9787, rotations about the X, Y and Z axes of the reference frame — roll, pitch and yaw — applied as an intrinsic rotation in the order Z, then Y, then X. Angles are in radians.

Given A (roll), B (pitch), C (yaw), let

cr = cos(A/2)   sr = sin(A/2)

cp = cos(B/2)   sp = sin(B/2)

cy = cos(C/2)   sy = sin(C/2)

then the unit quaternion ordered (x, y, z, w) is

x = sr*cp*cy - cr*sp*sy

y = cr*sp*cy + sr*cp*sy

z = cr*cp*sy - sr*sp*cy

w = cr*cp*cy + sr*sp*sy

and the inverse is

A = atan2( 2*(w*x + y*z), 1 - 2*(x*x + y*y) )

B = asin ( clamp( 2*(w*y - z*x), -1, +1 ) )

C = atan2( 2*(w*z + x*y), 1 - 2*(y*y + z*z) )

Three properties of this conversion are normative:

- The clamp in the expression for B is required. Floating-point error can carry the argument of asin outside [−1, +1], and without the clamp a conversion that should yield a pole orientation yields a domain error instead.

- q and −q denote the same orientation. A Server should emit the representative whose w is non-negative, so that two Servers describing one orientation produce the same four numbers.

- The conversion is lossy in one direction only at the poles, where B is ±π/2 and roll and yaw are not separately recoverable. A Server converting a pose that originated as a quaternion shall not round-trip it through ThreeDOrientation when it can avoid doing so.

(informative)

Informative alignments

These are not normative references and impose no dependency. They are recorded because this model borrowed from them deliberately, and an implementer who knows them will recognise what was borrowed.

- PLCopen Motion Control — BufferModeEnum adopts MC_BufferMode unchanged, including the rule that the predecessor completes when blending begins. The Accepted / Queued distinction is PLCopen's Busy without Active.

- VDA 5050 — BlockingModeEnum is the blockingType matrix. The base/horizon mission model, the monotonic update identifier and the rejection of an outdated update are that specification's order model. IntentCapabilitiesType plays the role of its factsheet.

- ROS 2 actions — the goal handle returned on submission, the server's right to refuse a cancel, and the explicit Cancelling state before Cancelled.

- MoveIt — MotionSequenceItem.blend_radius is the same abstraction as BlendDataType.Radius.

- OPC 10031-4 job control — the separation of cancelling work that has not started from aborting work that has.

- Capability–Skill–Service (Plattform Industrie 4.0) — in that vocabulary this model is the service layer over a robot's skills, discovered by capability. IntentCapabilitiesType is the capability declaration.

- Vendor motion languages — URScript, RAPID, KRL, TP, INFORM and AS. The three motion intents are their common denominator, and BlendDataType is the portable form of r, zonedata, $APO, CNT and PL. The process intents are drawn the same way: ArcWeldIntentDataType from ABB seamdata/welddata/weavedata, FANUC weld schedules and KUKA ArcTech; SpotWeldIntentDataType from ABB SpotL and FANUC SpotTool; DispenseIntentDataType from ABB DispL.

- ROS control_msgs/FollowJointTrajectory — the shape of TrajectoryIntentDataType, including the separation of path, goal and goal-time tolerance. Every ROS robot driver already accepts it, which is the point.

- OPC UA FX (OPC 10000-80 to -84) — the open transport a brokered channel may name, and the only one in RealTimeTransportEnum that is an OPC Foundation specification.

- OPC 40450 / OPC 40451 (Industrial Joining Technologies) — where fastening results and joint definitions belong. FastenIntentDataType references them rather than restating them.

- IEC 61131-3 sequential function charts — the step, transition and divergence model of §7.4. Behaviour trees were considered and not adopted: their tick semantics need a runtime that controller vendors do not provide, and their serialization is a library's rather than a standard's.

- ISO 15609 — welding procedure specifications, named by ArcWeldIntentDataType.WeldProcedureRef and not restated here.

- The OPC UA robot skill model developed in the VDMA SOArc working group (http://opcfoundation.org/UA/Skills/) is prior art in this area. This specification uses a different namespace and does not extend it.

(normative)

Vision interop profile

A vision model that publishes a grasp pose and this model that executes it are deployed on the same cell, and each defines its own CoordinateFrameType. Without a rule the flange is described twice, with two FrameId strings and two transforms that can disagree.

This annex imposes no NodeSet dependency in either direction. Both models keep the base OPC UA namespace as their only RequiredModel, and a Server implementing only this one is unaffected.

E.1 This model's frame tree decides. Where a Server implements both for the same robot, the frames here are authoritative. This model owns ToolType.TcpFrame and is what the robot actually moves to; a pose that disagrees with it is wrong however carefully it was measured.

E.2 FrameId corresponds by value. A frame present in both models shall carry the same FrameId string in each. That string, not the NodeId, is what Pose3DDataType names.

E.3 Roles correspond by name, never by number. The two vocabularies agree on World, Base, MechanicalInterface, Tool, Object and Other. A vision model may additionally define a camera role, which this model does not; such a frame shall appear here as Other. A gateway shall map by literal name and shall not cast the integer between the two enumerations, because each is decoded against the DataType of the Variable carrying it.

E.4 Poses transcode explicitly. A vision pose may carry a covariance field this model's Pose3DDataType does not. A boundary shall drop it inbound and shall not fabricate one outbound. Both sides use metres and a unit quaternion ordered (x, y, z, w) in a right-handed frame, so no numeric conversion is required — but 6.7 rule 3 still applies, and an inbound pose whose quaternion is not normalised shall be rejected with ParameterInvalid rather than renormalised.

E.5 An empty FrameId is not passed outward. 6.7 rule 4 reads an empty FrameId as this Server's default work frame. A vision model may forbid an empty value entirely, so a boundary publishing a pose outward shall substitute the named frame explicitly.

E.6 A grasp pose is resolved to the tool centre point. A pose received for execution shall be resolved, through the frame tree, to the Tool frame named by the intent's ToolFrame. A hand-eye calibration resolves to the mechanical interface, and the offset from there to the tool centre point is exactly what it does not measure — so a Server shall not execute a pose that resolves only to MechanicalInterface, and shall refuse it with ParameterInvalid.

_____________

—————————